For ordering see ORDER.EXE in the Stomper-Directory or order online

on our web-page with your Mastercard

Stomper  3.41        10/14/97

Modem and ISDN Sharing-Software for Novell or Netbios Based Networks

DOS/Windows/Netware

� 1994/1997 Pflug Datentechnik.

Hardware and software requirements for installation:

-       network, preferably NetWare or WfW-Network

-	minimum DOS 3.3

-	Windows 3.x  / 95 / NT

-       1 serial interface per modem server, FIFO buffered (UART 16550)

-	Processor depending on number of modem servers on one computer,

minimum AT (80386)

1 INTRODUCTION

2 THE CONCEPT

2.1 WHAT IS A MODEM-/ISDN-SERVER?

2.2 SERVER-CLIENT OPERATION

3 INSTALLATION

3.1 GENERAL (VERY IMPORTANT!)

3.2 INSTALLING A MODEM / ISDN SERVER

3.2.1 Installation under Windows 3.x and Windows 95

3.2.2 Installation on the Netware Fileserver (Modem Server Only)

3.2.3 Installation under DOS

3.2.3.1 Modemserver

3.2.3.2 The ISDN Server

3.3 INSTALLING A  MODEM / ISDN CLIENT

3.3.1 Installation under Windows 3.x and Windows 95

3.3.2 Installation under DOS

3.3.2.1 Modem-Client

3.3.2.2 ISDN-Client

4 SELECT SPECIFIC MODEM SERVERS BY DIFFERENT PORTS

5 THE COOPERATIVE MODE

6 QUEUE MANAGEMENT

7 THE MONITOR

8 PROGRAMS USING INT 14 (BIOS)

9 APPLYING ACCESS RIGHTS

Appendix A Suitable Programs

Appendix B NETINFO.EXE

Appendix C Tips

1. Telemate

2. Terminate

3. Telix For Windows

4. Crosspoint

5. pcAnywhere

6. CFOS

7. Tobit FaxWare

Appendix D Troubleshooting

Appendix E Performance

Appendix F What is a FOSSIL driver?

Appendix G What is a network fossil driver?

Appendix H Support

1 Introduction

Included in this new version 3.30 of Stomper is a complete Windows setup

program. Furthermore, the following improvements vs. version 3.21 have

been incorporated:

- CAPI 1.1 Windows-Server (WIN3.x/95/NT)

- CAPI.DLL 1.1 Client (WIN3.x/95/NT)

- INT14/Fossil support in WIN95-DOS-Box using

STCLNT.VXD. This supports simple FOSSIL programs like Telemate

or Terminate.

In case you already have installed the free TEST/DEMO version of Stomper:

Then you just have to enter the registration key and name to extend your

license (refer to the accompanying diskette for registration key and name).

Under Windows, you register by activating the icon "Registration" in the

Stomper program group.

Registration under DOS is done by entering the command line: register

Registration_Key Name in the Stomper directory (refer to the accompanying

diskette for registration key and name).

If you have not yet installed Stomper on your system, please proceed reading

this manual.

2 The Concept

2.1 What is a modem-/ISDN-server?

The modemserver is a program, that accesses the device directly, that should #

be made available throughout the network.

There may be more than one modem server (or ISDN-server) on one computer or

there may be several servers spread over the whole network.

When a client accesses a modem server, all communication between

communication software (terminal, fax, ...) and the modem is directed through

the network.

Stomper is responsible for repacking the data suitable for the network, to

send it and to convert it back. It appears to the user as if the modem was

attached directly to his local workstation.

Often the modem server is compared to a printer server. This is not quite

correct.

The main difference is, that the printer server is "batch-driven". This means,

that a print job is placed into a temporary file, which is read by the printer

server whenever it is ready and the contents then are sent to the printer.

A modem server works online. All data is sent to the device immediately.

Additionally all line-control information is passed on.

2.2 Server-Client Operation

As soon as a client requests an available modem or an ISDN-adapter (this

happens

automatically, when your communications software is started), the client-

program (NETFOS, NFCAPI, ...) searches for a modemserver and initiates a

session.

From this moment on, the server is not available for other stations. It

is reserved for this one client.

At the end of the session, the server will get released.

3 Installation

3.1 General (very important!)

Note the following key issues prior to installation:

If you want to run Stomper on a Netware 3.11 (or higher) network

You must load the file STMGR.NLM (included on the diskette) on the file

server prior to the installation.

Make sure that the settings for "IPX Sockets" and "SPX Connections" are at

least 30-60 on all the stations. The default settings are too small in any

case.

If you want to run Stomper on a Windows for Workgroups network

In the Windows network setup, set NetBEUI as the standard protocol

(important!).

Make sure that the settings for "Max. connections" is at least 30 and the

setting for "NCBs" is at least 80. The default settings are too small in

any case.

Note: Troubleshooting is greatly simplified if you first install the modem

and ISDN server and the client software thereafter.

3.2 Installing a Modem / ISDN Server

3.2.1 Installation under Windows 3.x and Windows 95

Insert the installation disk into your diskette drive and run SETUP.EXE.

After you have entered the destination path for the installation, you are

asked to choose between modem server and client. Select "Modem-/ISDN Server

Installation" (Note: The setup program allows either to install a server or

a client at one time).

Next, select the desired network protocol.

Only select the IPX/SPX protocol if you are using a Novell Netware network.

After you have finished your selections, the setup program will copy all

necessary files. You can then enter your registration information (if you

own a valid Stomper-license).

This registration information is then incorporated firmly in NETFOS.COM,

NFCAPI.COM, STOMPER.COM, STOMPERF.COM, STISDN.COM and

STOMPER.NLM.

STWIN.EXE, CAPI.DLL and STCLNT.VXD however, will read the registration

information from the [Stomper] section in SYSTEM.INI.

The installation will be completed and finally, a "Stomper" program group

will be created.

3.2.2 Installation on the Netware Fileserver (Modem Server Only)

Copy STMGR.NLM and STOMPER.NLM preferrably to the directory

SYS:SYSTEM.

3 components have to be loaded:

the Stomper queue manager STMGR.NLM

an AIO driver

the modem server STOMPER.NLM

1. The Stomper Queue Manager

The queue manager performs the following tasks:

1. "Tracker", observes the status of all modem servers installed in the

network, containing the following information: Status of the modem server

(busy/free), the Client (Netfos) which is presently served, number of bytes

transmitted in a session.

This information is analysed and displayed for example by STMON, the

monitor program.

This part of STMGR can exist several times in the network.

2. "Queue manager", allocates free modem servers to the clients. In order to

avoid chaos, the queue manager can only exist once in the network.

3. IPX routing. The transmitter must always know the destination network in

order to transport IPX packets over several network segments. STMGR

therefore gives his address to all routers in the network (the file server

can also work as a router and therefore adds this address to its bindary).

Stomper identifies this address by ordering it from the nearest router. When

the queue manager allocates a modem server to a client, the network

address of Stomper is supplied to the client.

4. Logging. If the option /L is specified, a log file will be generated, that logs

all actions.

5. Station restrictions (only STMGR.NLM) See Chapter Applying access

rights.

1. In order to install STMGR.NLM on the file server, enter the following

command at the console

load STMGR

STMGR will display all installed modem servers of the network on the

screen. At this point of the installation you will usually not have installed any

modem servers. However, as soon as you start any modem or ISDN server in

the network it will be displayed on the STMGR screen.

Command line options for STMGR.NLM:

/L...   If you specifiy this parameter, the queue manager will create a log

file. In this file, every access to any modem server in the network

will be logged (time of access, numbers dialed, etc.).

You can specify the name of the log file, e.g.:

/LSYS:SYSTEM\ST.LOG

If you only specify /L STMGR.NLM will create a log file with the

default name STOMPER.LOG in the executable's directory.

/A      Use this parameter to allow "autoattach" (ref. NETFOS /A)

2.      The AIO Driver (Asynchronous I/O)

The standard AIO driver for conventional serial ports is included with

Netware 3.12 and higher. However, up to Netware 4.x Novell is distributing

outdated versions of this driver. Therefore, you must install a newer

version of this driver. You can download the updated files e.g. from our

support BBS.

We suggest that you use at least AIOCOMX Version 2.11 and the

corresponding AIO.NLM version 6.01a.

There are also other AIO drivers for various multiport extension boards

available (e.g. Digiboard), that are usually distributed by the board's

manufacturer.

Here are typical command lines for various COM ports:

COM1

load aiocomx port=3f8 int=4 maxrate=115200

COM2

load aiocomx port=2f8 int=3 maxrate=115200

COM3

load aiocomx port=3e8 int=4 maxrate=115200

COM4

load aiocomx port=2e8 int=4 maxrate=115200

If you want to install more than one modem server, accordingly load

AIOCOMX.NLM multiple times.

3. The Modem Server STOMPER.NLM:

On the first installation, load STOMPER.NLM without command line

parameters:

load stomper

Stomper will thus display a list of all the installed AIO serial ports.

You can then select the desired serial port. Subsequently, the modem server

is started and shows the command line that you have to enter for future

starts of STOMPER.NLM (i.e. the command line that should be included in your

AUTOEXEC.NCF), like:

Command Line:

load stomper /p1,0,0

Additional command line options for STOMPER.NLM:

/Na	This is used to give the modem server an individual identification.

'a' is replaced by an arbitrary digit or character. If you want to

install more than one modem server, each one has to have a unique

identification, e.g. /N1, /N2, /Ni, ...

/R	Forces STOMPER.NLM to always use the hardware handshake

(RTS/CTS). In particular, set this option when you intend using a

fax software.

/Lxxxxx	"Lock Baud". Using this option, you can force STOMPER.NLM to

use a fixed bps rate (e.g. /L38400). For this to work, your

application must access the modem via NETFOS and INT14 or

FOSSIL.

Windows applications will not be affected by this option.

You are able to install multiple modem servers (provided you purchased a

license for multiple modem servers) by loading STOMPER.NLM more than

once. (Please note that you have to use the /N command line option if you are

installing multiple modem servers).

With

unload stomper

ALL modem servers loaded on the file server will be removed.

Caution !!

Netware 3.x - 4.x have a bug in the SPX-Protokoll. This bug known to

STOMPER.NLM and Stomper tries hard to avoid problems that might arise.

But we do strongly advise you to install the following Novell patches:

PATCHMAN.NLM

SPXDDFIX.NLM

SPXFIX2.NLM

These three modules are contained in the archive STRTL3.EXE, that can be

downloaded free from Compuserve NOVLIB or from our support BBS.

If you have problems loading AIO.NLM, STOMPER.NLM and

STMGR.NLM, and you get a message like

"Public symbol IsColorMonitor not found",

then you have an old version of CLIB.NLM (Netware 3.11). Just install an

updated version, that is part of the archive LIBUP4.EXE (Compuserve

NOVLIB or our support BBS)

Never ever enter IRQ 2 for a hardware driver (e.g. when loading

AIOCOMX). Netware uses this IRQ internally and "misusing" it will

generally destroy the file allocation table of the server's drives and you will

completely loose all data.

3.2.3 Installation under DOS

If you intend to run Stomper in a Novell Netware 3.11 or higher network, we

strongly recommend to install STMGR.NLM on the fileserver rather than

installing the queue manager on a DOS station.

Besides, STMGR.COM has the same command line options as STMGR.NLM.

3.2.3.1 Modemserver

(Most simple example:   STOMPER /P2)

The modem server accesses the modem (or another device), that is connected to

the RS-232 port. Apart from the application data, the line condition will be

updated whenever necessary. This condition includes DCD (Data Carrier Detect),

DTR (Data Terminal Ready), RI (Ring Indicator), DSR (Data Set Ready), RTS

(Ready To Send) and CTS (Clear To Send).

For DOS there are 2 variants of the modemserver:

STOMPER.COM	accesses the serial port directly

STOMPERF.COM	uses a FOSSIL driver to

communicate with the hardware

Using STOMPERF.COM it is possible to share any communications hardware in

the network, as long as there is an apropriate FOSSIL driver available (like

CFOS for ISDN or Digiboard).

Command line parameters

STOMPER [/Px] [/Na] [/Lxxxx] [/U] [/F] [/Qxx] [/Bxxxx] [/Caaa...] [/R]

/P	selects the COM port for the modem, for example /P2 for COM2. If

omitted, COM1 is selected automatically.

/U	is given to remove any already installed STOMPER from memory.

This is only possible while Stomper is not in use.

/F	STOMPER is forced to use Netbios exclusively (even if IPX is

loaded)

/Q 	for unconventional IRQs. /Q7 => IRQ7

/B1	for unconventional ports. /B3E8  => Port 3E8

/C	Cooperative mode: refer to chapter "Cooperative Mode"

/L1	Lock Baud. Independent of the bps rate chosen by the application

(terminal program,...), Stomper always selects automatically the bps

rate of this parameter, for example /L57600 (locked).

Note: Windows applications ignore this parameter!

/N	one single character is added to this option (for example one character

or one digit), giving the modem server an individual identification.

Each modem server has to be identified individually (several modem

servers on one computer each have a unique character or digit).

/R	forces RTS/CTS data flow control. This is very helpful, e.g. if you

encounter problems when sending with Winfax. Normally this option

does not affect any other applications, therefore you may always set it.

/Z 	16550 FIFO buffer control

/Z0	FIFO off

/Z1	FIFO on 1 byte threshold

/Z4	FIFO on 4 byte threshold

/Z8	FIFO on 8 byte threshold (default)

/Z14	FIFO on 14 byte threshold

It is not recommended to use this parameter! Stomper normally selects an

8 byte threshold for the FIFO buffer as the most sensible value to avoid

errors.

However, on very slow PCs (286 - 6 MHz), it may be necessary to use /Z1.

3.2.3.2 The ISDN Server

The ISDN server (STISDN.COM) makes a CAPI 1.1 (not 2.0) accessible to the

clients on the network.

One ISDN server can only serve one client at a time. To let more than one

client access the CAPI at the same time, multiple ISDN servers have to be

loaded. These can use the same CAPI, depending on how many simultaneous

applications this particular CAPI can support (usually 2 to 4).

(Most simple example:  STISDN   - without any parameters)

Please note that command line parameter /M is usually necessary!

Command line parameters

STISDN [/Na] [/U] [/F] [/M...] [/Kn]

/U	is used to remove any already installed STISDN from memory. This is

only possible while STISDN is not in use.

/F	STISDN is forced to use Netbios exclusively (even if IPX is loaded)

/N	one single character is added to this option (for example one character

or one digit), giving the modem server an individual identification.

Each ISDN server has to be identified uniquely (several ISDN servers

on one computer each have a unique character or digit).

/Kn	If you have multiple ISDN-adapters installed, you can tell STISDN to

use the specified adapter (/K0 for adapter number 1). If you don't

specifiy this parameter, the application will determine which adapter is

used.

/M...	defines, how much memory is reserved for communications. The

default is 16384 Bytes. Values may range from 4000 to 65500 Bytes.

The necessary value strongly depends on the application:

Example:

On the client station, the CAPI should be made accessible for CFOS:

CFOS i

The necessary memory can then be calculated from:

- B2 frame length	2048 Bytes

- Windowsize	2

- Max. Messages	20

2048 Bytes * 2 + 20*180 = 7696 Bytes

CFOS i -w3 -a0

- B2 frame length	2048 Bytes

- Windowsize	3

- Max. Messages	20

- Max B-Channels	2

2048 Bytes * 3 * 2 + 20 * 180 = 15888 Bytes

The optimum really depends on the software you will be using and the

number of concurrent ISDN connections. The default value usually is

enough for one application. If you want to use two applications at the

same time, you should increase the value accordingly. ISDN programs

like Eurofile transfer or ISDN fax will usually need a setting of

/M65000.

3.3 Installing a  Modem / ISDN Client

3.3.1 Installation under Windows 3.x and Windows 95

Note that the CAPI 2.0 client is only available under Windows 95

Insert the Installation disk into your diskette drive and run SETUP.EXE.

After you have entered the destination path for the installation, you are

asked to choose between modem server and client. Select "Client

Installation" (Note: The setup program allows either to install a server

or a client at one time).

Next, select the desired network protocol.

Only select the IPX/SPX protocol if you are using a Novell Netware network.

After you have finished your selections, the setup program will copy all

necessary files. You can then enter your registration information, in case

you own a valid Stomper license.

This registration information (key) is then incorporated firmly in NETFOS.COM,

NFCAPI.COM, STOMPER.COM, STOMPERF.COM, STISDN.COM and

STOMPER.NLM.

STWIN.EXE, CAPI.DLL and STCLNT.VXD however, will read the registration

information from the [Stomper] section in SYSTEM.INI.

The installation will be completed and finally, a "Stomper" program

group will be created.

3.3.2 Installation under DOS

3.3.2.1 Modem Client

The modem client provides access to a modem server. All applications that

use a FOSSIL driver or BIOS-INT14 (option /I) for communication can use

it to access a network modem.

Command line parameters

NETFOS [/Px] [/U] [/I] [/F] [/Naaa...] [/A|/A+]

/P 	specifies which COM port will be redirected. E.g. /P2 for COM2.

COM1 is chosen automatically when this parameter is not used.

/U	removes an installed NETFOS from memory.

/I	This parameter is mandatory to redirect programs which communicate

using the BIOS-INT-14. With this setting, Netfos finds out if an

application is able to serve a fossil interface, and reacts accordingly.

Without this setting, Netfos can only serve Windows programs or

programs which support a fossil driver.

/F	Netfos uses Netbios (even if IPX is loaded)

/N1	All modem servers wanted in the network are listed here. When for

example. /N123 is given, the queuemanager selects the first available

modem server from the list. The priority is from left to right, so

that the queuemanager selects STOMPER /N2 when STOMPER /N1 is

occupied.

This option is invalid (and meaningless) in the 1-server version.

/A	"Autoattach"-mode. When a modemserver is assigned to a user after he

was placed in the queue (waiting for a modemserver to become

available), this option suppresses the message "Connected to

modemserver". So the connection will be established without

interaction. For both /A and /A+ STMGR must have been loaded with

the option /A as well!

/A+	Same as /A, but additionally the user will be placed into the waiting

queue automatically, in case there is no modemserver available. This

mode runs completely unattended.

Note: /A and /A+ are only supported in the commercial version.

Note that the parameter /Px does NOT have to be identical for the two

drivers. For STOMPER /Px defines the interface which is physically connected

to the modem. Whereas NETFOS /Px defines only the interface which is

addressed by the terminal program.

3.3.2.2 ISDN Client (CAPI 1.1 only)

The ISDN lient (NFCAPI) provides applications with a virtual CAPI v1.1-

interface. Up to 4 applications can be serviced by the same NFCAPI.

Command line parameters

NFCAPI [/U] [/F] [/Naaa...] [/A[+]]

/U	removes an installed NFCAPI from memory.

/F	NFCAPI is forced to use Netbios (even if IPX is loaded)

/N	All ISDN servers wanted in the network are listed here. E.g. if /N123 is

given, the queue manager selects the first available ISDN server from

the list. The priority is from left to right, so the queue manager selects

STISDN /N2 when STISDN /N1 is occupied.

This option is invalid (and meaningless) in the 1-server version.

/A	"Autoattach"-mode. When a ISDN server is assigned to a user after he

was placed in the queue (waiting for a ISDN server to become

available), this option suppresses the message "Connected to ISDN

server". So the connection will be established without interaction. For

both /A and /A+ STMGR must have been loaded with the option /A as

well!

/A+	Same as /A, but additionally the user will be placed into the waiting

queue automatically, in case there is no ISDN server available. This

mode runs completely unattended.

Note: /A and /A+ are only supported in the commercial version.

If your ISDN adapter comes with the Windows CAPI driver CAPI.DLL, it can

run on top of NFCAPI to enable ISDN applications to access the ISDN server.

However, a prerequisite is that CAPI.DLL works with any DOS CAPI drivers

(e.g. Teles).

4 Select specific modem servers by different ports

Example:

You have 5 modem / ISDN servers installed in your network:

three modems with IDs '1', '2' and '3'

two ISDN-lines via CFOS with IDs '4' and '5'

Of course, you will want to use an ISDN-line, when you need an ISDN-line

and a modem line, when you need a modem line. You can do this by simply

entering the following command line:

NETFOS /P1(123) /P2(45)

When your Application addresses COM1, you will get connected to modemserver 1,

2 or 3, whichever is available. When your application uses COM2, one of the

ISDN lines will be selected.

Note that there is no option /N specified, since this is completely replaced

by the values in brackets.

You can specify up to 4 ports.

Only one of these ports may be used at one time.

Of course you can use this method to distinguish different modem types as

well - ISDN was chosen as a typical example of different transport media.

5 The Cooperative Mode

This mode - under certain circumstances - enables the use of another

software (e.g. a mailbox) on the modem server - sharing the port

cooperatively with Stomper. It should be used with caution, as it is no

"well behaving" method.

Regarding the ISDN server, this is not a problem, since CAPI supports

cooperative applications anyway.

Currently, the cooperative mode is not ready for Windows.

Parameter: /Cxxxxx

xxxxx stands for any modem command. At the end of a session this command is

sent to the modem in order to re-initialize it. CR/LF is added to this

string (for example ATZ), automatically.

Thus the modem server can run a mailbox, which will be stopped automatically

when a client in the network demands the modem.

While a caller is logged into the mailbox, Stomper is inactive.

In order to avoid fatal failures on the computer, please read the following

lines even if this sounds a bit technical:

For safety reasons a network-modem demand is rejected, when the CD signal is

active, for example, when someone is logged into the mailbox. Stomper will

also be blocked for another 60 seconds after the last ring.

This mode has to be handled with care, as it is quite rude to block an

application just like that, but this can be done for example with FrontDoor

and Remote Access.

As during the blocking of the modem the bps- rate could be changed, at the

end of such a session Stomper sends an optional command (see above, "/Caaa")

to the modem in order to set the correct bps rate. For this reason, the modem

must have an automatic baudrate identification.

In the cooperative mode Stomper MUST NOT BE LOADED in the high memory

area (UMB), due to the following:

The computer will be stopped at an appropriate time, that is in the moment

of a timer interrupt (produced by the computer 18 times per second).

"Appropriate" means:

1.	there is no hard disk or disk access

2.	Windows is inactive

3.	The program interrupted is placed in the memory above stomper and

under the high memory area from A000:0000 on.

No. 3 ensures that no IPX routine has been interrupted.

As Stomper depends on IPX, it would not dare to touch this without being

affected personally. As Stomper needs the IPX (or Netbios) drivers, they

are normally placed in a lower memory section or in an UMB. But if Stomper

has also been loaded in the high memory area, there is no certainty where

the network drivers are placed related to Stomper.

It is possible that the interrupt does not appear. This happens for example.

when the command line in DOS is used, which is mainly executed in

COMMAND.COM (below Stomper) or in BIOS (>A000). The use of the interface

is not affected in this case.

6 Queue Management

In order to enable a precise access to one or several modem servers in the

network, the program

STMGR.COM

has to be loaded on one computer (any) in the network. STMGR.COM should

already be active in the network, before a Client (Netfos) or modem server

(Stomper) is loaded. Preferably this program should be installed on a modem

server.

If you are using Netware 3.11 or higher, it is strongly advised to run

STMGR.NLM on the file server rather than using STMGR.COM on a DOS station.

This program operates completely in the background and does not require much

memory.

The queue manager's strategies

When a client demands a modem server, the queue manager tries to allocate a

free modem server which is selected by NETFOS /Naaa...(from left to right).

In case all modem servers are busy, the client computer receives the

following message:

No modemserver available. Queue in?	   Y/N

After confirming Y, the following message appears:

Queued in. Wait in foreground?         Y/N

Now there are two possibilities:

1.	"Wait in the background" (N) tells the application that no fossil

driver is installed (not recommended for pure BIOS INT14

applications). The computer can be used and as soon as a modem

server is free, the following message appears:

Connected to modem server			(ENTER)

The queue manager marks the modem server as "reserved". The user has

1.5 minutes to start his application and to use the modem server.

2.	While waiting in the foreground, the following message appears

Waiting for modem server...			(ESC)

In the meantime the computer cannot be used for other applications.

Under windows this process is executed accordingly, but only the application

in use is paused. Other applications may be used - as far as multitasking is

possible.

7 The Monitor

The monitor (STMON.EXE) provides an overview over the modem servers installed

in the network and the proceedings can be observed. The monitor is started by

loading

STMON

As STMON displays the information obtained from the tracker, it depends on

STMGR.COM being loaded. If STMGR is not already in memory, the monitor will

load it automatically. For this, however, STMGR.COM must be either

-	in the same directory as STMON.EXE or

-	in the DOS path.

Attention: The monitor must not be started in a DOS box under windows!

The monitor displays ten sections on the screen, each containing information

of an individual modem server.

Field identifications:

Network address - individual address in the network which is preset by the

network adaptor. In order to obtain a better overview, a file called

NODES.LST

can be created, in which names are allocated to the network addresses. An

example for such a file:

924e32a 	John's abacus

80ca23e5 	venus

... allocating the names "John's abacus" and "venus" to the according

network addresses. John's abacus and venus will replace the cryptic address

names on the monitor.

Served interface - the serial interface served by the modem server or CAPI

in case of an ISDN adapter.

Status - status of the modem server. Possible states:

- free           the modem server is available at the moment

- Ser. <Client>  the modem server is just serving the client named

<Client>

- Res. <Client>  the modem server is presently reserved for the client

<Client>.

- locally in use The modem server is presently used locally in the /"C"

mode (for example for a user in the mailbox)

Sent / received bytes - number of bytes transmitted in the current session.

This information will be held until the start of a new session.

Identification - the identification of the modem server (/Na).

connected with - The number dialed will be displayed here as soon as a

Carrier Detect (CD) has been encountered.

8 Programs Using INT 14 (BIOS)

The fossil driver is to be understood as an extension to the BIOS-INT 14

interrupt.

However, Stomper expects that the software used initializes the fossil

driver (not necessary with BIOS routines).

For programs only using BIOS-INT 14, NETFOS has to be loaded with the

parameter /I.

With pure INT 14 applications, Stomper always switches to RTS/CTS data flow

control.

Approximately 20 seconds after the end of an "INT-14 program" the modem

server becomes available for others again.

When using fax software, which communicates via INT 14, it is absolutely

necessary to lock Stomper on an appropriate rate. The ideal bps rate

depends on the modem used.

Standard modems transmit faxes with a rate of up to 14400 bps, or

19200 bps on the phone line. In this case the bps rate has to be locked

on 19200 bps. Most modems accept a higher rate, for example 57600 bps, when

transmitting faxes.

For modems with a maximum baud rate of 9600 bps, Stomper should be locked on

9600 bps (see parameter /L on page 6).

9 Applying access rights

STMGR.NLM provides a mechanism to refuse access to all or specific

modemservers by username, stationaddress and/or network.

This mechanism is activated, as soon as STMGR.NLM finds a file named

QMREST.LST in the same directory, STMGR.NLM is contained in (the activation

of the access rights happens during startup of STMGR). If this file is not

present, no restrictions apply.

The contents of the file QMREST.LST could look like this:

[stations]

928485

80A23E42       12Q

[nets]

1000002

1000003        1

[names]

supervisor

Peter       24

This file is shown on the next page with comments included.

Rules:

there are 3 possible sections: [stations], [nets] and [names]

you can omit particular sections. There will be no restrictions for these

sections. If a section is present, it defines objects (stationaddresses,

usernames or networkaddresses), that have access to modemservers. All

objects of this sectiontype, that are not defined, will not have access to

any modemserver.

if only the object is entered in a line, it receives access rights to all

modemservers throughout the network. If you additionally enter a list of

modemserver-ids -separated by spaces or tabs from the objects designation

- the object will only have access to the specified modemservers.

access rights are combined by logical and, which means, the user must

qualify by station, network and username (in case restrictions apply

respectively)

Typically the file QMREST.LST will only contain a [names] or [nets] section.

Example for QMREST.LST with comments

; Lines beginning with a semicolon are treated as comments and are

; ignored

; There are 3 sections in this file:

; [stations]

; [nets]

; [names]

;

; Sections can be omitted. If there is no section named [stations],

; then there will be no station restrictions.

; The section [stations] lists all stations, that have access to

; modemservers

[stations]

; Station 928485 has access to all modemservers:

928485

; Station 80A23E42 has access to modemservers 1, 2 and Q:

80A23E42       12Q

; All other stations have no access rights to any modemservers!

[nets]

; All stations in net 1000002 may use all modemservers, if they are

; not restricted by username or network number

1000002

; The stations in network number 1000003 only have access to

; modemserver 1

1000003        1

; All other networks have no access to any modemserver!

[names]

; Supervisor has access to all modemservers, as long as no

; station/network restrictions apply:

supervisor

; Peter may only use modemserver 2 and 4

Peter       24

; If there is no section [names], you can even use modemservers

; without logging in.

; Note! Even though the supervisor is generally given access to all

; modemservers, station- and network-restrictions still are active!

; Example: Peter is logged in on station 80A23E42 in Network number

; 1000002. Therefore he can only use the modemserver with ID '2

Appendix A Suitable Programs

All windows programs, using the functions (API) provided by windows

COMM.DRV for the interface addressing, are suitable.

DOS:

Not every terminal program supports a fossil driver. An example for a program

with FOSSIL support is Telemate (In Options / Terminal, FOSSIL can be

selected)

For the use of Telemate note the following: in order to work with the dial

list, for each listing the fossil driver has to be selected individually.

To select the fossil support in Telemate, first the baud rate has to be set

to a maximum of 38400bps. Otherwise fossil will be ignored!

BinkleyTerm, FrontDoor (mailer systems) and RemoteAccess (a mailbox system)

have no problems working with Stomper.

Appendix B NETINFO.EXE

This a small diagnostic tool added to the packet, giving some information of the

network environment (received from netbios), for example

Permanent name:                 0000C0039A79  ; network address

Jumper:                         40

POST result:                    00

Version:                        0.1

Minutes up:                     0

Number of CRC errors:           0

Number of alignment errors:     0

Number of collisions:           0

Number of aborts:               1

Number of transmitted packets:	6

Number of received packets:     10

Number of retransmits:          4

Number of out-of-buffer-counts:	0

Free NCBs:                      10

Max. NCBs:                      12

Active sessions:                32

Max. packet size:               1024

Names:                          1

NFOSMPX1                        0

Attention! This tool can only be useful when Netbios is actually installed.

Otherwise no useful information will be displayed here! This information

is of no use for IPX.

Appendix C Tips

1. Telemate

Before you select FOSSIL, be careful to set the speed to not more than

38400 bps.

Otherwise Telemate will reject the FOSSIL-Setting (without displaying a

message).

In case even this fails, the safest way is to edit TM.CFG manually. Just

change

Connection=Modem

to

Connection=Fossil

and make sure that the entry Port= is correct.

Also, FOSSIL has to be selected for every entry in the Dial-List.

2. Terminate

Early versions of Terminate (<3.x) had a problem identifying a FOSSIL driver

that does not support all serial ports at once, like does NETFOS. If you

have Terminate 3.x or higher, you don't have to read on.

The first time Terminate is set up for FOSSIL, it expects that a FOSSIL

driver is installed on COM1. This, of course, is not always given (e.g.

NETFOS installed on COM2). It may result in a system-crash.

Workaround:

either use NETFOS on COM1 or

the first time, you switch Terminate to FOSSIL, do it with a regular FOSSIL

driver (like BNU or X00) installed. From then on you can use NETFOS.

If you don't have a FOSSIL driver handy, load

NETFOS /P1() /P2()

This installs NETFOS on both COM1 and COM2.

3. Telix For Windows

Use the COM-Setting, not FOSSIL! In Windows it is always recommended to use

STCOMM whenever possible.

Apart from that, the FOSSIL support of early versions of TfW is buggy.

4. Crosspoint

In case Crosspoint does not work properly with Stomper, you might try loading

NETFOS with the /I option. Crosspoint's FOSSIL support is not very clean in

some places.

5. pcAnywhere

Use the plain INT14-Setting and do not set higher speeds than 9600 bps.

Otherwise pcAnywhere will not be able to initialize the port.

To achieve higher speeds, refer to the modemserver switch /L.

6. CFOS

These tips refer to CFOS/DOS.

CFOS/Win should be treated similarily.

There are two ways to install an ISDN server using CFOS:

1. CFOS installed on the modem server.

The drivers need to be loaded in this sequence

CAPI

CFOS

STOMPERF

Thus, clients can only access ISDN via the FOSSIL interface. Applications

that access the CAPI must have STISDN / NFCAPI.

2. CFOS installed on the client stations.

On the modem server, you load

CAPI

STISDN [...]

And on the clients, you load

NFCAPI [...]

CFOS [...]

The main drawback of this second alternative is that the ISDN server is

accessed and locked by the client station as soon as CFOS is loaded.

Whereas in the first alternative, the ISDN server is accessed only if there

really is an application demanding access via NETFOS.

We suggest to load CFOS with the following options:

/w3 /jr

7. Tobit FaxWare

STOMPER.NLM can share the same port with Tobit FaxWare. However,

FaxWare may then not be set to automatic fax reception since the port is

continuously in use by FaxWare in this case.

Appendix D Troubleshooting

This chapter has been moved to a small program named HELP!.EXE. Using this

little DOS program, you can get help for specific problems.

Just start the program by entering HELP! in the Stomper directory on the DOS

command line or in a DOS box of Windows.

Appendix E Performance

The modem server may transmit up to 17,000 Bytes per second. Anyways, you

may only get to this limit if you are using CFOS / STOMPERF with channel

bundling.

The ISDN-Server allows up to 34,000 Bytes per second, corresponding to

about 4 bundled channels.

Appendix F What is a FOSSIL driver?

There are various possibilities for a DOS program to address serial

interfaces of a PC:

There is the hardware related method, communicating directly with the

interface components. This method is used very often because it�s quick

and uncomplicated for the program user - no unnecessary thoughts about

a fossil driver and its implementation.

"Polite" programs use the 4 functions provided by the computer BIOS (INT 14):

Baud rate setting, character transmission, character receive and fetching the

interface status. This method has the advantage of being hardware independent

(only BIOS has to have the information how to communicate with the hardware -

the programmer isn�t troubled with that). Big and decisive disadvantage:

under DOS, BIOS allows max. 9600 bps, which is out of date today. Furthermore

BIOS routines are generally quite slow.

Another possibility is the fossil driver, combining the advantages of the

solutions mentioned above - speed and hardware independence - troubling the

user with only one installation.

The fossil driver offers buffered interface communication as an additional

function to the BIOS, emulating the interface functions of BIOS. Data arriving

at the interface needn�t be handled immediately by the software, but are

stored in a ring buffer. A program using BIOS for example, has to handle

every single character before the next arrives in order to avoid a loss of

data. With a fossil driver the data can be fetched and handled in blocks.

Appendix G What is a network fossil driver?

It�s easy to exploit the advantage of a hardware independent program with

the use of a fossil driver. A suitable driver can pretend any digital

communication element against such a hardware independent program, for

example as COM2. The programm doesn�t care how this works, as long as the

driver handles the mass of data.

Stomper uses this circumstance. A modem server is installed on a network

integrated computer linked to a modem. The modem server waits for a network

fossil driver connection, which again bluffs to be communicating with a

serial interface. At the same time a lot of instruction data and other data

are passed over the network, but don�t worry, a normal network is hardly

concerned with that.

Appendix H Support

Pflug Datentechnik may be reached via:

Fax     +49 7157-216-22

e-mail  support@pflugdt.lb.eunet.de

Compuserve    Pflug Datentechnik 100340,1047

Fido	2:246/1009		(Only Crashmail!, please)

Mailbox (BBS)   +49 7157-216-23

Current Stomper versions can be downloaded from our BBS or from the WEB:

http://pweb.eunet.de/pflugdt.lb