BOOL 2:  LOGIC DESIGN SOFTWARE - TECHNICAL DESCRIPTION

1  OVERVIEW

BOOL is a general tool for synthesizing two level logic from

a behavioral description, and is mainly aimed at ASIC and

PLD synthesis.  It contains an HDL compiler, a two level

minimizer, a high level event driven simulator, and output

formatting routines.  The HDL compiler can read a behavioral

description of a digital circuit, written in a language

similar to the C programming language, minimize it, simulate

it, and print it in various formats.  The minimizer,

simulator, and the I/O capability are all controlled by

special statements or function calls in the description

source file.   The C-like language is powerful, yet simple

and easy to learn.

A unique feature of BOOL, built into the compiler, is its

ability to extract implicit don't care output conditions

from the behavioral description, according to the principle:

what you don't define is don't care.   The don't cares are

extracted, for example, from syntax constructs such as

unspecified states in state machines, assignments done under

if conditions etc..  Other sources of don't cares come from

implicit behavior definition of circuit elements such as JK

flip-flops.   The don't cares are used by the minimizer to

further reduce the size of the hardware.   Typical reduction

in logic size, due only to the inclusion of don't cares can

be as much as 30% in the number of product terms, and 50% in

the total number of literals.

The software will propagate don't cares through re-

assignments of temporary values, complex operators (e.g.

arithmetic), loops, function calls, etc.

2  LANGUAGE SPECIFICATION

2.1  TYPES OF VARIABLES

Types of variables used to describe circuit elements

include: input, output, node and flip-flop.

Page 1

boolean variables can be used to hold intermediate values

that relate to circuit behavior.

All the above types can be used as scalars, or in the form

of groups .  groups (or buses) can be declared either as

arrays, or specified as a collection of other variables.

Also supported are integers, integer arrays and strings.

These can be used for tasks such as controlling program

flow, calculating constant values, addressing individual

elements in groups (buses), or generating special output

formats.

Attributes can be attached to each signal when it is

declared, and used later to control the synthesis process.

For example, attributes can specify the desired minimization

strategy for a specific variable, a type definition of a

flip-flop (e.g. JK, RS, D), or even a library name of a

flip-flop that should be mapped to a specific cell of a gate

array.

2.2  EXPRESSIONS AND OPERATORS

Expressions may include a mixture of types of variables,

arithmetic (+ - / *), boolean (! ~ & | ^), shift (>> <<) and

comparison ( == != > < >= <=) operators, parentheses and

function calls.   When used in expressions, groups are

treated as integer numbers where each element (or signal) is

treated as a single bit.  Precedence of operators is the

same as in C.   A single element of a group can be accessed

by indexing the group with an integer expression.

2.3  CONTROL FLOW STATEMENTS

The syntax supports a behavioral model by using constructs

such as conditional execution under if - else, function

calls and for and while loops.  break and continue are

supported inside loops.  A special syntactic construct

similar to the switch statement in C is provided for

definitions of state machines and truth tables.  State

transition are defined with jump and stay statements.

2.4  STRUCTURED CODE

Code may be nested using braces {...} with no practical

limit on nesting level.

The language supports C-like functions which may accept any

type of parameters, and return a value with type of either

boolean, group, integer or string.

Page 2

2.5  PREPROCESSOR

A preprocessor pass supports the directives:  #include

#define #undef #ifdef #ifndef #else #endif.

3  THE MINIMIZER FACILITY

Several state of the art two level minimization algorithms

are accessible through function calls.  A tradeoff between

time and memory spent on minimization versus results quality

may be specified by a minimization level number which range

from 1 for the simplest and fastest minimization up to 63

which guarantees an absolute minimum.

The absolute minimizer guarantees a solution with minimal

number of product terms, and if several of those exist, it

will choose the one with minimal number of literals.

The simplest minimizer (level 1) will usually come as close

as 90% to 100% to the absolute minimum, but since it is

faster and more economical in memory usage than the absolute

minimizer, it can handle much bigger designs.

The minimizer can also be instructed to decide whether a

signal should be minimized in inverted polarity if this

yields a better result.  Also, it can be instructed to

generate hazard free logic which is invaluable for

asynchronous designs.  Different minimization strategies may

be used in the same design on different variables.

4  THE SIMULATOR FACILITY

The simulator facility is an event driven simulator which

operates at the same level of abstraction as the design

description.  Although not tuned for speed, it is invaluable

for checking circuit behavior at the conceptual level.  It

enables to specify and freeze state of inputs, nodes or

flip-flops, trigger individual flip-flops and print values

of signals or buses using formatted print statements.

The simulator facility can be used in conjunction with other

language elements such as loops and if-else statements,

function calls, use of temporary variables etc. to generate

complex stimuli.

Page 3

5  THE I/O FACILITY

Boolean equations format and Berkeley PLA table format are

directly supported through built in functions.  The compiler

can also read Berkeley PLA files.  Other formats can be

generated through the use of a combination of formatted

print facility (Similar to the one available in C) and other

built in functions.

Page 4