TABLE OF CONTENTS

TABLE OF CONTENTS                                          1

CHAPTER 1:  TUTORIAL                                       5

1.1 HOW TO LEARN BOOL                                  5

1.2 USING THE TUTORIAL                                 5

1.3 GETTING STARTED                                    6

1.4 VARIABLES, AND ASSIGNMENT STATEMENTS               7

1.4.1 EXAMPLE: 3-INPUT MAJORITY VOTE CIRCUIT       8

1.5 CONFLICTS AND THE OVERRIDING ASSIGNMENT           11

1.6 GROUP VARIABLES                                   13

1.7 OPERATORS AND EXPRESSIONS                         14

1.7.1 MIXING GROUPS AND SCALARS IN EXPRESSIONS    16

1.7.2 ALU EXAMPLE                                 17

1.7.3 ALU DESIGN USING THE .SWITCH STATEMENT      19

1.8 DON'T CARES. WHAT ARE THEY AND HOW ARE THEY GENERATED    20

1.9 STATE MACHINE DESIGN                              21

1.9.1 DESIGNING WITH JK FLIP-FLOPS                25

1.10 NUMERIC CONSTANTS                                29

1.11 LOOPS, CONTROL FLOW AND INTEGER VARIABLES        30

1.11.1 USING INTEGERS FOR THE MAJORITY VOTE CIRCUIT DESIGN

30

1.11.2 MORE INFORMATION ON FOR AND WHILE STATEMENTS32

1.11.3 USING INTEGER VARIABLES FOR STATE DEFINITION33

1.11.4 MORE ON INTEGER VARIABLES AND EXPRESSIONS  34

1.11.5 CONTROLLING LOOPS WITH BREAK AND CONTINUE STATEMENTS

35

1.11.6 EXAMPLE: A SIMPLE PRIORITY ENCODER         36

1.11.7 EXAMPLE: COUNTING BITS IN A GROUP          37

1.12 CONTROLLING PROGRAM FLOW WITH FUNCTIONS          37

1.12.1 EXAMPLE: COUNTING BITS, THE GENERAL CASE   39

1.12.2 LOCAL VARIABLES IN A FUNCTION              41

1.13 SIMULATING YOUR DESIGN                           42

1.13.1 USING THE VALUES STATEMENT                 43

1.13.2 USING THE PRINTF FUNCTION                  43

1.13.3 STRING VARIABLES                           44

1.13.4 MORE ON SIMULATION - NODES, FLIP-FLOPS AND DON'T

CARES                                     45

1.14 SELECTING THE PROPER MINIMIZATION ALGORITHM      46

1.14.1 MINIMIZATION POLARITY AND MINIMIZATION LEVEL46

1.14.2 HAZARD FREE LOGIC                          47

1.14.3 HAZARD FREE LOGIC TERMINOLOGY:             49

1.14.4 HAZARD FREE MINIMIZATION IN BOOL           49

1.15 A FEW WORDS ON THE PREPROCESSOR FACILITY         50

1.16 MORE ON PRINTING RESULTS AND FORMATTING DEFAULTS 51

1.17 INTRODUCTION TO THE STANDARD FUNCTIONS LIBRARY   52

1.18 PROGRAMMING TIPS                                 53

1.18.1 INTRODUCTION                               53

1.18.2 EXPRESSION FACTORING                       53

1.18.3 BEWARE OF PATHOLOGICAL EXPRESSIONS         54

1.18.4 PARTITION YOUR DESIGN                      54

CHAPTER 2: LANGUAGE REFERENCE MANUAL                      55

2.1 PROGRAM FORMAT AND STRUCTURE                      55

2.2 BASIC ELEMENTS - NAMES, VARIABLES AND OPERATORS   55

2.2.1 SYMBOLIC NAMES                              55

2.2.2 NUMERIC CONSTANTS                           56

2.2.3 DATA TYPES                                  57

2.2.4 GLOBAL VALUES                               57

2.2.5 VARIABLES                                   58

2.2.6 PREDEFINED VARIABLES - UNIVERSAL GROUPS     59

2.2.7 OPERATORS                                   60

2.2.7.1 OPERATIONS ON INTEGER DATA            60

2.2.7.2 OPERATIONS ON SCALAR BOOLEAN DATA     61

2.2.7.3 OPERATIONS BETWEEN SCALAR BOOLEAN DATA AND

INTEGER DATA                          61

2.2.7.4 OPERATIONS BETWEEN SCALAR BOOLEAN DATA AND A

GROUP.                                61

2.2.7.5 OPERATIONS ON GROUPS                  62

2.3 EXPRESSIONS                                       62

2.3.1 PRECEDENCE OF OPERATORS                     63

2.4 PREPROCESSOR DIRECTIVES                           63

2.5 STATEMENTS                                        64

2.5.1 DECLARATION OF GLOBAL VARIABLES             64

2.5.2 EXPLICIT GROUP DECLARATION                  65

2.5.3 DECLARATION OF ATTRIBUTES                   66

2.5.4 USING ATTRIBUTES IN A DECLARATION OF A VARIABLE    67

2.5.5 FUNCTIONS                                   69

2.5.6 DECLARING FORMAL PARAMETERS OF A FUNCTION   70

2.5.7 DECLARING LOCAL VARIABLES OF A FUNCTION     70

2.5.8 CALLING A FUNCTION AND PARAMETER PASSING    70

2.5.9 RETURNING FROM A FUNCTION                   71

2.5.10 COMPOUND STATEMENT                         72

2.5.11 SIMPLE ASSIGNMENT STATEMENT                72

2.5.12 OVERRIDING ASSIGNMENT                      73

2.5.13 CLEAR AND ASSIGN                           74

2.5.14 IF...ELSE STATEMENTS                       74

2.5.15 WHILE LOOP                                 75

2.5.16 FOR LOOP                                   76

2.5.17 .SWITCH STATEMENT                          77

2.5.18 SETPARAM STATEMENT                         78

2.5.19 VALUES STATEMENT                           80

2.5.20 END STATEMENT                              81

CHAPTER 3: RUNNING THE PROGRAM                            82

3.1 GENERAL                                           82

3.2 COMMAND LINE PARAMETERS                           82

CHAPTER 4: BUILT IN FUNCTIONS LIBRARY                     86

4.1 GENERAL INFORMATION                               86

4.1.1. PARAMETERS AND RETURNED VALUE              86

4.1.2 STATUS WORD OF A VARIABLE                   87

4.1.3 VARIABLE IDENTIFICATION NUMBER AND THE $v MACRO    88

4.1.4 ORIGINAL NAME OF A VARIABLE                 89

4.2 INPUT / OUTPUT HANDLING                           89

4.2.1 fileinfo    Accessing information on an output file

90

Page 2

4.2.2 get_source_name    get name of source file  90

4.2.3 open_inpfile    Opening a file for input    91

4.2.4 open_outfile    Opening a file for output   91

4.2.5 print    Printing equations in standard format     91

4.2.6 printf    Formatted print facility          92

4.2.7 printsim    Simple output of simulation results    94

4.2.8 readpla    Reading input files in PLA format94

4.2.9 writepla    Generating PLA output file      95

4.2.10 writestd    Generating several standard formats   95

4.3 MINIMIZATION                                      97

4.3.1 extmin    Calling an external minimizer     98

4.3.2 hazard_free   Generating hazard free logic  99

4.3.3 lminimize    Calling the built in minimizer100

4.3.4 minimize    Calling the built in minimizer 100

4.3.5 pminimize    Calling the built in minimizer101

4.4 SIMULATOR                                        101

4.4.1 clockff    Simulating a clock trigger to flip-flops

101

4.4.2 eval    Accessing the simulated value of a variable

102

4.5 ACCESSING AND MANIPULATING BOOLEAN DATA          102

4.5.1 clr    Clearing variables to an initial state.     103

4.5.2 ffj and ffk    Converting a flip-flop from D type to JK

103

4.5.3 getpt    Extracting a single product term  104

4.5.4 gr    Generating a group from a list of variables  105

4.5.5 pass   pass value of parameter             105

4.5.6 one    Extracting the on-set of a boolean  106

4.5.7 ptcomp    Comparing two product terms      106

4.5.8 valueof   Retrieve value of a boolean class variables

107

4.5.9 zero    Extracting the off-set of a boolean107

4.6 ACCESSING INFORMATION ON VARIABLES               107

4.6.1 getvarname    Accessing the name of a variable as a

string                                   108

4.6.2 isdc    Check if a boolean is global don't care    108

4.6.3 isone    Check if a boolean is global one  108

4.6.4 isused_input    Check for unused input literals    109

4.6.5 iszero    Check if a boolean is global zero109

4.6.6 lit    Number of input literals in a product term  109

4.6.7 litn    Total number of input literals     109

4.6.8 lser    Serial number of an input literal  110

4.6.9 lstat    Status of an input literal in a product term

110

4.6.10 minstat    Minimization status of a variable111

4.6.11 ptn    Total number of product terms      111

4.6.12 size    Number of elements in a group or array    112

4.7 ACCESSING INFORMATION ON STATUS WORD AND ATTRIBUTES OF

VARIABLES                                    112

4.7.1 add_dependents_status   Append to a Status Word List

113

4.7.2 add_literals_status   Append to a Status Word List 114

4.7.3 add_status_by_attrib   Append to a Status Word List

114

Page 3

4.7.4 get_attrib_num   Get numeric value of an attribute 115

4.7.5 get_attrib_status   Status of an attribute 116

4.7.6 get_attrib_str   Get string value of an attribute  116

4.7.7 get_list_status   Prepare a Status Word List117

4.7.8 get_literals_status   Prepare a Status Word List   117

4.7.9 get_status   Get Status Word of a Variable 118

4.7.10 general_attrib_info   Get general information on an

attribute                                118

4.7.11 general_attrib_num   Get general information on an

attribute                                119

4.7.12 general_attrib_str   Get general information on an

attribute                                119

4.8 STRING HANDLING                                  120

4.8.1 sprintf   Formatted conversion             120

4.8.2 strcat   Catenate two strings              121

4.8.3 strcmp   Compare two strings               121

4.8.4 strlen   Get length of a string            121

4.9 MISCELLANEOUS                                    122

4.9.1 exit   Abnormal termination of a program   122

4.10 OLD FUNCTIONS                                   122

4.10.1 length   Get length of a name of a variable122

4.10.2 printvarname    Print name of a variable  123

CHAPTER 5: ERROR AND WARNING MESSAGES                    124

5.1 GENERAL INFORMATION ON ERROR AND WARNING MESSAGES124

5.2 WARNING MESSAGES                                 126

5.3 DATA FILE ERROR MESSAGES                         128

5.4 OTHER ERROR MESSAGES                             130

CHAPTER 6: EXTERNAL LIBRARY                              147

6.1 GENERAL                                          147

6.1.1 GENERAL UTILITY FILES                      147

CHAPTER 7: APPENDIX                                      148

7.1 PLA TABLE FORMAT                                 148

7.2 LIST OF RESERVED WORDS                           151

INDEX                                                    152

Page 4

CHAPTER 1:  TUTORIAL

1.1 HOW TO LEARN BOOL

Our aim in this tutorial is to get you started as quickly as

possible.  By use of examples, the tutorial presents the essential

elements of the language.  We are not trying to be complete or even

precise. However, an attempt was made in the tutorial to present all

the necessary information for the beginner.   A concise but complete

definition of the syntax is presented in the reference part of the

manual. Other parts of the manual include explanations on advanced

topics that are not covered by the tutorial.

We suggest that you read the tutorial from start to end.  You may

skip some parts when you find in the text a recommendation to do so.

This may happen if you are not familiar with a high level,

programming language such as C or Pascal. In that case you may want

to leave some topics for a later time, after you get more familiar

with BOOL.

As we stated above, the tutorial is not complete, and does not cover

all the details. It includes however, all the elements that seem to

be necessary for designing state machines and combinational logic by

the use of a sequential (or behavioral) description.

In order to create or modify interfaces to other layout tools for

example, or to write designs in the most efficient way, a better

knowledge of BOOL and its functions library is needed.  However, we

recommend that you read the reference part of the manual only after

you tried several examples yourself, and feel comfortable with the

information in the tutorial.

1.2 USING THE TUTORIAL

The tutorial is arranged as a collection of informal definitions

along with examples and explanations. Each example is designed to

show different features of the language.

Page 5

TUTORIAL

1.3 GETTING STARTED

We start by writing a small program which writes on the screen the

message: "HELLO WORLD".  Running this program successfully will

verify that BOOL was installed correctly in your system.

Using your favorite editor or word processor, prepare the following

text file:

/* this is my first BOOL program,

it writes HELLO WORLD to the screen */

printf(1,"HELLO WORLD\n");

end;

A text file  which contains instructions to the BOOL compiler (Such

as this example) is referred to in the manual as "source code", or

"source" or just "code". Because of its similarity to a regular

computer program, the collection of all source code relating to a

single design is also referred to sometimes by the name "program".

Try to run the program using the following command line:

BOOL progname

where progname is the name you gave to the text file.  If nothing

goes wrong, the message HELLO WORLD followed by a carriage return,

should appear on the screen.

Even from this simple example we can learn several things:

Comments are written in /* ...  */

The last statement in the program must be the statement:

end;

Strings can be output by the use of the printf function.

Next, we run the same program again, but instead of writing the

message to the screen, we will write it to the file outfile.

Type the following command line:

BOOL  progname  outfile

outfile is the name of the file to receive the message.

Check the contents of outfile.  It should contain the one line

message:

HELLO WORLD

You can notice that here, the selection of an output file name is

done from the system command level. There is also a way to specify

the file name in the source code.

Page 6

TUTORIAL

Before proceeding into more examples, some general rules should be

mentioned on the syntax:

The input text has a free format.  You can start and end any

statement at any column in the line, have many statements on one

source line, or have one statement on several lines.  The number of

spaces, tabs and carriage returns is immaterial.  They are used only

for spacing and readability.  Spaces and tabs are significant

however, in strings (Strings are enclosed in "..." as in "HELLO

WORLD").

Case of characters is usually significant. However, the compiler will

recognize reserved keywords in either lower or upper case.  For

example, the keyword end may also be written as End or END because it

is a reserved keyword. On the other hand, user defined symbols and

function names are case sensitive.

1.4 VARIABLES, AND ASSIGNMENT STATEMENTS

All variables used in the program (Except local variables of

functions and function parameters, which will be discussed in a

different section) must be declared at the top of the program.

Variables that are declared at the top of the program are called

global variables since they are accessible by any part of the

program.

Variables are declared with symbolic names by which they can be

referenced later.  Symbolic names may consist of alphanumeric

characters, underscore and period, and may be up to 32 characters

long.   For example:  JOE_SMITH  JOE.SMITH  JoeSmith  Joe.Smith are

all valid variable names and are all different.  (case of characters

is significant).  One restriction is that symbolic names may not

start with a digit.

Variable are associated with values.  However, the values here have a

special meaning.  input and output variables belong to a class of

variables called boolean class variables.  This class contains the

types: input, output, node, flip_flop and boolean. Later we will

introduce a new type of variable called group which is also a boolean

class variable.

A boolean class variable is said to hold a boolean data type, which

is a boolean function of other network elements such as inputs, nodes

or flip-flops.

Input, output, node and flip_flop variables represent signals that

are synthesized to logic and are referred to, therefore, as network

elements, while boolean variables are used to hold temporary values.

In the following drawing we show an example of each of the types

(Except the boolean type).

Page 7

TUTORIAL

+----------------------------------------------------------+

|                                                          |

|       +------+          +-------------+                  |

| IN1 -->      |          |             |                  |

|       |  OR  >----------> D  FF     Q >--------> FF1     |

| IN2 -->      |          |             |                  |

|       +------+          +-------------+                  |

|                           +-------+  +-------------->ND1 |

| IN3 ---------------------->       |  |   +-------+       |

|                           |  NOR  >--+--->       |       |

|                        |-->       |      |  NOR  >-->OUT1|

|          +------+  ND2 |  +-------+   +-->       |       |

| IN4 -----> INV  >------+              |  +-------+       |

|          +------+      |--------------+                  |

+----------------------------------------------------------+

The BOOL source code that describes the above circuit may look like

the following:

input  IN1, IN2, IN3, IN4;

output  OUT1;

flip_flop  FF1;

node  ND1, ND2;

OUT1 = ! (ND1 | ND2) ;  /* assignment to an output */

FF1:d = IN1 | IN2 ;     /* assignment to a flip-flop */

ND1 = ! (IN3 | ND2) ;   /* assignment to a node */

ND2 = ! IN4 ;   /* assignment to a node */

1.4.1 EXAMPLE: 3-INPUT MAJORITY VOTE CIRCUIT

Let us consider the following Example: We have to design a circuit

which performs a majority vote.  The inputs to the circuit are three

signals named: A,B,C.  The outputs are two signals named: RESULT,

ERROR.  The signal RESULT should have the value 1 if at least 2 of

the three signals A,B,C are 1, and the value 0 otherwise.  The signal

ERROR should be 0 if all the signals A,B,C have the same value, and

the value 1 otherwise.  The program to solve this problem is printed

below, and can also be found in the file MAJORV1.B (A more elegant

solution which solves the general case is presented later).

Page 8

TUTORIAL

input A,B,C;

output  RESULT,ERROR;

if (A == B)

{  if (B == C) ERROR = 0; else ERROR = 1;

RESULT = A;

}

if (B == C)

{  if (A == B) ERROR = 0; else ERROR = 1;

RESULT = B;

}

if (A == C)

{  if (A == B) ERROR = 0; else ERROR = 1;

RESULT = A;

}

minimize(ERROR,RESULT);

print(ERROR,RESULT);

end;

Again, you can direct the results to the screen or to an output file,

by running the program with one of the following command lines:

BOOL MAJORV1.B

or:

BOOL MAJORV1.B outfile

The output should look like the following:

ERROR = !B & C |

B & !C |

!A & C |

A & !C ;

RESULT = B & C |

A & C |

A & B ;

Let us explain now the program and the results.

First, we notice that the input signals are declared in the program

with the keyword input and the output signals are declared with the

keyword output. The keyword inp is also accepted as a synonym for

input, and out is accepted as a synonym for output.

A,B,C,RESULT,ERROR are variables. Referring to the explanation above

on variables, A,B,C are inputs to the circuit and are therefore input

variables. RESULT and ERROR  are outputs from the circuit and are

therefore output variables.

The value that a boolean class variable can hold is not a number as

in other programming languages, but a symbolic function of the input

variables.  When printed with the print function, the value is

written to the output as a sum of product terms, where each product

Page 9

TUTORIAL

term is the logical AND of several input variables.  Each of the

inputs can appear inverted or uninverted in the product term.  Each

symbol whether inverted or not, that appears in a product term is

called an input literal or just a literal. The equation of ERROR has

4 product terms, and the equation of RESULT has 3 product terms.

Each of the product terms has 2 input literals.

The character | is an OR operator, the character & is an AND

operator, and the character ! stands is the NOT operator (inverse).

We use the terms boolean equation or boolean function to refer to the

sum of products representation of a boolean class variable.

A semicolon is used to end each equation in the output file.

An input variable cannot hold symbolic values like other boolean

class variables, or to be more accurate, it holds a constant symbolic

value which is its own name.  Therefore, assignments cannot be made

to input variables.

The program itself can be written in several ways, where each way

reflects a different approach to the problem.  The approach we chose

to solve the problem, is the following:

Chose any two inputs.  If they are not equal, don't make any decision

yet.

If they are equal, then RESULT should have their value.  (since they

are equal, we can assign the value of any of them to RESULT).  Check

the third input.  If it also has the same value then the ERROR signal

should be 0.  If it has a different value, then the ERROR signal

should be 1.

Perform the same checking for any two inputs.  (A total of three

checks.)

Lets analyze more carefully how this is written.  We shall analyze

the following fragment from the program:

if (A == B)

{  if (B == C) ERROR = 0; else ERROR = 1;

RESULT = A;

}

This fragment is one of the three checks mentioned above.

The first line: if (A == B) checks if A is equal to B.  The ==

defines the "equal to" operator.  Everything inside the braces {...}

belongs now to this check, and executes under the assumption that A

is equal to B.  There is a special definition for that:  Everything

inside the braces is called a compound statement, or a block.  We say

that all the statements in the above compound statement are executed

within the scope of the condition A == B.    Compound statements are

a powerful construct that is available in all structured high level

languages.  It can be used wherever a simple statement can be used.

Page 10

TUTORIAL

The second line assigns a value to ERROR.  If B and C are equal, then

the value 0 is assigned to ERROR.  The statement ERROR = 0; is called

an assignment statement.

The rest of the line:  else ERROR = 1;  specifies that a value of 1

is assigned to ERROR if the condition is false (B does not equal C).

Notice that instead of writing if (B == C) we could write

if (A == C) and it would have been the same, because the statement is

executed under the assumption that A is equal to B.

The third line: RESULT = A; simply assigns the value of A to RESULT.

Again, we can also write: RESULT = B; and it would make no

difference.

The statement:   minimize(ERROR,RESULT);   invokes the minimizer to

minimize the number of product terms and input literals in the

equations of ERROR and RESULT.  Here, the simplest minimization

algorithm is invoked.  Later, you will learn how to call other

minimization algorithms.

At last, the statement:

print(ERROR,RESULT);

Prints the results as boolean equations.

Later we will show a different, more elegant approach to solving the

same problem, by counting the number of ones and zeros in the input.

1.5 CONFLICTS AND THE OVERRIDING ASSIGNMENT

BOOL contains a powerful mechanism for detecting design errors, by

flagging conflicts in the description.

What is a conflict?  Simply put, a conflict is the case where a

boolean class variable is assigned different values under the same

condition. Consider the following source code:

output X;

X = 1;

X = 0;

In this case X is assigned once the value 1 and then the value 0,

which is an obvious conflict. Other conflicts may be more subtle. for

example:

input A,B;

output X;

if(A==1) X = 1;

if(B==0) X = 0;

Page 11

TUTORIAL

In this example, as long as A is not 1 or B is not 0 there is no

conflict. However, if both A is 1 and B is 0, there is a conflict.

BOOL will detect the conflict while compiling the source code and

will produce a warning as follows:

** warning ** line: nn col: mm  new value overrides old value.

nn and mm point to the exact location in the source file where the

conflict was detected.

What the above message mean is that a conflict was detected.  Under

those conditions, where the conflict exists, the last assignment will

take place.  In this example, if A is 1 and B is not 0 then X is 1,

however if B is 0 then X is 0 regardless of the value of A.

Sometimes, we want to avoid the warning message as in the following

problem:  An output X depends on its inputs A,B,C as follows: If all

three inputs A,B,C are 0 than the output is 0, otherwise, the output

should equal the value of a fourth input D.

One way to formulate it is by using an if-else pair as follows:

input A,B,C,D;

output X;

if(A==0 & B==0 & C==0) X = 0;

else X = D;

However, to demonstrate our point we show an alternative way. In the

following example, X is assigned a default value with the statement X

= D; and then an additional assignment defines different values for

X.

input A,B,C,D;

output X;

X = D;

if(A==0 & B==0 & C==0) X = 0;

This piece of code, although giving the correct result, generates a

warning message because there is a conflict when all three inputs

A,B,C are 0, and D is 1. To avoid the warning, we can use a special

assignment statement, called overriding assignment, as follows:

input A,B,C,D;

output X;

X = D;

if(A==0 & B==0 & C==0) X := 0;

The overriding assignment uses the characters: := instead of the

single character = .  Its function is the same as the regular

assignment, but it instructs the compiler to ignore any conflicts.

In the case of a conflict, the new value simply overrides the

Page 12

TUTORIAL

previous values.  In the above example, X will have the value of D.

However, if all three inputs A,B,C are 0, X will be forced to 0.

To summarize, the overriding assignment allows us to assign and re-

assign values to a variable.

1.6 GROUP VARIABLES

In many cases, inputs or outputs are grouped together, and it is more

convenient to refer to this group with one name.  Another incentive

to use groups of signals is that some operations are defined only on

groups of signals.  As an example, a circuit called magnitude

comparator has two groups of signals as inputs, and produces three

outputs.  The magnitude comparator compares the two groups of signals

as unsigned integers, and asserts one of the three outputs labeled as

EQ, GT or LT ( for: EQUAL, GREATER THAN and LESS THAN.)

In the following example we design a 4-bit magnitude comparator,

using a new data type called group.

input A[4],B[4];

output EQ,GT,LT;

if (A == B) EQ = 1; else EQ = 0;

if (A > B) GT = 1; else GT = 0;

if (A < B) LT = 1; else LT = 0;

minimize(EQ,GT,LT);

print(EQ,GT,LT);

end;

Each of the variables A,B represent a group of four input variables,

and is therefore referred to as "input group".   Groups are

especially useful when the data they contain is associated with

numbers (As in this example) and not just zeros and ones.  For

example, the new operators that we introduced here: > (Greater than)

and < (Less than) operate on groups (as unsigned integers) but not on

scalar boolean variables.  Also note that the == operator which was

used for scalar variables can operate also on groups, and has the

same meaning as an equality operator between numbers (meaning that

all bits are equal).

Each of the variables that compose a group is referred to as: element

of a group.  A variable such as EQ or A1 is called a scalar variable

because it represents a single signal.

The size of a group is defined as the number of elements in the

group.   Different groups may have different sizes.  Except for

variable size boolean groups (which are not discussed in the

tutorial) and formal parameters of functions, the size of all groups

is fixed and is specified in the declaration statement of the group.

Page 13

TUTORIAL

Group elements are always boolean class variables.  Since groups

represent a collection of symbolic boolean equations, they also

considered as boolean class variables.

A and B are each a group of 4 input signals. The individual signals

in A can be referred to by the names A[0], A[1], A[2] and A[3].  An

alternative way to define groups is by using an explicit group

declaration The following examples shows how the declaration part of

the previous example can be re-written:

input A3,A2,A1,A0,B3,B2,B1,B0;

group A = [A3,A2,A1,A0];

group B = [B3,B2,B1,B0];

output EQ,GT,LT;

if (A == B) EQ = 1; else EQ = 0;

if (A > B) GT = 1; else GT = 0;

if (A < B) LT = 1; else LT = 0;

minimize(EQ,GT,LT);

print(EQ,GT,LT);

end;

Here, the input signals are first declared explicitly as A3,A2 etc.

and then they are given group names with the statement that starts

with the group keyword.  The individual signals in the group A can be

referred to by two names. For example the first element in A can be

referred to as either A[0] or as A0.

We should also mention that explicit group declarations may use

elements from other groups as part of the declaration.  For example

examine the declaration:

input A[10],B[10],C[10];

group X = [ A, B, C ];

group Y = [ A[0], B[0], C[0] ];

group Z = [ A[9,7], B[9,7], C[9,7] ];

This declaration declares group X as a 30 elements group containing

all elements from A, B and C.  Group Y is declared as a 3 elements

group containing only the first element from each of A, B and C.

Group Z is declared as a 9 elements group containing the last 3

elements (elements 7 through 9) from each of A, B and C.

The ordering of elements in explicit group declaration is from right

to left.  In the example above, the first element of group Y can be

referenced as either Y[0] or as C[0].

1.7 OPERATORS AND EXPRESSIONS

In the previous sections we were introduced to the three basic

operators:  & | !  and also the ==  > and < operators.  We mention

here a few more operators:

Page 14

TUTORIAL

XOR operator (exclusive or) as in:  A ^ B .

Not equal operator, as in:  A != B .Greater equal operator, as in A

>= B

Less equal operator as in A <= B

Arithmetic plus operator as in A + B

Arithmetic minus operator as in A - B

The last four operators cannot be used on scalar boolean class

variables but only on groups. Later we will explore examples that

make use of the arithmetic operations.

The operators == > < >= <= are called relational operators.   Their

use is not limited to conditions inside if() statements. They are

legal boolean operators which always produce scalar boolean results.

For example, the == is equivalent to the XNOR (not XOR) operation

(for scalar variables).  Referring to the example of the magnitude

comparator above, instead of the statement:

if (A == B) EQ = 1; else EQ = 0;

We can use the statement:

if (!(A ^ B)) EQ = 1; else EQ = 0;

Furthermore the above statement is equivalent to:

EQ =  A == B;

All three forms are equivalent. In the same way, the != (not equal)

operator is equivalent to the ^ (XOR) operator.  Now it becomes clear

how the compiler interprets the expression inside the parentheses of

the if() statement:

The boolean expression inside the parentheses is evaluated.  This

expression is considered to be true if it has the value 1, and false

if zero.

Following this explanation, we can also conclude that the statement:

if (G == 1) Y = X;

is equivalent to:

if (G) Y = X;

In the same way that the statement:

if (H == 0) Q = P;

is equivalent to:

if (!H) Q = P;

For completeness we mention also the operators:

Page 15

TUTORIAL

~       bitwise inverse

@       grouping operator

*       multiply operator

/       divide operator

>>      shift right operator

<<      shift left operator

The grouping and the shift operators are explained below in the ALU

example.  The other operators are explained later in the section on

integer variables.

Operators are categorized into two groups: unary operators and binary

operators.  Unary operators are operators that operate on a single

operand as in !X.  Binary operators require two operands as in:

X & Y.

The operators ! and ~ appear only in unary form. The operators @ and

- appear in both unary and binary form.

What about precedence of operators?

The order of precedence is similar to the one defined in the C

programming language.  The list below is arranged in decreasing

precedence order, where operators with same precedence are in the

same line:

All unary operators ~  !  @  -  have highest precedence.

Binary operators come next in the following order:

HIGHEST PRECEDENCE

@

*  /

+  -

>>  <<

<  >  <=  >=

==  !=

&

^

|

LOWEST PRECEDENCE

Parentheses can be used to change order of evaluation, as in the

following example:

R = X + (Y & (Z | Q));

1.7.1 MIXING GROUPS AND SCALARS IN EXPRESSIONS

The operators & ^ | accept also a mix of scalar and group operands.

The specified operation in that case is performed between the scalar

and each of the elements of the group to yield a group result with

the same number of elements.

Page 16

TUTORIAL

1.7.2 ALU EXAMPLE

Our next example is the design of a four bit ALU slice.  The ALU has

three groups of inputs: A,B,C, each composed of 4 inputs.  A,B are

the data inputs and C is the control input.  The type of operation

done between A and B is defined according to the value in the C port.

The result from the ALU is the four elements group S.  The ALU also

has a CYIN input and a CYOUT output which are the carry input and

carry output when doing an add operation, and serve as borrow input

and borrow output when doing subtract operation.

The following table defines the type of operation according to the

value of C :

Value of C        operation

----------------------------------------------------------

0             S = A AND B

1             S = A AND NOT B

2             S = A XOR B

3             S = A OR  B

4             S = NOT A

5             S = NOT B

6             S = shift A right one position

7             S = shift A left one position

8             S = A + B + CYIN          (arithmetic)

9             S = A - B - CYIN          (arithmetic)

In the subtract operation (C = 9), the carry signal serves as the

borrow.  If the result of the subtraction has a borrow, CYOUT is 1,

otherwise, CYOUT is 0.

According to this definition, ALU slices can be cascaded by

connecting the CYIN of one ALU to the CYOUT of another ALU.  The

value of CYOUT is defined only for operations 8,9 (add, subtract).

Following is the complete ALU program:

Page 17

TUTORIAL

input A[4], B[4], C[4];

input  CYIN;

output  S[4],CYOUT;

group SX = [CYOUT,S];

if (C == 4@0) S = A & B;                /* and */

if (C == 4@1) S = A & !B;               /* and not */

if (C == 4@2) S = A ^ B;                /* XOR */

if (C == 4@3) S = A | B;                /* or */

if (C == 4@4) S = !A;                   /* not A */

if (C == 4@5) S = !B;                   /* not B */

if (C == 4@6) S = A >> 1;               /* shift right*/

if (C == 4@7) S = A << 1;               /* shift left */

if (C == 4@8)

SX = gr(5,A) + gr(5,B) + gr(5,CYIN); /* A + B + CYIN */

if (C == 4@9)

SX = gr(5,A) - gr(5,B) - gr(5,CYIN); /* A - B - CYIN  */

minimize(SX);

print(SX);  /* print all 4 elements of S and CYOUT */

end;

We introduced here the shift operators >> and << which perform right

and left shift respectively on groups.  Any number of shift positions

can be specified as in:  X >> 3 which will cause a right shift by 3

positions. During shifts, vacant bits are filled with zeros.

We also introduced the grouping operator @ which operates on integer

numbers and produces a group result.

We will see later that BOOL recognizes integer variables that can

also be used with the grouping and shift operators.

To understand the grouping operator, examine the expression

(C == 4@3).  Intuitively, we understand that this expression is 1

(true) if the elements of C have the value of 3 (binary: 0011).

The number 4 to the left of the @ defines that we are dealing with 4

elements.  4@3 is therefore a group which has 4 elements with values

that give the decimal code 3.

Although A, B and S are each 4 elements group, the operations + and -

were carried out with 5 bits arithmetic in order not to lose the

carry (or borrow) bit. This was achieved by extending A, B and S to 5

elements groups. SX  was derived from S by extending it with CYOUT as

the most significant element by using the group declaration. A and B

were extended by using the function gr.

gr is a built in function (called a grouping function) that builds a

group with a specified number of elements.  The first argument (the

number 5 in the example) is the number of elements we want in the

group. All other arguments are used as elements. For example,

gr(5,A) is a 5 elements group with four least significant elements

taken from the group A and a zero most significant element.

Page 18

TUTORIAL

gr(5,CYOUT) is a five elements group with CYOUT as the least

significant element. All other four elements are zero.

gr can accept a variable number of arguments, groups, scalars or

integer.  For example, gr(9,1,A,B) is a nine elements group,  With

elements from B occupying the first 4 positions, A occupying the next

4 positions, and a constant 1 as the most significant element.   Note

that if the first argument to gr is 0, all elements from all other

arguments are included, so gr(9,1,A,B) in this case is equivalent to

gr(0,1,A,B).

Let us investigate now how groups are handled in several cases.

print(SX);

is equivalent to:

print(S[0],S[1],S[2],S[3],CYOUT);

The boolean operators ! & ^ | are used in this example on groups.

When applied to groups, these operators yield a result which is also

a group, by operating on each element separately.

For example, the statement:

S = A & B;

is equivalent to the four statements:

S[0] = A[0] & B[0];

S[1] = A[1] & B[1];

S[2] = A[2] & B[2];

S[3] = A[3] & B[3];

1.7.3 ALU DESIGN USING THE .SWITCH STATEMENT

The ALU program presented in the previous section can be written in a

more elegant way with the .SWITCH statement:

Page 19

TUTORIAL

input A[4], B[4], C[4];

input  CYIN;

output  S[4],CYOUT;

group SX = [CYOUT,S];

.switch (C)

{   case 0: S = A & B;                  /* and */

case 1: S = A & !B;                 /* and not */

case 2: S = A ^ B;                  /* XOR */

case 3: S = A | B;                  /* or */

case 4: S = !A;                     /* not A */

case 5: S = !B;                     /* not B */

case 6: S = A >> 1;                 /* shift right */

case 7: S = B << 1;                 /* shift left */

case 8: SX = gr(5,A) + gr(5,B) + gr(5,CYIN);/*add*/

case 9: SX = gr(5,A) - gr(5,B) - gr(5,CYIN);/*subtract*/

}

minimize(SX);

print(SX);

end;

This program is equivalent to the previous one, and will give exactly

the same results.

The character @ may be used instead of the keyword case. For example,

instead of:

case 1: S = A & !B;

one my write:

@1: S = A & !B;

Notice that All the if statements were replaced with one .switch

statement.  The .switch statement accepts inside the parenthesis a

group variable (C in our example).  According to the value

represented by the group, one of the case values is selected. All

statements starting from that case up to the next case (or the

closing "}") are executed.

The .switch statement in BOOL has some similarities to the switch

statement in the C programming language, however there are also some

differences which should not confuse the reader.  For this reason, it

was given a slightly different name.  The .switch statement is mainly

used in BOOL to construct tables or define state machines. Its use

for state machines will be discussed later.

1.8 DON'T CARES. WHAT ARE THEY AND HOW ARE THEY GENERATED

In the ALU design we defined the value of S only for values of C

between 0 and 9.  But C has 4 bits and can assume values in the range

0 to 15.  What happens to S for values of C between 10 and 15 ?

Page 20

TUTORIAL

The answer, as you have probably guessed by now, is that since we did

not define what happens to those values, they remain don't cares (or

undefined).

Generally, we state that anything that is not explicitly defined,

remains don't care.  Since these values remain don't cares, the

minimization algorithm can assign any value to S for values of C

between 10 and 15.  Giving the minimizer the freedom to choose values

for the don't cares, can produce a better solution.  CYOUT has even

more don't cares.  It is defined only for C=8 or C=9.  It has don't

cares for any other values of C.

As you can see, the use of don't cares is similar to our way of

thinking.  You don't have to define don't cares explicitly.  Instead,

concentrate on the things you want your design to do, define them,

and all the rest is considered as don't care.

To be more specific, values that are generated under some condition

that depends on other signals (Through the use of an if or .switch

statements) will imbed don't cares which relate to the case when the

condition is false.

Since what you don't define remains don't care, we say that undefined

is a synonym for don't care.

A boolean variable (or expression) that imbeds don't cares is called

partially specified because the output is not specified (defined) for

some combinations of input values.   We say on this variable that it

has a non empty don't care set.

The other case - a boolean variable that does not imbed any don't

cares is called fully specified since its value is specified for all

combinations of input values.  We say on this variable that it has an

empty don't care set.

1.9 STATE MACHINE DESIGN

To demonstrate the methodology of a state machine design, we start

with a very simple state machine that is designed in the classic way.

The problem is defined as follows:

A state machine has an input A and an output Q. The state machine has

three states: 0,1,2.  The state machine goes from state to state in a

cyclical order: 0->1->2->0 etc. However, in state 0, the state

machine will proceed to state 1 only if A is 1. If A is 0, The state

machine should stay in state 0. Regardless of the input, the output Q

will be 1 when in state 1 and 0 in states 0 and 2.

input A;

output Q;

flip_flop S[2];  /* two bit state variable */

Page 21

TUTORIAL

.switch(S)

{

case 0: if(A == 1) jump 1;

else stay;

Q = 0;

case 1: jump 2;

Q = 1;

case 2: jump 0;

Q = 0;

}

minimize(S,Q);

print(S,Q);

end;

In the above example we introduced a new type of variable called

flip-flop. Actually the statement: flip_flop S[2]; defines a group of

two flip-flops which are used in this case as the state variable.

The keyword ff may be used instead of flip_flop (with exactly the

same meaning).

When used in a .switch statement, the keywords jump and stay may be

used to define state transitions, which are actually a definition of

the next state of each flip-flop as a function of the present state

(The outputs of the flip-flops) and the inputs to the circuit.

The statement:   jump 2;   indicates that there is a transition to

state 2 (from state 1).  It is actually equivalent to assigning the

value 2 to the state variable S.  In the same way, the statement

stay;  indicates that the state machine should stay in the current

state which is state 0.  In this case, the stay statement is

equivalent therefore, to assigning the value 0 to the state variable.

In the next example we explore a more complex design problem, and

show that by using an approach that is different from the classic we

can write a description that is shorter and easier to understand.

The problem is the design of a synchronous four bit decimal counter,

with parallel load inputs, synchronous master clear, and carry out.

Let us first define the design goals in more details.

The counter is based on four D-type flip-flops. The flip-flops are

grouped together under the name F and are therefore accessible as:

F[3],F[2],F[1],F[0].

Inputs are:

1. P[3],P[2],P[1],P[0]  - four data inputs used for parallel load.

2. COUNT - count enable input.  The counter should count only when

this line is 1.

3. LOAD - when this line is 1, the counter is loaded with the

contents of P[0]-P[3], and counting is inhibited regardless of the

value of COUNT.

Page 22

TUTORIAL

4. CLEAR - When this line is 1 the counter is cleared to zero

synchronously, regardless of the values of LOAD and COUNT.

The outputs from the counter are F[0] to F[3] and CARRY which is 1

when the counter reaches the value 9.  There are no restrictions on

the value of CARRY when LOAD or CLEAR are asserted.  Cascading two

counters by connecting the CARRY of one counter to the COUNT of the

next counter requires that CARRY should be 1 only when the counter

reaches the number 9 and COUNT is asserted.

Our task is to write a BOOL program that will compute the equations

of the inputs to the flip-flops and CARRY as a function of the

outputs of the flip-flops and the inputs COUNT,LOAD,CLEAR and P[0] to

P[3].

And here is the program:

/* declarations */

flip_flop  F[4];   /* four flip-flops */

input P[4];        /* parallel load inputs */

input COUNT,LOAD,CLEAR;

output CARRY;

/* executable part */

if (CLEAR)

F:d = 4@0; /* reset F to 0 */

else if (LOAD)

F:d = P;  /* load F with the inputs P */

else if (COUNT == 0)

{

F:d = F;   /* stay in the current count */

CARRY = 0; /* keep carry at zero */

}

else    /* if all the conditions above are false - count */

{

CARRY = 0;    /* define a default value for CARRY */

.switch (F)

{ case 0: jump 1;

case 1: jump 2;

case 2: jump 3;

case 3: jump 4;

case 4: jump 5;

case 5: jump 6;

case 6: jump 7;

case 7: jump 8;

case 8: jump 9;

case 9: jump 0;

CARRY := 1;    /* override the default value */

}

}

minimize(F,CARRY);  /* minimize results */

print(F,CARRY);     /* print as boolean equations */

end;

Page 23

TUTORIAL

You can notice that some shortcuts were done, in order to save

writing.  The .switch statement is used here to define the state

transitions during counting only.  All the rest is done by if and

else statements.

Let us first analyze the structure which appears at the top of the

program:

if(condition1) statement1;

else if(condition2) statement2;

else if(condition3) statement3;

else statement4;

statement5;

For those of you who are familiar with programming in a high level

language, this structure might look familiar.  And indeed, this works

similar to the if-else-if structure used in other programming

languages.  The general idea is that condition1,condition2 and

condition3 are checked in order. If any of them is found to be true

(or 1) the corresponding statement is executed, and execution

proceeds from statement5. If none is true, statement4 is executed.

In any case, one and only one of statement1, statement2, statement3

or statement4 is executed.

The above example shows us how to write assignments to flip-flops.

For example, the statement  F:d = 4@0; assigns a value of 0 to the

inputs of all four flip-flops.  In the same way, if we wanted to

assign a value of 0 only to the input of F[0], we would write: F[0]:d

= 0;    The reason for this notation is as follows:  The name F

refers to the output of the flip-flops, which may or may not

necessarily contain the same value as the inputs (which are referred

to by F:d).  The contents of the inputs are transferred to the

outputs only when a clock is applied.  For this reason, different

notations are used for the inputs and the outputs of flip-flops.   A

good example is the statement F:d = F; which implies a feedback from

the outputs of the flip-flops to their inputs. It means actually that

when a clock is applied, the contents of the flip-flops should not

change.

Inside the braces of the .switch statement, the statement jump 1; is

equivalent to the statement  F:d = 4@1;  This is of course, only a

different way to write it, but it makes the program more easy to

understand, and it enforces a standard format for writing state

machines.

The series of jump statements, defines that the state machine "jumps"

from one state to the next, until it reaches 9, and then "jumps" to

0.

The .switch statement is actually internal to if statements, which

demonstrates that you don't have to think of a state machine

description as something with rigid rules.  If we wanted to describe

the state machine according to the classic way, we should move all

Page 24

TUTORIAL

the checks to inside of the .switch statement, and define the outputs

with the appropriate checks explicitly for each state.  This method

would result in a much bigger program which is more difficult to

understand and maintain.

Lets examine the shortcut made with CARRY.  Instead of assigning a

value to it every state, we defined a default value of 0 for all the

cases where it is defined at all.  Then, only for state 9 we override

the above assignment with a value of 1.  This new assignment is in

conflict with the previous assignment of 0, but no warning message is

generated because we use the overriding assignment statement  CARRY

:= 1;

Notice that the value of CARRY was not defined if CLEAR or LOAD are

asserted.  This means that if CLEAR or LOAD are asserted, then CARRY

has a don't care value.  Also, when counting, both CARRY and F:d are

undefined (or don't care) for values of F above 9.

Since the jump  statement is actually equivalent to an assignment

statement, if it conflicts with a previous assignment to the state

variable, you will get a warning message similar to a conflict during

assignment.   In some cases the conflict is intentional. For example

when you assign a default value to the state variable (This will

define a default next state for all unspecified states).   To avoid

the conflict message in an assignment statement, you will use the

overriding assignment (Using ':=' instead of '=').   Here, in a

similar way, to avoid a conflict message, you may use the xjump

keyword instead of jump and the xstay keyword instead of stay.

1.9.1 DESIGNING WITH JK FLIP-FLOPS

Our next goal is to carry out the above design with JK flip flops.

The use of JK flip-flops can produce more compact equations.

Fortunately, we don't have to rewrite the whole design, but we can

adopt the results to JK flip flops by using a transformation from a D

flip-flop to a JK flip flop.

The next example shows how this is done in practice. However, if you

are interested in the theory how the transformation is done, here is

the explanation. (If you are not interested in the theory, you can

skip this part).

If the j and k inputs and the output q of a single flip flop are

known, then the next state d of the flip flop is given by the

following table:

j   k  -->  d

--------------

0   0       q

0   1       0

1   0       1

1   1       !q

Page 25

TUTORIAL

We are actually interested in the inverse of this table, because we

know the next state d and the present state q, but we don't know the

value of j and k.

From the above table we can conclude the inverse table which is given

below:

X stands for don't care.

q    d  -->   j    k

--------------------

0    0        0    X

0    1        1    X

1    0        X    1

1    1        X    0

We can see from this table that if q is 0, then j gets the value of

d, and k is a don't care.  If q is 1 then k is the inverse of d, and

j is a don't care.

Using BOOL syntax this is written as:

if (q == 0) j = d;  else k = !d;

As we have already learned, if the name of the flip-flop is M then

the output q is written as M and the input d is written as M:d.

Using this formula we can convert any D flip-flop design to a JK

design.   In our design we have to apply this formula separately to

each of the four flip flops.  BOOL has built in functions that can do

it for us.  The functions are ffj and ffk and they are called by the

statements:

J = ffj(M);

K = ffk(M);

The function ffj calculates the j input to a JK flip-flop whose name

is M.  ffk calculates the k input based on the same assumptions.  J

and K are temporary boolean variables that will hold the resulting

equations. (An explanation on this type follows below after the

example).  The built in functions ffj,ffk can also operate on a group

of flip flops, by repeating the above formula for each flip flop

separately.

To make things clear we bring here the same design, but now we

calculate the equations relating to JK flip-flop, and not to D

flip-flops.

Page 26

TUTORIAL

/* declarations */

flip_flop  F[4];     /* four flip-flops */

input P[4];          /* parallel load inputs */

input COUNT,LOAD,CLEAR;

output CARRY;

boolean J[4],K[4];   /* J,K inputs of the flip flops */

/* executable part */

if (CLEAR)

F:d = 4@0; /* reset F to 0 */

else if (LOAD)

F:d = P;  /* load F with the inputs P */

else if (COUNT == 0)

{

F:d = F;   /* stay in the current count */

CARRY = 0; /* keep carry at zero */

}

else    /* if all the conditions above are false - count */

{

CARRY = 0;    /* define a default value for CARRY */

.switch (F)

{ case 0: jump 1;

case 1: jump 2;

case 2: jump 3;

case 3: jump 4;

case 4: jump 5;

case 5: jump 6;

case 6: jump 7;

case 7: jump 8;

case 8: jump 9;

case 9: jump 0;

CARRY := 1;    /* override the default value */

}

}

J = ffj(F);

K = ffk(F);

minimize(J,K,CARRY);

print(J,K,CARRY);

end;

We introduced here a new variable type declared with the keyword

boolean (BOOL is also accepted as a synonym for boolean). In the

above example we declared two groups: J and K, each of type boolean

and each having 4 elements.

The boolean variable type is similar to the output type because like

the output type it represents a boolean function, but it does not

represent any input literal.   But unlike the output type which

represents a signal (or wire) in the circuit, the boolean type is

used only to hold temporary values. Syntactically there is almost no

difference between the two types, and in almost all cases they will

be treated the same.

Page 27

TUTORIAL

However, when we discuss later library functions that interface to

technology specific tools, we shall see that all input, output, node

and flip-flop variables are synthesized by the interface functions to

logic, and appear in the output file.  boolean variables, on the

other hand, are ignored by these functions.

Notice that the program almost did not change.  After calculating F:d

we calculate J and K using the results obtained for F:d.  The don't

care conditions which F:d has, are propagated to the J and K, which

means that all conditions which were not defined for F:d are also a

don't care in J and K.

As an additional exercise, try to carry out the same state machine

with RS flip flops

Hint: By the use of a table, show that the r and s are calculated

from the q and d by:

if ((q & d) == 0) s = d;

if ((q | d) == 1) r = !d;

Later, you will learn that you can write your own function and use it

the same way you used the functions ffj,ffk.

For completeness it should be mentioned that the whole .switch

statement can be replaced by something even more compact.  Since the

value of the next state of the flip-flop always equals the value of

the present state plus 1 (For counts 0 to 9) we can simply define it

with an arithmetic add operation. The final version of the counter,

using this method is shown below:

Page 28

TUTORIAL

/* declarations */

flip_flop  F[4];   /* four flip-flops */

input P[4];        /* parallel load inputs */

input COUNT,LOAD,CLEAR;

output CARRY;

/* executable part */

if (CLEAR)

F:d = 4@0; /* reset F to 0 */

else if (LOAD)

F:d = P;  /* load F with the inputs P */

else if (COUNT == 0)

{

F:d = F;   /* stay in the current count */

CARRY = 0; /* keep carry at zero */

}

else    /* if all the conditions above are false - count */

{

if (F <= 4@8)  /* F between 0 and 8 */

{ F:d = F + 4@1;   CARRY = 0;  }

if (F == 4@9)  /* F equals 9 */

{ F:d = 4@0;       CARRY = 1;  }

}

minimize(F,CARRY);

print(F,CARRY);

end;

1.10 NUMERIC CONSTANTS

Numeric constants can be written as either decimal, hexadecimal,

octal, binary or as a value of an ASCII character. For example  1111B

is a binary number which represents the value 15.  The suffix B tells

the compiler that the number should be interpreted as a binary

number. In the same way, 1C3H is interpreted as a hex 1C3, and 73O is

interpreted as an octal 73.  For those of you who prefer the C

language notation, BOOL will also interpret a leading 0x as an

indication of a hex value and a leading 0 as an indication of an

octal value.  Therefore  0x1C3 is hex 1C3 and 073 is octal 73.  A

suffix of D indicates a decimal constant but is usually not needed

since decimal is the default radix.  ASCII values of characters are

also accepted as numeric values. for example: 'B' represents the

ASCII value of the character B.

Page 29

TUTORIAL

1.11 LOOPS, CONTROL FLOW AND INTEGER VARIABLES

In addition to the if-else statements that were already introduced,

The syntax supports program control flow in the form of loops and

function calls (subprograms).

Loops are supported by the statements while and for. In addition, the

statements break and continue which work similar to the break and

continue in the C programming language, give additional control on

loop execution.

Loops are controlled by integer expressions or integer variables.

Unlike boolean class variables that can hold symbolic functions of

input literals, integer variables can hold only constant integer

numbers. As we shall see below there are also other uses for integer

variables, in addition to controlling loops.

The break and continue statements may be controlled by conditions

specified by if or .switch statements (e.g. if (A == B) break;) .

This gives a powerful way to conditionally control what parts of the

loop will be executed (By using continue) or what iterations will be

skipped (by using break). These conditions may be either integer or

boolean (meaning, they may also depend on value of other signals).

1.11.1 USING INTEGERS FOR THE MAJORITY VOTE CIRCUIT DESIGN

To demonstrate the use of integer variables and control flow

statements, lets consider the design of a 7-input majority vote

circuit. The definition of the required behavior is the same as the

3-input majority vote circuit that was introduced before: The two

outputs are RESULT and ERROR. ERROR is 0 only if all seven inputs

have identical values. RESULT is 1 if more than half of the inputs (4

or more) are at logic 1, and 0 if more than half of the inputs are at

logic 0.  The inputs are defined as a group with 7 elements with the

name IN.  Here we chose a different approach to solve the problem.

We count the number of inputs set to 1, and set RESULT to 1 if more

than half of the inputs are set to 1.

Following is the program:

Page 30

TUTORIAL

/* MAJORITY VOTE CIRCUIT WITH 7 INPUTS */

#define insize 7

input IN[insize];

output RESULT,ERROR;

boolean count[3];

integer i;

integer threshold;

threshold = insize / 2;

/* First, count the number of inputs that are set to one */

count = @0;

for ( i = 0,insize-1 )

if ( IN[i] == 1 ) count := count + @1;

/* calculate the outputs according to count */

if (count > @threshold ) RESULT = 1;

else   RESULT = 0;

if (count == @0 | count == @insize) ERROR = 0;

else  ERROR = 1;

Several new things were introduced here.  First, instead of hard

coding the number 7 into the program, we defined the symbol insize to

be 7 with the statement #define insize 7. The effect of this

statement is to cause the word insize to be replaced with the

character 7 everywhere in the program. This is done even before the

compiler starts to parse the source code, in a special pass called a

preprocessor pass.

The advantage of using insize instead of coding directly the number 7

is that later we can change the design to handle, for example, a 5-

input majority vote circuit by just changing the number 7 to 5 in one

place.  We also had to declare a new variable called threshold which

we set in this case to 3 (integer part of 7/2).

The next step is to count the number of inputs that are set to 1.

During the count, the group variable count accumulates the number of

inputs set to 1. This is done in two steps: first, the value of count

is set to 0 with the statement:  count = @0;  Then, the statement:

if ( IN[i] == 1 ) count := count + @1;   is executed 7 times.  The

statement: for ( i = 0,insize-1 )  is a simple iterative loop that

causes the next statement to be executed 7 times.   In the first

iteration i starts with the value 0 and then incremented by 1 each

iteration. In this way, IN[i] selects each time a different input

signal from the group IN.

The statement count = @0; could have also been written as

count = 3@0; (which means a group of 3 zero bits), however, the form

written above is also perfectly legal.  Generally, @n is equivalent

to S@n where S is the word size of the host machine but not less than

32.  On an IBM-PC for example, S is therefore 32.  @0 is equivalent

therefore to 32@0.  This is also true later in the statement:

count := count + @1;  Although only 3 bits are needed, the extra bits

are ignored when the result is written into count. The only effect is

Page 31

TUTORIAL

probably more time spent on computing the unnecessary bits by the

compiler.

1.11.2 MORE INFORMATION ON FOR AND WHILE STATEMENTS

Notice that unlike the for statement in the C programming language,

the for statement in BOOL allows only loops with a predefined number

of iterations.

The for statement has also a different form which allows the loop

index to be incremented or decremented by an arbitrary number, as in

the following example:

for ( n = 2,10,3)

X = X + @n;

Here, the statement X = X + @n; is executed 3 times with the variable

n stepping through the values: 2,5,8.

It is also possible to form a loop that will count downwards as in:

for ( n = 10,2,-3)

X = X + @n;

Here, the index n starts with the value 10 and is decremented by 3

each iteration.  It will therefore step through the values: 10,7,4.

Loops based on any integer condition may also be formed with the

while statement.  The condition is an integer expression specified in

parenthesis after the keyword while.  The loop is repeatedly executed

until the expression becomes zero (false), or until a break is

executed.  For example, the following piece of code, performs a bit

reversal on a 16 bits bus:

integer ii;

output X[16];

input Y[16];

ii = 0;

while (ii <= 15)

{

X[ii] = Y[15-ii];

ii = ii + 1;

}

This code is equivalent to:

integer ii;

output X[16];

input Y[16];

for (ii = 0,15)

X[ii] = Y[15-ii];

Page 32

TUTORIAL

1.11.3 USING INTEGER VARIABLES FOR STATE DEFINITION

Integer variables can serve also for improving readability.  The

states inside a .switch statement can be replaced with names of

integer variables.  Choosing appropriate names for the integer

variables will make the code self documented.

As an example, consider the following state machine:

flip_flop STATE_FF[2];

input LOCK,TRIG;

output LOCK_CONTROL;

.switch (STATE_FF)

{ case 0:

if (LOCK) jump 3;

else jump 2;

LOCK_CONTROL = 1;

case 1:

jump 2;

LOCK_CONTROL = 0;

case 2:

jump 3;

LOCK_CONTROL = 0;

case 3:

if (TRIG) jump 0;

else jump 1;

}

We can rewrite the above example in the following way:

Page 33

TUTORIAL

integer CHECK_LOCK, WAIT1, WAIT2, CHECK_TRIG;

flip_flop STATE_FF[2];

input LOCK,TRIG;

output LOCK_CONTROL;

CHECK_LOCK = 0;

WAIT1 = 1;

WAIT2 = 2;

CHECK_TRIG = 3;

.switch (STATE_FF)

{

case CHECK_LOCK:

if (LOCK) jump CHECK_TRIG;

else jump WAIT2;

LOCK_CONTROL = 1;

case WAIT1:

jump WAIT2;

LOCK_CONTROL = 0;

case WAIT2:

jump CHECK_TRIG;

LOCK_CONTROL = 0;

case CHECK_TRIG:

if (TRIG) jump CHECK_LOCK;

else jump WAIT1;

}

The later form, is not only easier to understand, but also enables to

experiment with state assignments by changing only the assignments to

the integer numbers.

1.11.4 MORE ON INTEGER VARIABLES AND EXPRESSIONS

Integer variables can be used as part of expressions and can be

assigned integer values or expressions.  All the operators that were

introduced earlier for boolean class variables (scalars and groups)

will also accept integer operands.  Moreover, the operators * and /

(multiply and divide) which are not available for groups, can be used

for integers.  You should note however, that some operators behave

differently on groups and integers. Following is a list of

differences:

Integer operations are always done in a fixed precision (which is 32

bits for IBM/PC and SUN workstations).  The number of bits in a group

operation is the size of the operand with the greater size.

Integer variables and constants are considered as signed, while

groups are considered unsigned.  This has an implication on right

shift and compare operations.   A right shift operation of an integer

is sign extended while a right shift of a group is zero extended.

Integers, when compared with any of: <  >  <=  >=  ==  !=  give an

integer result of 1 for true or 0 for false.  The comparison is

carried out using signed arithmetic.

Page 34

TUTORIAL

The inverse operator ! will invert all bits in a group.  However when

operated on integers, it behaves as in the C language: It produces

the number 1 if the operand is 0 and the number 0 for any non zero

operand.  To invert all bits in an integer, the bitwise inverse

operator ~ should be used as in: X = ~10;

Integer numbers or expressions can also be used as the condition in

an if() statement.  In this case, the condition is considered true if

the expression is non zero, and as false if the expression is 0.

Integer arrays can also be declared and used.  For example:

integer IA[10]; /* An integer array with 10 elements */

IA[3] = 12;  /* assign 12 to the fourth element */

One important use of integer variables and expressions is as an index

for accessing specific elements of groups (and integer arrays).   We

have already discussed examples that make use of integer variables to

address a specific element in a group.

At last we should mention that expressions can be formed by mixing

integers and boolean class variables.   The result in that case would

be boolean.   Except for the shift operators, you may not mix

integers and groups, unless you first convert the integers to groups

with the grouping operator.

When mixing an integer with a boolean in an expression, (or when

assigning an integer value to a boolean class variable) only the

values 0 or 1 make sense.  If the integer is non zero, it is treated

as 1.  However, a warning is issued if its value is neither 0 nor 1.

1.11.5 CONTROLLING LOOPS WITH BREAK AND CONTINUE STATEMENTS

The break and continue statements can be used inside for and while

loops in a way similar to the break and continue statements in the C

programming language.  Basically, the break statement, when executed,

will cause the loop to terminate immediately and control to pass to

the statement following the last statement in the loop.  The continue

however, causes only the remaining statements in the current

iteration to be skipped (up to the enclosing curly brace), and a new

iteration to be started.

If the break (or continue) statement is executed under a condition,

(e.g. if (cond) break; ),  the result depends on the condition.  For

example, if the condition is the constant integer value 1, then a

break will cause an unconditional termination of the loop.

A more interesting case, is when the condition is a boolean data

type. For example, suppose that a break is specified under the

condition that an input variable X equals to 1. Specifically,

consider the following piece of code:

Page 35

TUTORIAL

input X;

output Y;

integer i;

Y = 0;

for (i = 0,0)

{

if (X) break;

Y := 1;

}

In this example, the loop is specified with only one iteration, with

i having the value 0.   The compiler will synthesize in that case for

Y a logic equation that will reflect the two possible cases: For X

having the value 1 (TRUE), the synthesized equation will show that

the loop terminated, and the statement Y := 1; was not executed

(which leaves Y with the value 0).

For X having the value 0, the logic equation will show that the

statement Y := 1; was executed.  The logic equation that will be

synthesized for Y will be therefore:  Y = !X.

To demonstrate a practical case where a break or a continue statement

would be used we examine two examples.   The first is a simple

priority encoder, and the second is a circuit that counts the number

of zero bits in an input group.

1.11.6 EXAMPLE: A SIMPLE PRIORITY ENCODER

Our task here is to design a circuit that performs the function of a

simple priority encoder.  The priority encoder has an input group A

with 8 elements, and an output group CNT.  The input A is scanned

from the least to the most significant bit for the first occurrence

of a set (Non zero) bit.  The position of that bit is output in CNT.

The position of bits are counted starting at 1   (The position of the

first bit is 1).  If all bits in A  are zero, a value of zero is

output.

input A[8];

integer ii;

output CNT[4];

for (ii = 1,8)

{

if (A[ii-1] == 1)

{

CNT = @ii;

break;

}

}

if (A == @0) CNT = @0;

The complete program, including statements which simulate the design

can be found in the file ENCODER1.B.

Page 36

TUTORIAL

1.11.7 EXAMPLE: COUNTING BITS IN A GROUP

Here we show a more complex example that makes use of break and

continue statements.

A group of 8 inputs (named in)represents an eight bit binary number.

The number is considered legal if it consists at most one string of

consecutive zeros.  For example 10001111 and 01111111 are legal

numbers, but 10100011 and 01011111 are not. The output from the

circuit is a 4 element output group called count.  If the number is

legal, the number of zeros in the string should be output by count.

If the input number is illegal, then count should have the value 15

instead.

input in[8];

output count[4];

boolean seenzero, seentrailing;

integer i;

seenzero = 0; seentrailing = 0;

count = @0;

for (i = 0,7)

{

if (seentrailing  &  in[i] == 0)

{

count := 4@15;  /* illegal number */

break;

}

if (seenzero  &  in[i] == 1)  seentrailing := 1;

if (in[i] == 1)  continue;

count := count + @1;

seenzero := 1;

}

The design approach above is as follows: We define two intermediate

variables seenzero and seentrailing. The bits in the input number are

scanned.  seenzero becomes 1 (and stays 1) when detecting a zero bit

(Each bit is represented by in[i] ).    seentrailing becomes 1 when

detecting the end of the string of zero bits.  From that point on, if

a new zero bit is detected, it indicates an illegal number.  In that

case count is set to the value 15, and the break statement causes the

loop to terminate.    The continue statement, which is executed under

the condition in[i] == 1 makes sure that the last two statements in

the loop which increment count and set seenzero are executed only if

the bit in[i] is zero.

1.12 CONTROLLING PROGRAM FLOW WITH FUNCTIONS

Functions are a more advanced topic of BOOL. If you are not familiar

with any high level programming language, you might find this topic

more difficult to absorb, and you may leave it therefore, to a later

stage after you have gained some practice in writing BOOL programs.

Page 37

TUTORIAL

Functions are used mainly for two purposes:  The first is simply when

the same piece of code has to be used several times in the same

design, or to preserve pieces of source code in a way that is easier

to maintain and reuse for other designs.   The second is to help in

structuring complex programs or algorithms by partitioning them into

several simpler functions each with a well defined interface and

functionality (termed in software jargon as structured programming).

BOOL supports functions with a syntax similar to the K&R definition

of the C language.  Functions in BOOL can return a value to the

calling program.  The type of that value may only be one of: integer,

boolean, group or string.

Parameters can also be passed to functions.  These may have any of

the possible variable types, or it may also be an expression.   The

expression or variable that is passed to the function is called an

actual parameter, while the name of the parameter as declared inside

the function is called a formal parameter.

A function that accepts parameters must have the types of the formal

parameters declared at the top (header) of the function.   Actual

parameters that are passed to a function must be of the same type as

the formal parameters, or belong to a data type that can be converted

to the appropriate type.  For example, the value of an input variable

can be stored in a boolean variable, but not the other way.

If the type of a variable that is passed as an actual parameter to a

function matches exactly the type of the formal parameter, that

variable is passed by reference.   What this means is that any

assignment done to the formal parameter, will be reflected in the

actual parameter.

If an expression is passed as an actual parameter, or if there is a

type mismatch between the formal and actual parameters, the parameter

is passed by value.  This means that only the value of the actual

parameter is passed to the function, and any assignments or changes

made to the formal parameter will not be reflected in the actual

parameter.  In other words, the function is using a copy of the

actual parameter.

To call and execute a function from within your BOOL source code, all

you need to do is write the name of the function and specify the

parameters in parenthesis.  The name of the function can be used as

part of an expression (in that case the name of the function is

replaced by the value it returns) or it can be used alone as a

statement. In the latter case, the returned value is discarded.

The definition of the function should appear before any executable

statement, along with all other declarations.

The statements in the function are executed until a return statement

is executed, which returns control to the point from where the

function was called.  The return statement also specifies the value

returned by the function.  As in the case of the break and continue,

Page 38

TUTORIAL

the return statement can be executed under a condition (or set of

conditions).

Consider the following example:  We want to write a function that

will replace the description of a multiplexer: The data inputs to the

multiplexer are A and B, with a select input SEL. If SEL is high, A

is selected, otherwise B is selected.  The following function is

defined as having three input parameters all of type boolean, and

returning a boolean value.

boolean MUX(A,B,SEL)

boolean A,B,SEL;

{

if(SEL) return A;

else return B;

}

A multiplexer function that operates on groups, will be written as

follows:

group GMUX(A,B,SEL)

boolean A[],B[],SEL;

{

if(SEL) return A;

else return B;

}

The following piece of code shows an example how the functions above

are called and executed from the main program:

input A,B,GA[4],GB[4],S;

output X,Y[4];

X = MUX(A,B,S);

Y = GMUX(GA,GB,S);

Notice that the number of elements is not specified in the

declaration of the formal parameters A and B in GMUX. Actually it is

illegal to specify the number of elements of a formal parameter.

This means that any size of group may be passed to the function

through the parameters A or B.

It is possible to find out the exact size of a group by using the

built in function size.   For example, the expression:  size(A)

inside the function GMUX will return the value 4, since A has 4

elements.

1.12.1 EXAMPLE: COUNTING BITS, THE GENERAL CASE

Next we examine a larger design example which makes use of a

function.  Recall the problem presented above of counting the zeros

in an 8 bit number, and determining if the number is legal or not.

Page 39

TUTORIAL

We make the problem a bit more complicated by introducing another

factor: The polarity of the input is unknown.  We first have to check

the number in positive polarity. Only if the number is found to be

illegal, we check it in negative polarity.  If the number is found as

illegal in both cases, a value of 15 is output, otherwise the number

of zeros (or ones in the case of inverted polarity) is output.

For example: both 10011111 and 01100000 are legal and will produce

the output 2.  10100000 however is illegal and will produce the

output 15.  As another example 11100000 is a legal number and will

produce the output number 5.  Here, since the number is legal as both

inverted and uninverted, the uninverted form is chosen.

Our approach in this example is to define the general count zeros

algorithm in a function called count_zeros, and then call count_zeros

twice.

group count_zeros(in)

boolean in[];

{

boolean count[4];

boolean seenzero, seentrailing;

integer i;

seenzero = 0; seentrailing = 0;

count = @0;

for (i = 0,7)

{

if (seentrailing  &  in[i] == 0)

return 4@15;   /* illegal number */

if (seenzero  &  in[i] == 1)

seentrailing := 1;

if (in[i] == 1)  continue;

count := count + @1;

seenzero := 1;

}

return count;

}

/* main program starts here */

input in[8];

output count[4];

count = count_zeros(in);

if (count == @15)

count := count_zeros(!in);

/* end of circuit definition */

In this example, the function count_zeros is called first with the

parameter in. The function counts the number of zeros in in and the

returned result is written into count. However, if count_zeros found

more than one string of zeros, the returned value is 15.  In that

case, count_zeros is called again with the inverse of in.

Page 40

TUTORIAL

Notice that the name of the formal parameter in the definition of

count_zeros is in, same as the name of the actual parameter used in

the main program. This is not a requirement. Different names could

also be used.

Notice also that the name count is used both as a local variable in

the function and as a global variable in the main program.  Do not

get confused: These are two different variables.  Inside the

function, count is declared as a boolean group, while in the main

program count is declared as an output group.

If you examine the body of the function count_zeros you will notice

that the code is almost identical to the previous program that

counted only the number of zeros.  However, the section:

{

count := 4@15;  /* illegal number */

break;

}

was replaced with the single statement:

return 4@15;   /* illegal number */

which causes an exit from the function with a return value of 15.

The return statement, since it is located inside a loop will not only

cause the function to exit, but will also terminates the loop (All

this under the specified condition).

1.12.2 LOCAL VARIABLES IN A FUNCTION

In the above example we saw that the function has its own set of

local variables which are distinct from the variables defined in the

main program (and perhaps other functions), even if they use the same

names.   Local variables of functions must be declared at the top of

the function body immediately following the opening brace, and before

any executable statement.  Local variables retain their value only as

long as the function is executing.  Therefore they may not represent

real network elements, meaning they may not be of type input, output,

node or flip-flop. The only types allowed are boolean (scalar or

group), integer (scalar or array) or string.

Each time when a function is called, all local variables in the

function are initialized to a predefined value. (The same value that

global program variables start from). Note that this is different

than many other computer languages.  For example, in the C

programming language, regular local variables of a function start

with an unknown value meaning that they may start with a different

value each time the function is called.  In BOOL there is no such

thing as unknown values, since all variables of all types always

start from a definite known value.  (This is true also for global

variables).

Page 41

TUTORIAL

String variables will start from an initial empty string value.

Integer variables and elements of integer arrays start from an

initial value of 0, and boolean variables and groups start with a

value of global don't care (Or undefined).  Note that in BOOL

terminology, undefined is a synonym for don't care, and is definitely

not an unknown value.

1.13 SIMULATING YOUR DESIGN

In  many cases, you will be interested in simulating your design on

the behavioral level, before you map your design into any technology.

BOOL has a built in, event driven simulator that works on the same

level of abstraction as your design description.  The simulator is

built into the language as a collection of statements that can be

used in conjunction with other BOOL constructs such as flow control.

You can simulate your design before minimization.  The advantages of

simulation before technology mapping or minimization is that your

design still contains information on don't cares: those states or

input conditions that should never really occur in normal operation.

The simulator can indicate whether your input stimuli drives any of

your network variables (outputs, flip-flops or nodes) into a don't

care condition.

Another advantage is that you can save time in the early design

cycles, by verifying that the basic algorithms work.

To demonstrate a simple use of the simulator, lets examine the

previous design of the bit counting problem.  Our goal here is to

scan all the input values of the input group in and get a printout of

all the corresponding values of the output group count.  To scan all

the possible values (which are 256) we use a for loop.  The following

piece of code depicts the main program with the simulator statements.

We omitted here the function count_zeros since it remains the same as

in the previous example:

Page 42

TUTORIAL

/* main program starts here */

input in[8];

output count[4];

integer i;

count = count_zeros(in);

if (count == @15)

count := count_zeros(!in);

/* end of circuit definition */

/* starting simulator statements */

for (i = 0,255)

{

values (in = 8@i);

printf(1,"in=%b    count=%u\n",in,count);

}

The complete design example can be found in the file GCOUNT.B.  You

can run it and watch the results.

1.13.1 USING THE VALUES STATEMENT

In the example above, a loop is executed 256 times. In each iteration

i is assigned a different value from 0 to 255.  The simulator is

using this value as a stimulus for the input group in.  This is done

by the values statement:

values (in = 8@i);

We are already familiar with the expression 8@i which means an 8

element group which represent the value of i.

Several variables can be assigned a stimuli at the same time as in:

values (A = 1, gg1[2] = 0, gg2 = 4@3);

After defining the stimuli to the circuit, the simulated values of

the output port is printed to the output with the printf function.

1.13.2 USING THE PRINTF FUNCTION

Simulation results are printed here by the function printf.   This

function is similar to the fprintf function in the C programming

language.  The function accepts a variable number of parameters: The

first parameter is always an integer number called file number, which

indicates where the output should be written.   The number 1

indicates that the output will be directed to the screen, unless an

output file name is specified in the command line (after the input

file name).

The second parameter to printf is an ASCII string called format

specifier.  Following the format specifier are all the values that

Page 43

TUTORIAL

should be printed.  The format specifier instructs the compiler how

to format the output.  The string is scanned from left to right and

each character is printed to the output.  When the character % is

found in the string, a special action is taken:   The next parameter

is taken from the list and printed to the output according to the

character following the %.  In the example above we used %b to

indicate that a binary conversion should be used for the parameter

in, and %u to indicate that an unsigned integer conversion should be

used for the parameter count.  %b (and %u) are called conversion

specifiers.

There are other conversion specifiers for printing octal (%o), hex

(%h or %x), signed decimal (%d), strings (%s) and characters (%c) are

also available.

The character pair \n (In this example, at the end of the string) is

converted to an end of line character.  The character \ is used as an

escape code.  For example, \t  \"  \\  can each be used to indicate a

tab, a double quote and a backslash respectively.

The printf function will accept as parameters string and integer

variables as well as boolean class and group variables (any of:

input, output node, flip-flop, boolean).  The simulated value of

boolean class variables is first calculated, and then formatted

according to the format specifier.

1.13.3 STRING VARIABLES

So far we mentioned several times string variables but we never

explained what they really are.  String variables, can hold strings

of ASCII characters and may be used whenever a constant string is

required (As in the printf() function).

String variables, like integer variables, contain data that is not a

function of input literals (inputs, nodes or flip-flops).

Strings are declared with the string keyword and may be used as in

the following example:

string st1,st2;

st1 = "st2 IS %s";

st2 = "qqqq";

printf(1,st1,st2);

The printout from this example would be: st2 IS qqqq

Page 44

TUTORIAL

1.13.4 MORE ON SIMULATION - NODES, FLIP-FLOPS AND DON'T CARES

The example above showed simulation of a simple case of a

combinational circuit.  The built in simulator can handle also nodes

and flip-flops by resolving signal propagation using an event driven

mechanism.  To resolve signal arrival time, a unit delay is

associated with each node and flip-flop.  However, the internal

simulation time that the simulator is using is not available to the

user, and after each specification of a stimuli, the simulator will

propagate the internal simulation time until the circuit stabilizes

(or in other words, until the event queue is empty).  Feedback loops

made of nodes which cannot stabilize will cause therefore an error

message by the simulator.

The simulator can be stimulated in one of two ways. A stimuli can be

presented with the values statement. Or flip-flops can be triggered

with the clockff function.

Consider the following example:  A simple 4 bit decimal counter that

counts in an infinite loop from 0 to 9.  We want to simulate the

design as follows:  First we want to initialize the counter to the

value 5 and then simulate 7 clock ticks.

flip_flop cnt[4];

integer n;

/*** specifying the circuit ***/

if (cnt < @9) cnt:d = cnt + @1;

if (cnt == @9) cnt:d = @0;

/*** simulating ***/

values (cnt = @5);  /* load initial value of 5 */

values ();          /* enable changes to cnt */

for (n = 1,7)       /* iterate 7 times */

{

printf(1,"cnt = %u\n",cnt);

clockff(cnt);

}

clockff can accept more than one argument.  Each argument must be

either a flip-flop or a group of flip-flops. All flip-flops specified

in a single clockff call are triggered at the same time.  If clockff

is called with no arguments, as in: clockff(); all flip-flops in the

circuit are triggered.

The argument to the values statement above is a group of flip-flops.

This demonstrates that values can be used not only for input

variables but for any variable that can serve as an input literal.

This includes inputs, nodes and flip-flops.  When applied to nodes or

flip-flops, values that are set with values statement are kept frozen

until the next values statement.  In other words, until the next

values statement, the specified values will remain unchanged

regardless of any events that otherwise would have caused these nodes

or flip-flops to change their simulation value.  This is the reason

Page 45

TUTORIAL

why we need the second values statement without any arguments: to

unlock the freeze.

We mentioned earlier that simulating designs before minimization can

sometimes reveal design bugs if the simulator hits a don't care

condition.  To enable detection of don't cares during simulation, you

have to instruct the simulator to do so by executing the statement:

setparam check_sim_dc yes;

Actually this feature can also be turned off by using the keyword no

instead of yes.

To demonstrate its use, suppose that in the previous example of the

free running counter we forgot the statement:

if (cnt == @9) cnt:d = @0;

This would cause the simulator to hit a don't care condition when it

reaches a count of 9 (Nothing is specified for values of 9 and

above).  Not only will the simulator issue a warning message, but the

printf statement will also print the character * to indicate a don't

care condition. (The character X will be printed for single bits)

COMMENT: Not always does a don't care during simulation indicate a

real bug.  Sometimes a don't care condition can really occur to a

signal in a circuit, if other conditions cause the value of this

signal to be ignored by the rest of the circuit.  For example, the

input to a flip-flop may be a don't care as long as no clock is

applied.

1.14 SELECTING THE PROPER MINIMIZATION ALGORITHM

We already learned that the built in minimizer can be called through

the minimize() function.   However, if you do not give any specific

instructions, only a very simple minimization algorithm is invoked.

The minimizer in BOOL can actually be controlled by several

parameters depending on your demands.  The parameters that can be

controlled are signal polarity, minimization quality (which

indirectly affects computation time and memory requirements) and

automatic generation of redundant product terms for hazard free

logic.

1.14.1 MINIMIZATION POLARITY AND MINIMIZATION LEVEL

The polarity of the minimized signal can be selected with a parameter

called minimization polarity. We can chose between three

possibilities:  minimization with positive polarity, minimization

with negative polarity, and minimization with programmable polarity.

Page 46

TUTORIAL

The last option means that both negative and positive polarities are

tried out, and the polarity that yields a better solution (less

product terms) is chosen.

The minimizer can be instructed to 'try harder'. This usually comes

on the expense of consuming more computing time and memory.  To

control this parameter we specify a 'minimization level' number

between 1 and 63.  The number 1 which is the default when calling

minimize(), invokes a very simple minimization algorithm which only

guarantees a non redundant solution.  The number 63 guarantees a

solution with a minimal number of product terms (absolute minimum),

and if several of those exist, the chosen solution is the one with

the minimal number of input literals.

If a smaller minimization level is specified, the minimizer may or

may not arrive to the absolute minimum.  In many cases, a

minimization level of just 3 will produce the absolute minimum.

Specifying a higher minimization level will produce the same result

and consume the same amount of computing time and memory.

There are several ways to specify the first two parameters

(minimization polarity, and minimization level).  The simplest way is

to use the function pminimize as in the following example:

pminimize(10,0,X1,YY2);

This will minimize X1 and YY2 with positive polarity and with a

minimization level of 10.  Instead of the second parameter 0 you can

also use 1 to indicate minimization with negative polarity, or 2 to

indicate programmable polarity.

The default setting of the minimization polarity and minimization

level that will be used in the minimize() function can also be

controlled with the setparam statement as in:

setparam default_minlevel 10;

setparam default_polarity "pos";

The other possibilities for the default_polarity parameter (except

for "pos") are "neg" for negative polarity, and "prog" for

programmable polarity.

1.14.2 HAZARD FREE LOGIC

The minimizer can be instructed to produce hazard free logic by

executing a call to the built in function hazard_free() prior to

calling the minimizer.  The minimizer minimizes the logic but insures

that the result has the necessary set of redundant product terms to

make it hazard free.

Generating hazard free logic is an important issue when designing

asynchronous circuits.  Even when designing logic that is basically

synchronous, signals driving such functions as a clock or an

Page 47

TUTORIAL

asynchronous reset to flip-flops, are usually required to be hazard

free, otherwise, glitches which occur due to differences in path

delay may cause unintentional triggering of the function.

How are glitches generated?

To understand how glitches may occur in a non hazard-free design

examine the following example:

A small combinational circuit has three inputs: A,B,C and one output

X, with the following boolean relation:

Equation 1:  X = !A & B | A & C;

Suppose that this circuit is implemented, for example, with an

inverter, two AND gates and one OR gate.  If both B and C inputs have

value 1 (TRUE), then when A changes state from 0 to 1 (Or from 1 to

0), the output X should remain 1 (TRUE).  However, a negative glitch

may occur during the transition since one of the AND gates might be

slower than the other.

To correct this problem, we can add another product term (AND gate)

which will cover the transition as follows:

Equation 2:  X = !A & B | A & C | B & C ;

The addition of this AND gate ensures that changes in A cannot cause

glitches, since it forces the output to a logic 1 when both B and C

are at logic 1, regardless of the state of the other two AND gates.

In more complex cases, we may need to add several product terms to

avoid glitches that might be triggered by changes in more than one

input signal (It is assumed, however, that no more than one input

signal can change at a specific time).

We may choose to ignore changes in some inputs because we know, for

example, that these inputs are static configuration lines that never

change state during normal operation.

Consider the following example:

Equation 3:  Y = !A & B | A & C | !B & !C ;

Here, to make the equation completely hazard free, we need to add 3

product terms to avoid glitches when each of the inputs changes

state.

The resulting equation looks as follows:

Equation 4:  Y = !A & B | A & C | !B & !C | B & C | A & !B | !A & !C

;

However, if we choose to ignore changes in B, assuming that its state

will remain constant during normal operation, we get:

Page 48

TUTORIAL

Equation 5:  Y = !A & B | A & C | !B & !C | B & C | A & !B  ;

1.14.3 HAZARD FREE LOGIC TERMINOLOGY:

Referring to the examples above, Equation 1 is not hazard free, while

Equation 2 is hazard free.

The additional product term in Equation 2:  B & C  is called a

redundant product term, since that product term does not change the

boolean value of X.

Equation 4 is hazard free with respect to all inputs, while Equation

5 is hazard free only with respect to inputs A and C.

1.14.4 HAZARD FREE MINIMIZATION IN BOOL

Hazard free minimization is controlled by calling the built in

function hazard_free() prior to calling the minimizer.  Its effect

takes place until the next call to hazard_free().

hazard_free(1) will turn hazard free minimization on. Any call to any

of the minimizer functions: minimize, lminimize or pminimize,

thereafter will produce minimized hazard free equations (with respect

to all inputs).

To turn the hazard free option off, execute the call:

hazard_free(0).

To generate minimized logic that is hazard free only with respect to

a set of inputs, you can add a list of inputs to the parameter list

of hazard_free().  The interpretation of the list depends on the

value of the first parameter as in the following example:

The call:  hazard_free(1,U,V);   will cause minimization results to

be hazard free with respect to all inputs except U and V.

The call:  hazard_free(0,R,T);   will cause minimization results to

be hazard free only with respect to R and T.

U, V, R, T in the above examples are names of input literals and can

also be names of groups of input literals.  They may be of type

input, node or flip-flop.

To summarize, we present a short segment of code that demonstrates

the use of hazard_free().

Page 49

TUTORIAL

input A[2],B,C;

flip_flop X[4];

output Y1,Y2,Y3,Y4,Y5,Y6;

/*

... circuit definition comes here ...

*/

minimize(Y1);    /* regular (non hazard free) minimization */

hazard_free(1);

minimize(Y2);    /* hazard free with respect to all inputs */

pminimize(5,1,Y3); /* hazard free with respect to all inputs */

hazard_free(0,A,X[0]);

minimize(Y4);    /* hazard free only with respect to A[0], A[1], X[0]

*/

hazard_free(1,A,X);

minimize(Y5);    /* hazard free only with respect to B, C */

hazard_free(0);

minimize(Y6);    /* regular (non hazard free) minimization */

WARNING:   The minimizer, when generating hazard free logic, does not

check for dependencies across node variables.  Each node variable

that appears as an input literal is treated only as a pure input

literal.  The following example demonstrates the problem:

input A,B,C;

node  N;

bool X;

N = !A & B;

X = N | A & C ;

Functionally, this example is equivalent to Equation 1 (on page 48).

However, when X is minimized with a hazard free minimization

algorithm, no redundant product terms are added because the minimizer

treats N as an input literal and does not detect the dependency

between N and A.

1.15 A FEW WORDS ON THE PREPROCESSOR FACILITY

For those who are familiar with the C preprocessor, we mention that

BOOL supports a subset of the C preprocessor.  The directives

supported by BOOL are : #define  #undef  #ifdef  #else  #endif  and

#include.

These work the same as in the C programming language except the

following:

Continuation lines and comments are not supported.

#define does not accept parameters, and does not allow comments.

The other directives will ignore any characters that appear at the

end of the line.  (This allows you to put a comment provided that it

does not overflow to the next line)

Page 50

TUTORIAL

A complete definition of these directives is included in the

reference manual.

1.16 MORE ON PRINTING RESULTS AND FORMATTING DEFAULTS

In addition to the print and printf statements which were already

introduced, there are also other built in functions which enable you

to format, print, open and close files, or control the output in

other ways.  We mention here one of the most important ones: the

function writestd.  This function gives access to several generic

output formats, and its main use is as a building block for

generating special output formats.

The second parameter of writestd is the format number which selects

the desired format.  (The first parameter, as in printf, is the file

number).  For example, Berkeley PLA format is specified with format

number 2,  therefore, to print the contents of variables A,B,C using

Berkeley PLA format, to file number 1, add the following line:

writestd(1,2,A,B,C);

There is also a way to control some formatting parameters in the

print or writestd functions using the setparam statement.  setparam,

as we have already seen, is used to change run time defaults.

Formatting defaults can also be changed with the setparam statement.

For example, if you want to change the AND operator in the output

equations from & to * (As required by PALASM) you can do it by:

setparam and_operator " * ";

This statement has an effect which lasts until the parameter

and_operator is redefined by another setparam statement.  The above

statement will cause an AND operator to be represented by a * (with a

space before and after it) in all boolean equations that are printed

by print or writestd.

Following is a partial list of parameter names that may be used to

change defaults in the output.  For a complete list consult the

definition of setparam in the reference manual.

not_operator  -  define string to replace the ! mark.

or_operator  -  define string to replace the | mark.

and_operator  -  define string to replace the & mark.

assign_operator  -  define string to replace the = mark.

endeqn_mark  -  define string to be placed at end of equation instead

of the ; mark.

Page 51

TUTORIAL

Notice that the string you specify has to be enclosed in quotes.

Inside the quotes you may specify any characters including end of

line ("\n"). For example, you may specify that the end of equation

mark will include only an end of line by:

setparam endeqn_mark  "\n";

The maximal number of characters that are allowed on a single line

can also be controlled by the parameter chars_per_eqn_line as in:

setparam chars_per_eqn_line 70;

1.17 INTRODUCTION TO THE STANDARD FUNCTIONS LIBRARY

The BOOL compiler is supplied with a set of functions which actually

consists of two types.

Functions of the first type are called built in functions, and are

actually part of the compiler itself.  These functions supply

services that are either hard or impossible to achieve through

standard BOOL statements.  The functions printf and pminimize are two

examples.  Some of the built in functions, accept a variable number

and types of parameters (which is not possible with user defined

functions).

The second type are regular BOOL functions, supplied as BOOL source

code.  These are named external library functions, since they are not

part of the compiler.   These include common tasks or algorithms

written in BOOL  source code.  To use such function in your program,

you have to instruct the compiler to read the file where the function

is.  This is done through the use of the #include directive.

Consider the following example:  The user wants to design a 4X4 bits

signed multiplier.  A function called smult already exists in the

external library in a file called ARITH.INC.  Instead of re-writing

the algorithm for generating the equations of a signed multiplier,

the user can simply write the following code:

#include "ARITH.INC"

output  X[4];

input  A[4],B[4];

X = smult(A,B);

Page 52

TUTORIAL

1.18 PROGRAMMING TIPS

1.18.1 INTRODUCTION

If BOOL can process your design file in a reasonable amount of time,

and does not complain on memory shortage, you can probably skip this

chapter.  However, if you do experience any of the above problems,

you might find this chapter helpful.

Unlike regular programming languages, the execution time is not

linearly related to the number of source code lines executed by the

compiler.  Execution time and memory consumption of even relatively

small programs can easily overflow your computer's capabilities.

This is not a specific limitation of BOOL, but rather a well known

general limitation of synthesis algorithms.  Since statements are

executed as written, it is important in large designs that you

structure your code in a way that would not overload the computer.

Another observation is that the processing time of a boolean class

expression is directly related to the complexity of the values

involved, where the complexity is related to many factors from which

the most important is usually the number of product terms.  Compare

this to a regular programming language, where the computation time of

an expression is usually unrelated to the values involved.

1.18.2 EXPRESSION FACTORING

Some general programming practices are true also for BOOL. For

example if you have an expression that is used repeatedly in a loop,

or several times during the program, calculate it only once, store it

in a temporary variable and use this variable instead.  If the

expression yields an integer value, this is not as important, since

processing of integer values is much faster.

For example, consider the following code:

input A[5],B[5],U;

output X[5],Y[5];

if (U)  X =  A + B;

else Y =  A + B;

Here the expression A + B is calculated twice.  To make this code run

faster, a better way to write it would be:

input A[5],B[5],U;

output X[5],Y[5];

boolean A_PLUS_B[5];

A_PLUS_B = A + B;

if (U)  X =  A_PLUS_B;

else Y =  A_PLUS_B;

Page 53

TUTORIAL

Notice that unlike regular programming languages that will spend time

on executing either the if part or the else part but not both, here,

computation time is spent on both since the condition is an input

variable which may have both true and false values.

1.18.3 BEWARE OF PATHOLOGICAL EXPRESSIONS

Some expressions, when expanded into two level logic, create huge

boolean functions with lots of product terms.  One famous example is

the identity comparator.  For example, if A and B are both input

groups of 10 elements each, the expression X = A == B creates no less

than one thousand product terms.  If possible (meaning: supported by

the target technology), these kind of expressions should not be

flattened at all, but specified structurally using nodes.   For

example, our 10 bit identity comparator, can be specified as follows:

input A[10],B[10];

node NN[10];

output X;

NN = !(A ^ B);

X = 1;

for (i = 0,9) X := X & NN[i];

This form does not burden the compiler, since X is now described with

only one product term, and the total number of product terms in all

elements of NN is 20.

This method of course, cannot be used if global minimization is

needed when X itself is used as an intermediate value.

1.18.4 PARTITION YOUR DESIGN

Remember: Processing time (and memory consumption) is related not

only to the number of statements executed, but also to the total

number of network elements that are declared in the file.  This means

for example, that if you add to your design file declarations of

elements such as flip-flops, nodes or inputs, even if they are not

used,  they might cause the program to run slower.

The advice is therefore: If your design is made up of pieces that are

loosely coupled, separate them into different design files, and run

BOOL independently on each file.  In this way, each file will have

less network elements to be processed.

Page 54

CHAPTER 2: LANGUAGE REFERENCE MANUAL

2.1 PROGRAM FORMAT AND STRUCTURE

The order of statements required by the BOOL compiler is as follows:

(1)  declaration of global variables, functions and attributes (in

any order).

(2)  executable statements of main program.

(3)  END;

Although not required, we recommend that you arrange your code in the

following order:

(1)  definition of attributes

(2)  declaration of functions

(3)  declaration of global variables

(4)  executable statements dealing with circuit definition

(5)  executable  statements dealing with circuit simulation

(6)  minimization of variables

(7)  printing minimized results

(8)  END;

Only parts (3), (4) must be present. The others are optional and

depend on your needs.

The syntax is free format, and may span on several source lines.

White space characters (space, tabs and end of line) are ignored

except when used as a delimiter or inside strings.

Comments are enclosed in /* ... */  and may be nested.  For example:

/* comment /* nested comment */ */ .

2.2 BASIC ELEMENTS - NAMES, VARIABLES AND OPERATORS

2.2.1 SYMBOLIC NAMES

Symbolic names, or symbols for short, are used for the following

purposes:

names of variables

names of functions

names of preprocessor macros

Page 55

REFERENCE MANUAL

Symbols may contain a combination of alphanumeric characters, period

and underscore, but may not start with a digit.

Symbols are case sensitive.

Symbols may be up to 32 characters long.

Examples of legal symbols:  Jack  n12345_0  FR.n.x  .uu  _GGXu1

2.2.2 NUMERIC CONSTANTS

Numeric constants represent signed integer numbers in a range that is

dependent on the host's word size, (but no less than 32 bits).

A numeric constant may be written as binary, octal, decimal,

hexadecimal or as an ASCII value.  The default is decimal.  The radix

is usually determined by a suffix letter as specified below.

Binary numbers are composed of the digits 0 and 1 and end with the

letter b or B.

Octal numbers are composed of the digits 0-7 and end with the letter

o or O.  The C language notation for octal numbers is also supported,

which means that a number starting with a leading zero is considered

octal if it does not end with any of the other suffixes.

Decimal numbers are composed of the digits 0-9.  The letter d or D

may optionally be added to the right.  The letter d (or D) is

required as a suffix to a decimal number if the number starts with a

leading zero, otherwise it would be interpreted as an octal number.

Hexadecimal numbers are composed of the digits 0-9 and the characters

a-f or A-F and end with one of the letters h,H,x,X.  The leftmost

character in a hexadecimal number must be one of the digits 0-9.  The

C language notation is also supported, which means that a number

starting with 0x (zero followed by lower case x) is also interpreted

as a hexadecimal number.

An ASCII value is a single character enclosed in quotes.  The value

represented by the string is the ASCII value of the character.   The

backslash character is used in escape sequences as follows:

'\n'    End Of Line

'\t'    Horizontal tab

'\\'    Backslash character

'\''    Quote character

'\c'    Any other character (Interpreted as 'c')

Examples of legal numeric constants:

binary:  10110b 001110B

octal:  53o 176O 0546

Page 56

REFERENCE MANUAL

decimal:  19865  765400d  12D  045D

hexadecimal:  176h  27AFh  0afDEH  0x709A 23a7X

character:  'G'  '"'  't'  '\t'

2.2.3 DATA TYPES

The compiler distinguishes between the following types of data:

boolean data, integer data type, and string data type.

Boolean data is a symbolic boolean function of input literals

(inputs, nodes or flip-flops).  This is the kind of data that is held

by boolean class variables.

Boolean data appears in scalar or in group form.  The scalar form

represents a single bit, while the group form is a vector, and

represents usually an integer value.    A group is also identified by

its size which is the number of bits (or elements) in the group.

The size of one group may be different than the size of another

group.

Unlike an integer data type (discussed below) the range of integer

numbers that may be represented by a boolean group, is not limited by

the host word size, but only by the size of the group.

Integer and string data represent values that do not depend on any

circuit element.

Integer data may be held in integer variables, or specified as a

numeric constant.   The range of integer data is limited by the host

word size (but no less than 32 bits).

String data is held by string variables.  Strings may also be

specified as constants by enclosing a string in double quotes  (e.g.

"This is a string").   The backslash is used as an escape character

(For details, see section on numeric constants)

2.2.4 GLOBAL VALUES

The values 0 and 1 are integer values.  However, they may be assigned

to scalar boolean class variables, or even be used directly in

expressions instead of a scalar boolean variable.  In that case they

are converted to their boolean equivalent which are named here:

global zero and global one  (synonyms that are used in other texts

are: Logical High and Low, True and False, etc.).

More formally, a global zero, for example, is a boolean function that

has the value 0 for any combination of values of input literals.

Another value is the global don't care.   Unlike the global zero and

global one, this value is not represented by any special symbol.   A

Page 57

REFERENCE MANUAL

global don't care is a boolean function that is don't care for any

combination of input values.   If there is an explicit need for a

symbol that represents a global don't care value, any uninitialized

boolean variable can be used.  Uninitialized means that no value was

assigned to it in the program.

Comment: Global values are also referred to sometimes by the term:

constant values.  Although not accurate, this term is sometimes used

since it indicates a value that is constant with respect to the

input.

2.2.5 VARIABLES

A limited set of types is supported.    The basic types are:

input, output, node, flip_flop, boolean, integer, string.

In addition, all types except string have also a vector form which is

called an array (For integer) or a group (For the other types).

The types are categorized as follows:

input, output, node, flip_flop, boolean are called boolean class

variables since they hold boolean data type (symbolic boolean

functions of input literals).

Each of the basic scalar types is also available in a group form

which is an ordered set (vector) of elements.  The groups are

referred to by the names: input group, output group etc.

Groups may be accessed as a single entity, and are treated in that

case as representing an unsigned integer number (with each element

representing a single bit).   Single elements in a group may also be

accessed by indexing the group as in X[5].

The size of a group is the number of elements in the group.  This is

usually fixed.  An exception is the variable size boolean group

discussed below.

The size of a variable size boolean group is not fixed and may vary

during execution.  The initial number of elements in a variable size

boolean group is 1.   However, by assigning to it (With an assignment

statement) any group value, the size of the group is expanded if

needed to accommodate the size of the assigned value.  Also,

assigning a scalar value to an element which does not exist, will

also expand the size of the group to accommodate that element.

Input, output, node and flip-flop (In scalar or group form) represent

signals (or elements in a boolean network).  These  are referred to

as network elements.  Boolean variables on the other hand, are used

to hold temporary values, and will usually not be synthesized into

logic.

Page 58

REFERENCE MANUAL

Integer variables hold integer values.  The range of values that may

be stored in an integer number depends on the host's word size but is

no less than 32 bits for any host.

Integer arrays are also supported.  Single elements in an integer

array are accessed by indexing the array as in A[3].   We define the

size of an integer array as the number of elements in the array.

String variables hold ASCII strings and may be used wherever an ASCII

string is required.

Notice that integer (Scalar or arrays) and string variables hold

values that are not dependent on the values of input literals.   In

other words, they hold constant values.  This values are constant

only with respect to the input literals, but may change during

program execution.  For example, an integer value that is used as a

loop index, will be incremented for each iteration of the loop.

Variables that are declared in the main program are called global

variables and are accessible by all parts of the main program and

also by any function.

Variables that are declared inside a function are called local

variables and are accessible only by that function.  Network elements

may only be declared as global variables.

All global variables are initialized to a predefined value (termed

virgin state) before the compiler starts to execute the program.

The virgin state of integer variables and all elements of integer

arrays is 0. The virgin state of string variables is an empty string,

and the virgin state of all boolean class variables is global don't

care.

Local variables of functions are initialized to their virgin state

every time the function is called.

2.2.6 PREDEFINED VARIABLES - UNIVERSAL GROUPS

There are several predefined group variables through which all

network elements may be accessed.   The names of the groups are:

univ.inpgroup  :  Includes all input variables from all declarations.

univ.ndgroup   :  Includes all node variables from all declarations.

univ.outgroup  :  Includes all output variables from all

declarations.

univ.ffgroup   :  Includes all flip-flop variables from all

declarations.

univ.igroup    :  Includes all input literals. (All inputs, nodes and

flip-flops)

Page 59

REFERENCE MANUAL

2.2.7 OPERATORS

We distinguish between unary operators and binary operators.

Unary operators are operators that operate on a single operand

(e.g. -A).

Binary operators operate on two operands and produce a single result

(e.g. A + B).

The following table is a summary of all available operators.

Operator Explanation

-------------------------------------------------------

- Y   Unary minus

~ Y   Unary inverse Type 1

! Y   Unary inverse Type 2

@ Y   Unary grouping operator: Convert Y to group

X @ Y   Convert Y to group with X elements

X * Y   multiply X by Y

X / Y   divide X by Y

X + Y   add Y to X

X - Y   subtract Y from X

X >> Y  shift X right by Y bits

X << Y  shift X left by Y bits

X > Y   compare: X greater than Y

X < Y   compare: X less than Y

X >= Y  compare: X greater equal than Y

X <= Y  compare: X less equal than Y

X == Y  compare: X equal to Y

X != Y  compare: X not equal to Y

X & Y   Logical AND operator

X ^ Y   Logical XOR operator (Exclusive Or)

X | Y   Logical OR operator

The rules for integer data and boolean data are different in some

cases, therefore, we describe each separately.  We start with the

definitions for integer data types, which is the simpler case, and

then examine the differences to the other cases.

2.2.7.1 OPERATIONS ON INTEGER DATA

Generally, for integer data, all operators have the same effect as in

the C programming language (Except for the @ operator which is not

defined in C).  Following is a brief summary:

The operators - + / *  (Minus, Plus, Divide, Multiply)  Have the

usual arithmetic meaning, using two's complement signed arithmetic.

The Unary inverse ~ is the regular logical inverse operator.  It

inverts all bits in an integer (One's complement inversion).

The unary inverse ! produces a zero result for any non zero operand,

and the value 1 if the operand is 0.

Page 60

REFERENCE MANUAL

The unary grouping operator ( @ Y ) generates a group from an integer

value.  Each element of the generated group corresponds to one bit in

the integer.  The size of the group equals to the number of bits in

an integer (Which is 32 bits for IBM/PC and SUN workstations).

The binary grouping operator ( X @ Y ) is similar to the unary

operator.   However, the number of elements in the group is limited

to X, by ignoring most significant bits in Y.

The shift right operator ( X >> Y ) shifts the bits in X right by Y

positions (bits) with sign extension (Most significant bit copied to

vacant bits).

The shift left operator ( X << Y ) shifts the bits in X left by Y

positions with vacant bits filled with 0.

All the comparison operators are done with signed, two's complement

arithmetic, and yield the value 1 if the tested condition is true,

and a value of 0 if the condition is false.

The three logical operators: & ^ | (AND, XOR, OR) have the usual

meaning from boolean algebra with operations done on corresponding

bits.

2.2.7.2 OPERATIONS ON SCALAR BOOLEAN DATA

The only operators that may be used on scalar boolean data are the

three logical operators & ^ | (AND, XOR, OR) and the two comparison

operators: == and != (equal, not equal).    The results have the

usual definitions from boolean algebra.   The operation X == Y is

equivalent to !(X ^ Y) and the operation (X != Y) is equivalent to

X ^ Y.

2.2.7.3 OPERATIONS BETWEEN SCALAR BOOLEAN DATA AND INTEGER DATA

An integer data and a scalar boolean may appear together in any of

the operations: & ^ |  ==  !=.   The integer value will be converted

in that case to either global zero (If it is zero) or a global one if

it is non zero.  If however, the integer is non zero, and does not

have the value 1, a warning message will be issued.   The type of the

result will be  boolean.

2.2.7.4 OPERATIONS BETWEEN SCALAR BOOLEAN DATA AND A GROUP.

The only operators that may be used between a scalar boolean data and

a group are the logical operators & ^ |.

The result is also a group with the same number of elements as the

group operand.  The result is formed by performing the operation

between the scalar and each of the elements in the group operand.

Page 61

REFERENCE MANUAL

2.2.7.5 OPERATIONS ON GROUPS

Except for cases which are explained below, all the operators that

are used between two integers may also be used between two groups

with a similar meaning (where each element in the group represents

one bit).   Notice however, the following differences:

Where applicable, operation on groups are carried out with unsigned

integer arithmetic.  This applies to the arithmetic operations + and

-, the comparison operators, and the shift right operator.  (During

shift right, vacant bits are filled with global zero).

The number of bits used in any operation equals to the size of the

operand with the larger size.  If the two operands do not have the

same size (Number of elements), the operand with the smaller size is

extended with global zero elements (appended as most significant

elements).   If the result of the operation is also a group, the size

of the result will also equal to the size of the operand with the

larger size.  Any carry during arithmetic operations is lost.

The two unary inverse operators  ! and ~ when operated on groups are

equivalent.  Both will invert each element individually.

The operators * / @ do not accept groups as operands.

The shift operators >>  and <<  require that the right operand (shift

count) is an integer data.  Only the left operand may be a group.

All comparison operators are carried out with unsigned integer

arithmetic and yield a scalar boolean result which is 1 if the

condition is true and 0 if the condition is false.

The logical operators & ^ |  are performed bitwise, and produce a

group.

2.3 EXPRESSIONS

Expressions are formed by combining constants, variables and function

calls together with operators.   Variables may be of any type (as

long as the operations are legal) and may include: scalar variables,

group variables and single elements of groups or integer arrays (By

indexing with an integer expression).  Expressions are evaluated from

left to right and operations are carried out according to precedence

of operators.

The order in which operations are carried out can be modified by

using parenthesis.

Expressions may yield any of the four basic data types: integer,

boolean, group or string, depending on the data involved.

Page 62

REFERENCE MANUAL

Notice that since there are no operators that operate on strings, a

string expression may only be a string constant, a string variable or

a call to a function that returns a string value.

2.3.1 PRECEDENCE OF OPERATORS

The list below is arranged in decreasing precedence order, where

operators with same precedence are in the same line:

All unary operators ~  !  @  -  have highest precedence.

Binary operators come next in the following order:

HIGHEST PRECEDENCE

@

*  /

+  -

>>  <<

<  >  <=  >=

==  !=

&

^

|

LOWEST PRECEDENCE

2.4 PREPROCESSOR DIRECTIVES

The preprocessor supports the directives: #define, #undef, #include,

#ifdef #ifndef, #else, #endif.

The preprocessor passes on the source input text before the main pass

(The compiler pass), and modifies the input text according to

directives that are included in the file.  The directives start on

the first column with the character #.  Each directive occupies one

source line.

Preprocessor lines may not include comments, however, except for the

#define directive, extra characters at the end of the line are

ignored.  The end of the line therefore may be used for comments.

Following is an explanation on each of the directives:

#define name subst-string

substitutes every occurrence of name in the source file with the

string subst-string.    For example: #define xxx yyy  will replace

all occurrences of the string xxx with the string yyy.  An attempt to

redefine a name will be honored but will produce a warning.  Unlike

the C preprocessor, nesting is not performed, which means that subst-

string itself will not be scanned for other substitutions.

Page 63

REFERENCE MANUAL

#undef name

undefines a name.   This cancels the effect of a previous #define.

#include "filename"

The line is substituted with the contents of the file specified by

filename.  The file is searched in several locations, which depend on

the operating system you are using.   For details, consult the Bool

User's Guide.

#ifdef, #ifndef, #else and #endif are used together to enable

conditional compilation of selected sections of the program depending

on whether a preprocessor symbol is defined or not.

If the preprocessor symbol name is not defined (by a previous

#define), all the text following a line that contains:

#ifdef name

will be skipped up to a line that contains:

endif

An #else directive can be inserted between the #ifdef and the #endif.

In that case, the text between the #ifdef and the #else will be

compiled if name is defined and skipped if name is not defined.  The

text between the #else and the #endif directives is compiled if name

is not defined and skipped if name is defined.

The #ifndef is similar to #ifdef, except that the inverse condition

is checked.  (Which means for example that the text between an

#ifndef name and an #else directive is compiled only if name is not

defined.)

#ifdef, #endif directives may be nested.  However, every #ifdef or

#ifndef must be matched by an #endif.

Notice that if text is skipped, it is not parsed by the compiler.

Therefore it is not even checked for syntax errors.   Also, other

preprocessor directives (such as #include, #define or nested #ifdef)

will not be executed in the skipped part.

2.5 STATEMENTS

2.5.1 DECLARATION OF GLOBAL VARIABLES

Variables are declared in a declaration statements.  Each declaration

statement may declare one or more variables.  All variables in a

single declaration statement will have the same attributes, and the

same general type.   The general syntax for a declaration statement

is:

type  attribute-list  list-of-variables;

Page 64

REFERENCE MANUAL

type  is any of: integer, string, boolean, input, output, flip_flop,

node.  int is accepted as a synonym for integer, bool is accepted as

a synonym for boolean, inp is accepted as a synonym for input, out is

accepted as a synonym for output, and ff is accepted as a synonym for

flip_flop.

attribute-list is a list of attributes enclosed in parenthesis.

Attributes are used primarily by the interface functions to do

automatic generation of special output formats. More information on

attributes, how to declare attributes, and the syntax of attribute-

list can be found in the section Declaration Of Attributes, and in

the section Using Attributes In A Declaration Of A Variable.

list-of-variables is a list of the variables in the declaration,

separated by commas.  A variable that appears by itself is

interpreted as a scalar.  Implicit group declarations (types:

boolean, input, output, flip_flop and node) can be done by using

square brackets.  The size of the group is specified as an integer

expression inside the brackets.  (e.g. output x[5]; )

In the same way, an integer array may be defined for type integer.

String variables may not be specified with brackets since there is no

such thing as an array of strings.

A variable size boolean group is specified by omitting the size

inside the brackets.

For example:

integer A[4],B;

bool  C,D[5],E[];

In this example, A is an integer array with 4 elements, B is a scalar

integer, C is a scalar boolean, D is a boolean group with 5 elements,

and E is a variable size boolean group.

2.5.2 EXPLICIT GROUP DECLARATION

An explicit group declaration declares a new variable name as a

group.   However, the elements of the group are specified explicitly,

usually in terms of other variables that were already declared.  All

the elements specified must be of the same type.  You may not mix for

example flip-flops and outputs in the same group.

The syntax of the explicit group declaration is:

group  name = [ list-of-elements ] ;

name is the name of the group to be declared.  list-of-elements is a

comma separated list of elements to be included in the group.   The

rightmost element in the group will serve as the first element (index

Page 65

REFERENCE MANUAL

0) and the leftmost element in the list will serve as the last

element of the group.

Except for names of scalar variables, the list may include also names

of other groups (All elements of the group are included), single

elements of other groups (By using an index) or a range of elements

from another group, by specifying the range in square brackets.  For

example, if A,B,C were declared as groups (implicitly or explicitly)

then:

group X = [A, B[2], C[7,2] ] ;

declares X as a group which includes: all elements from A, a single

element from B, and elements 2 through 7 from C.

When using an explicit group declaration to declare a boolean group

(that contains only boolean variables) you may also use the digits 0

or 1 to create private elements that are initialized to either global

zero or one respectively. For example:

group Y = [a,b,1,1,c,0,1];

Notice that an explicit group declaration inside a function, may

include either local or global variables, but may not include

parameters of the function.

2.5.3 DECLARATION OF ATTRIBUTES

Before an attribute can be referred to in a declaration statement of

any variable, the name and type of the attribute itself must be

declared.

Attributes may be specified only for network elements.

Different attributes may be specified for different types.  For

example, you can elect to define an attribute by the name clock only

for flip-flops, so it can be used later only in flip-flop

declarations.

The declaration of an attribute has the general syntax:

attrib-type  attrib-name  value-specifier;

An example for declaring an attribute can be found in the next

section.

attrib-type is one of: input_attrib, output_attrib, node_attrib or

ff_attrib for input, output, node or flip-flop respectively.    In

some cases a shorthand can be used: If attrib-type is anyout_attrib,

the attribute is defined for all three types: output, node and

flip-flop.  (It is equivalent to declaring the same attribute for the

three basic variable types)

Page 66

REFERENCE MANUAL

attrib-name is the name of the attribute

value-specifier may have one of the following forms:

The first form is a list of zero or more strings separated by spaces.

This form is called Enumerated Type Attribute.    Each of the strings

may contain any character. However, if the string contains special

characters (e.g. a space) or is longer than 32 characters, it must be

enclosed in double quotes.   A string may also be empty ("" is an

empty string).   The value of an Enumerated Type Attribute is a

string that is taken from this list.

The second form is the two character string %s. This form is called a

String Attribute.  The value of a string attribute may be any ASCII

string.

The third form is the two character string %n. This form is called a

Numeric Attribute.  The value of a numeric attribute may be any

integer value.

The fourth form is the two character string %v. This form is called a

Variable Attribute.  The usage of this attribute is to point to

another variable.  (The value that is assigned to the attribute is

the name of the variable pointed to).

2.5.4 USING ATTRIBUTES IN A DECLARATION OF A VARIABLE

Consider the general declaration syntax:

type  attribute-list  list-of-variables;

The attributes specified in attribute-list are shared by all the

variables in the list list-of-variables.   The list of attributes can

be separated into several sets of parenthesis, which is syntactically

equivalent to specifying all the attributes in one list.

Consider the following example:

input clk1;

ff_attrib  latch;

ff_attrib  clock %v;

ff_attrib  delay %n;

ff_attrib  comment %s;

ff_attrib  fftype  D RS;

flip_flop (latch, fftype = D, clock = clk1)

(delay = 5, comment="PN-02117")  x,y;

input  clockbus[8];

flip_flop (fftype = RS, clock = clockbus) gx[8];

Two scalar flip-flop variables are declared (x and y), with five

attributes: latch, fftype, clock, delay and comment.  The 8 element

group gx is declared with attributes: fftype and clock.

Page 67

REFERENCE MANUAL

latch  and fftype are Enumerated Type Attributes. latch is declared

with an empty list, and is not assigned any value in the declaration

of x and y.  fftype on the other hand is assigned the value D in the

declaration of x and y and the value RS in the declaration of gx.

clock is a Variable Attribute, and the value assigned to it in the

declaration of x and y is the name of the variable clk1.

In the declaration of gx however, each element is assigned a

different value of clock taken from the group clockbus.  clock has

value clockbus[0] for gx[0], clockbus[1] for gx[1] etc..  (additional

discussion follows below)

delay is a Numeric Attribute, and is assigned the value 5.

comment is a String Attribute and is assigned the value "PN-02117".

As shown in the example, a value is assigned to an attribute by using

the equal sign.  When assigning values to attributes notice the

following:

Values of Enumerated Type Attributes must be enclosed in double

quotes only if the value contains special characters or is longer

than 32 characters.  If no value is specified (As in latch), a

default value of zero is assigned. (If the list of strings is not

empty it points to the first string in the list)

The value of an Enumerated Type Attribute is usually associated with

an integer number which is the position (or index) of the string in

the list.   The index of the first string in the list is 0.

An Enumerated Type Attribute that is specified with an empty list may

not be assigned any value.   The only information that is available

on a variable regarding such an attribute is whether the variable was

declared with that attribute or not.

Values of String Attributes must be enclosed in double quotes.

Values of Numeric Attributes must evaluate to numbers that are in the

range accepted by the compiler for integers.

Variable Attributes have a value which represents a scalar network

element.  However, in a declaration, a group of network elements may

also be specified.  In this case, the elements of the group are

matched in order to the variables in the declaration.  If the group

has less elements than the number of variables or elements in the

declaration, the last element of the group is used for all the

remaining variables.  As an example, consider the following

declaration section:

Page 68

REFERENCE MANUAL

ff_attrib xxx %v;

input G[3];

flip_flop (xxx = G[0]) Y0;

flip_flop (xxx = G[1]) Y1;

flip_flop (xxx = G[2]) Y2;

group Y = [Y2,Y1,Y0];

The above declaration section can be written more efficiently as

follows:

ff_attrib xxx %v;

input G[3];

flip_flop (xxx = G) Y[3];

2.5.5 FUNCTIONS

BOOL supports two kinds of functions: built in functions and user

defined functions.

Built in functions are built into the compiler and the user may only

call them.   User defined functions on the other hand are defined

using BOOL statements.

Built in functions are described in a separate chapter.  All the rest

of the discussion here is on user defined functions which here, are

referred to shortly just by the name functions.

The syntax supporting functions is very similar to the K&R definition

of the C language.  The function declaration has the following

general format:

type  name (formal-parameters-list)

formal-parameters-declaration

{

declaration-of-local-variables

executable-statements

}

type is the type of value returned by the function, and may be only

one of: integer, string, boolean or group.

The type group indicates that the function returns a boolean group.

The size of the returned group is determined by the size of the value

specified in the return statement(s),  and is not known therefore

until the function executes a return statement.

name is the name of the function and is used when calling the

function.

formal-parameters-list is a list of names of formal parameters,

separated by commas.

Page 69

REFERENCE MANUAL

formal-parameters-declaration are declaration statements that declare

the type of the formal parameters that appear in formal-parameters-

list.  All and only declaration of formal parameters may appear in

that section.

declaration-of-local-variables are declaration statements of all

variables that are local to the function (Local variables).

executable-statements form the rest of the function body and consist

of all other statements.  Calls to other functions is legal.

However, recursive calls are not allowed and will be flagged by the

compiler (This means that a function may not call itself directly or

indirectly through calls to other functions).

The { }  are part of the syntax and define the function body.

2.5.6 DECLARING FORMAL PARAMETERS OF A FUNCTION

Formal parameters may be any of the variable types recognized by the

compiler.  The declaration of scalar variables of any type (string,

integer, boolean, input, output, node, flip-flop) are the same as for

global variables.

Group variables and integer arrays are declared similar to global

variables, but without specifying the size, (Nothing appears between

the square brackets).   This leaves the size of groups or integer

arrays unspecified until the function is actually called.

2.5.7 DECLARING LOCAL VARIABLES OF A FUNCTION

Not all types are allowed as local variables of a function.   The

only types allowed are: string, integer, integer array, boolean and

boolean group (fixed size and variable size).   An equivalent

statement is that all types are allowed except network elements.

The syntax is identical to the syntax used for global variables

declaration.

Explicit group declarations is legal, and any mixture of local and

global variables is allowed as long as their types match.  The syntax

is the same as for global variables.

2.5.8 CALLING A FUNCTION AND PARAMETER PASSING

A function may be called in one of two forms.

In the first form, the name of the function with the list of actual

parameters is specified as a statement by itself, ended by a

semicolon.  The function is called, executed, and the returned value

is discarded.

Page 70

REFERENCE MANUAL

In the second form, the name of the function is mentioned as part of

an expression.  The function is called, executed, and the returned

value is used in the expression.

When a function is called, a comma separated list of actual

parameters is specified with the call.  Each actual parameter is

either a variable or an expression.  The actual parameters in the

list are matched according to their position, with the list of formal

parameters of the function.

If an actual parameter is an expression, the expression is evaluated

and the value is passed to the function.  We say in this case that

the actual parameter was passed by value.   We can force a variable

to be passed by value by enclosing it in parenthesis.   It is then

considered as an expression.   Notice that any expression may yield

only one of the data types: string, integer, boolean or boolean

group.   The corresponding formal parameter must match the type of

the expression, unless a type conversion is possible (In this case

the only possible type conversion is from integer to boolean).

If an actual parameter is just the name of a variable, its type is

checked against the type of the corresponding formal parameter.  If

the types match exactly, the variable is passed by reference.  This

means that any manipulations done by the function on the formal

parameter will be reflected in the variable.

If the types do not match exactly, a type conversion is done, and the

actual parameter is passed by value.  Any assignments to the formal

parameter will not be reflected in the actual parameter.

The only type conversions that are allowed are from integer, input,

output, node or flip-flop to boolean (scalar or group).  If the

actual parameter is a group, the formal parameter must also be a

group and vice versa.  The type conversion has the same effect as an

assignment statement to the target type.

Notice that a function have no way of finding out if a parameter was

passed by value or by reference if the formal parameter has type

string, integer, boolean or boolean group.  However, if the formal

parameter is an integer array or a network element, then actual

parameters may only be passed by reference.

2.5.9 RETURNING FROM A FUNCTION

The function logically returns to the calling program when a return

statement is encountered.

The syntax of a return statement is:

return expression;

Page 71

REFERENCE MANUAL

expression is any expression which must evaluate to the type of the

function (The type of the function is specified at the function

header).

If the type of the function is boolean, then expression may also have

type integer. A type conversion is done in that case.

The value of expression is returned to the calling program.

Notice that in some cases the compiler may actually execute several

return statements before returning to the calling program.   This may

happen for example when both the if and the else part of a condition

based on an input are evaluated, and both have return statements.

You will notice that the compiler actually executes both return

statements if you request BOOL to print a trace of all statements

being executed.

2.5.10 COMPOUND STATEMENT

A compound statement, also known as block, is a group of statements

enclosed in { }.   A compound statement may be used instead of any

simple statement.   Compound statements may be nested.

2.5.11 SIMPLE ASSIGNMENT STATEMENT

When performing a simple assignment, the compiler checks that the

assignment does not conflict with any previous assignments to the

target variable.  (A conflict means an assignment of a different

value under the same or overlapping input conditions).   If a

conflict exists, the assignment is performed (The new value replaces

the previous value) but a warning message is generated.  To avoid the

warning message, use the overriding assignment.

A simple assignment statement has the syntax:

target = expression;

When assigning a value to a string, integer, boolean, output or a

node variable,  target is the name of the variable.   This is also

true for the group form of the last three types.

target may also be an element of a group or an element of an integer

array.   In that case it must be properly indexed by an integer

expression.

If the target is a flip-flop, a flip-flop group or an element of a

flip-flop group, it must be suffixed with the string :D  (or :d),

which indicates that the assignment is done to the D input of the

flip-flop, and will be reflected in its output only after a clock is

applied..

Page 72

REFERENCE MANUAL

The rules for matching the types of target and expression are as

follows:

A string target may only be assigned a string expression.

An integer target may only be assigned an integer expression.

An input may not serve as a target in an assignment statement.

Other types of scalar boolean class variables may only be assigned a

boolean expression or an integer expression.

If the expression is integer, it will be converted to boolean.   A

value of 0 is converted to global zero, and any non zero value is

converted to 1.  If the value is not 0 or 1, a warning is issued.

A group boolean class variable may only be assigned a boolean group

expression.   If the size (Number of elements) of expression does not

match the size of target, a size conversion is done.   The rules for

size conversion are as follows:

If the size of expression is less than the size of target, the value

of expression is extended by adding global zero elements as most

significant elements.

If the size of expression is larger than the size of target, and

target is not a variable size boolean group, the upper (Most

significant) elements of expression are ignored.  If however, target

is a variable size boolean group, the size of target is expanded to

match the size of expression.

2.5.12 OVERRIDING ASSIGNMENT

The overriding assignment works functionally the same as the simple

assignment, except that there is no check for conflicts with previous

assignments.  The new value simply replaces the old value.

The syntax of the overriding assignment is:

target := expression;

Page 73

REFERENCE MANUAL

2.5.13 CLEAR AND ASSIGN

BOOL supports a special kind of assignment that might be used in

special cases, usually when the same boolean variable has to be re-

used several times (e.g. in a loop), where each time it represents a

different signal in the design.  In that case we do not want

assignments to be combined.   The clear and assign statement will

work only on boolean variables or boolean groups.  It clears the

contents of the target to the initial (Virgin) state before the

assignment is done.  As a reminder, the virgin state of a scalar

boolean variable is a global don't care.   The syntax of this

assignment is:

target <= expression;

Actually, if target does not appear as part of expression, the above

statement is equivalent to the two statements:

clr (target);

target = expression;

This assignment should be used with great care since it destroys any

previous data that was accumulated in target from previous

assignments.   (It is treated as if no previous assignment was done

to it).

2.5.14 IF...ELSE STATEMENTS

The syntax of the if statement is:

if (expression) statement1

or:

if (expression) statement1

else   statement2

statement1 and statement2 may be any legal executable statement,

including compound statements or other if statements.

expression must evaluate to either an integer expression, or a scalar

boolean expression.

In both forms,  statement1 is executed only if expression is true

(Has value 1).  In the second form, statement2 is executed only  if

expression is false (Has value 0).

Notice that if expression has type boolean its value may also be

undefined (Or don't care).  In that case neither of statement1 or

statement2 is executed.

COMMENTS:

Page 74

REFERENCE MANUAL

If expression has type boolean, the compiler will unconditionally

analyze statement1 (and in the second form also statement2) since it

has to account for both cases, when expression is 1 and when

expression is 0.  You will notice that if you request execution

trace.

If expression has type integer, it represents a global condition that

does not depends on any input value.   In that case, the compiler

will do a shortcut (e.g. It will not even analyze statement1 if

expression evaluates to 0).

The keywords .if and .else are also accepted if expression has type

boolean for backward compatibility with previous versions.

2.5.15 WHILE LOOP

The while statement has the following syntax:

while (expression)

statement

expression must evaluate to an integer value.

statement  may be any executable statement (including compound

statement or even another loop statement).

statement is repeatedly executed as long as expression has a non zero

value.  If expression is initially zero, statement is not executed at

all.

If statement is a compound statement, there is also a way to modify

execution flow by using break or continue statements.  break will

cause the loop to terminate, while continue will just cause the

current iteration to terminate and to restart a new iteration.

The break statement has the following syntax:

break;

Execution of a break statement will cause all remaining statements in

the loop to be skipped, the loop is terminated, and control is

transferred to the statement following the loop.

The continue statement has the following syntax:

continue;

Execution of a continue statement will cause all remaining statements

in the loop to be skipped.  The loop is then restarted, by checking

the value of expression, and if it is non zero, execution is

restarted from the beginning of statement.

Page 75

REFERENCE MANUAL

2.5.16 FOR LOOP

A for loop is similar to a while loop but is aimed for loops with a

pre-known number of iterations.  break and continue may be used

inside a for loop with the same effect as in a while loop.

A for loop may appear in one of two possible forms.  The first, more

general form is:

for ( intvar = start , end , increment )

statement

intvar is a simple integer variable.

start, end and increment are each integer expressions, which are

calculated only when the loop is initiated  (They are not

recalculated each iteration).

statement may be any executable statement including a compound

statement.

If  increment is positive then the above for statement is equivalent

to the following:

intvar = start;

while (intvar <= end)

{

statement

intvar = intvar + increment;

}

If increment is negative, then the above for statement is equivalent

to:

intvar = start;

while (intvar >= end)

{

statement

intvar = intvar + increment;

}

The for loop has a second form without the increment part.   In that

case an increment of 1 is used.  The syntax is:

for (intvar = start , end)

statement

This is equivalent to:

for (intvar = start , end , 1)

statement

Page 76

REFERENCE MANUAL

2.5.17 .SWITCH STATEMENT

The basic syntax of the .switch statement is:

.switch (grvar)

{

case val1:

statement

statement

........

case val2:

statement

statement

........

case val3:

statement

statement

....................

}

grvar is the name of a group variable.

Each of the lines marked as statement represents any type of

executable statements (even compound statements or other .switch

statements).

val1, val2, val3,... represent integer expressions.

The line of dots indicates a repetition.  Any number of statements

may follow each case and any number of cases may appear in a .switch

statement, inside the braces .

The way the .switch statement operates is as follows:  The value of

the group grvar is interpreted as an integer number (where each

element represents one bit).  The value is checked against each of

the values that appear after the case keyword (val1, val2, val3,...).

If it matches any of the values, all the statements following it, up

to the next case keyword (or the closing } ) are executed.

The following text explains possible variations from the basic

syntax.

The character @ is accepted as a synonym for the keyword case.

Instead of only one value, several different values, separated by

commas may be specified after the case keyword.  The statements

following the case will be executed if grvar is equal to any of the

values.

When grvar is the name of a flip-flop group, the .switch statement

can be used to describe a state machine.

Page 77

REFERENCE MANUAL

grvar in that case serves as the state register.   Each state is

defined by the value following the case keyword, and all the

statements following it (up to the next case) belong to that state.

State transitions are defined with the statements jump and stay. The

syntax of a jump statement is:

jump state;

state  is an integer expression which specifies the next state.  (The

next state of a state machine is the value fed to the D inputs of the

state register).

The stay statement has the following format:

stay;

The stay statement will cause the state machine to stay in the

current state.

The jump and stay statements behave similarly to an assignment done

to the state register grvar, and will check for conflicts with

previous assignments to grvar.  A conflict may occur if there were

previous assignments to grvar, or if there are several conflicting

jump statements from the same state.  If a conflict is detected, a

warning is issued.  To avoid the warning, the keywords xjump and

xstay may be used instead of jump and stay.

2.5.18 SETPARAM STATEMENT

The setparam statement enables the user to change run time default

parameters that relate to different functions of BOOL, such as output

formats, the simulator, and the minimizer.

The setparam statement has the syntax:

setparam param-name value ;

param-name is the name of the parameter whose value should be

modified.

value is the new value that should be assigned to param-name.  The

type of the value depends on the parameter, and can be either an

integer, a string, or a keyword selected from a predefined list,

specific to that parameter.   string values must be enclosed in

double quotes only if they contain special characters, or are longer

than 32 characters.

For controlling output formats, (Especially boolean equations),

param-name is selected from the following list:

not_operator  -  define string to replace the ! mark.

Page 78

REFERENCE MANUAL

or_operator  -  define string to replace the | mark.

and_operator  -  define string to replace the & mark.

assign_operator  -  define string to replace the = mark.

endeqn_mark  -  define string to be placed at end of equation instead

of the ; mark.

continuation_line  -  define a string to be used instead of an end of

line mark for continuation lines.

name_index_prefix  -  define string to replace the [ mark.

name_index_suffix  -  define string to replace the ] mark.

reformat_extension - value can be either yes or no.  If set to yes,

the string which identifies the index of an element in a group will

be located just before the leftmost period in the name, if there is

one, instead of being catenated to the end of the group's name.  For

example: element #3 of the group XX.R is printed as XX[3].R instead

of XX.R[3].   Default setting is no.

zero_value  -  define string to replace the digit 0 when used as a

result (When it appears alone in the right side of an equation).

one_value  -  define string to replace the digit 1 when used as a

result (When it appears alone in the right side of an equation).

ff_extension  -  define string to replace the :D extension (which

indicates the 'next state' of a flip-flop).

chars_per_eqn_line  - define maximal number of characters that will

be printed on a single output line.

Notice that except for chars_per_eqn_line which accepts an integer

value, all the other parameters above accept string values.

For controlling the simulator, param-name can be:

check_sim_dc - if value is the string yes, the simulator will check

all nodes, outputs and flip-flop during simulation for don't care

condition.  If the input stimuli drives any variable of the above

type to a don't care value, a warning message is generated.   If

value is the string no, no checks are done.  The initial default

setting is no.

To get a * or X indication for don't cares when printing simulation

results, check_sim_dc must be set to yes.

max_sim_iterations  -  value is an integer number which specifies the

maximal number of simulation ticks that the simulator should evaluate

after executing a values statement or a clockff function.   If the

network does not stabilize after evaluating the specified number of

ticks, the simulator decides that this is probably due to an

Page 79

REFERENCE MANUAL

asynchronous loop which causes oscillations, and the program quits

with an error message.  The default setting is 256.

For controlling the minimizer, param-name can be:

default_minlevel  -  specifies the default minimization level to be

used in the minimize function.  value can be any positive integer

value between 1 (simplest algorithm) to 63 (absolute minimization).

The initial default setting is 1 unless the -M switch appears in the

command line.

default_polarity  - specifies the default polarity that should be

used in the minimize and lminimize functions.  value can be only one

of three choices:

pos  - minimize with positive polarity.

neg  - minimize with negative polarity.

prog - minimize with programmable polarity, meaning: choose the

better solution from either positive or negative polarity.  (Here,

better means less product terms).

The initial default setting is pos.

2.5.19 VALUES STATEMENT

The syntax of the values statement is:

values ( var1 = val1 , var2 = val2, ... );

var1, var2, ...  are names of variables.  The variables must be any

of the types: input, node or flip-flop, and may be either a scalar or

a group.

val1, val2, ...  are expressions.  Each expressions must match the

basic type of the variable.  If the variable is scalar, the

expression must evaluate to either an integer value or a boolean

value.   If the variable is a group, the expression must also be a

group.

The effect of the values statement is to attach a simulation value to

a variable and freeze that value during subsequent simulation steps.

This is done in the following order:

The expressions val1, val2, ...  are evaluated and their simulated

value is calculated.

The calculated simulated value is assigned to all the variables

(var1, var2, ...)   at once.

The internal simulation clock is advanced until the network

stabilizes. (A simple unit delay model is used).

COMMENTS:

Page 80

REFERENCE MANUAL

The simulation value of the variables var1, var2, ... is froze until

the next value statement.  This means that the simulation value does

not change regardless of any events that might otherwise toggle it.

This is significant of course, only to nodes and flip-flops, since

the only way to change simulation value of inputs is by using the

values statement.    If you only want to release the freeze, you may

issue a values statement with no operands, as in:

values () ;

Usually the expressions val1, val2, ...  are either 0 or 1  (For

scalar variables)  or a group containing a global value (e.g. 8@61).

The general case of using a boolean (or boolean group) expression is

for specifying hardware connections, that were not specified in the

description of the circuit, just for simulation.

2.5.20 END STATEMENT

The end statement has the following format:

end;

It must appear as the last statement in the program.

Page 81

CHAPTER 3: RUNNING THE PROGRAM

3.1 GENERAL

A Bool program is processed by the Bool compiler in three passes:

In the preprocessor  the input file is read into memory.   Comments

are stripped off, and preprocessor directives (e.g. #include,

#define) are expanded (or executed).    If a file by the name

bool.def exists in the current directory, or in any of the standard

locations that are searched by Bool, then this file is included

before any other file, as if your program has the directive:

#include "bool.def"

at the top of the main program file.

bool.def will usually be used for setting up program defaults that

are common to all your designs.

The preprocessor pass may produce error messages regarding incorrect

use of preprocessor directives, unbalanced comment marks, missing

include files etc.

After the preprocessor is finished, if no errors were detected, the

compiler starts the next phase which is called first pass.

During the first pass, the syntax of the source program is thoroughly

checked for errors.

If the first pass is completed successfully, the compiler starts the

second pass which is the execution phase.   Only during the second

pass the program is actually executed.   Any error detected during

this phase will cause the program to terminate.

3.2 COMMAND LINE PARAMETERS

The BOOL compiler is invoked from the command line level by the

command:

bool [switches] inpfile [outfile1] [outfile2] ... [outfile8]

The square brackets show optional parameters.  The only parameter

which must appear is inpfile.  inpfile is the name of your main

source file   The switches and the output files are optional.

Page 82

RUNNING THE PROGRAM

The location of the switches in the command line is immaterial and

they can appear anywhere in the line.

outfile1,...,outfile8 are names of output files, where the results

should be written.  Up to eight output files can be opened in this

way.  (The maximal number of files may also be limited by your

operating system).  The output files are numbered according to the

order in which they appear in the command line.  If less than 8 files

are declared in the command line, then all the undeclared numbers, up

to 8, are connected to the standard output device, which is the

screen, if not redirected.  Separate line and column number pointers

are maintained for each of the output files whether they appear in

the command line or not.

Output files specified in the command line are assigned file numbers

as follows:  file number 1 is assigned to outfile1, file number 2 is

assigned to outfile2, etc..   File number 0 is always assigned to the

screen.   Except for file number 0, other file numbers can be

reassigned in your program by use of the built in function

open_outfile.

Built in functions that write to output files, make use of the file

number.

Example.  Under DOS, type as a response to the DOS prompt the

following line:

bool DESIGN1.B RESULTS.O  REF.O

In this example, the input file is DESIGN1.B .  All output from the

print() function is sent to RESULTS.O .  printf statements which

specify the number 1 as their first parameter (file number), would

also write to RESULTS.O .  printf statements that specify file number

2 write to the file REF.O .  Results from printf statements that

specify a file number greater than 2 are written to the screen.

All the switches appearing in the command line must start with a

hyphen.  Lower or upper case can be used for the first character

(following the hyphen) in the switch.  The switches can appear in any

order, and anywhere in the command line.  Following is a list of the

switches:

-Ddatafile  -Iincludepath  -Llistfile  -Mmm   -Ttracefile  -V

-Ddatafile   This switch specifies a name of an input data file.

Several data files (up to 8) can be specified in this way and they

are assigned consecutive numbers according to their order of

appearance in the command line.  The first (leftmost) data file

declared in this way is assigned the number 1.  Data files can be

read by the built in function readpla.  For more information refer to

the description of readpla.

-Iincludepath   This switch specifies an alternative search path for

include files.  When an #include directive is encountered in the

Page 83

RUNNING THE PROGRAM

file, the preprocessor first tries to read the file using the name

specified in the directive without modifications.  If for some

reason, it cannot access the file, it tries to catenate the string

includepath in front of the name specified in the directive, and use

this new string to access the file.

-Llistfile   This switch causes all error and warning messages to be

sent to the file listfile.  If the -Llistfile does not appear in the

command line, all error and warning messages are sent to the screen.

-Mmm   defines the default minimization level.  mm is a decimal

number with one or two digits, in the range 0 to 63.  Higher numbers

are truncated to 63.  The default minimization level is used by the

minimize built in function.  If -Mmm does not appear, the default

minimization level is set to 1.

-Ttracefile  This switch causes execution tracing information to be

sent to the file tracefile.  The information includes file name, line

number and column number of each statement that is analyzed by the

compiler.  If only the characters -T are specified (without

tracefile) then the trace information is sent to the screen.

-V   This switch causes verbose information to be sent to the screen.

Important activities are reported to the screen, such as the name of

each variable being minimized, what files are read in etc.  Also a

summary on memory usage is printed at the end.   The number printed

includes only the net amount of memory requested by the BOOL compiler

from the operating system during execution, and does not include

system overhead, and memory occupied by the compiler code.

Example:

BOOL  mydes.b  -m9 res.o -Lerr.lst -I\LIB\ -Ddt1.pla -Ddt2.pla

In this example, the input file is mydes.b .

The default minimization level is 9.

Errors and warnings are sent to the file err.lst

Results written to file number 1, are sent to the file res.o

Include files which are not found in the current directory are

searched by the name \LIB\filename  where filename is the name

specified in the #include directive.

(In this example, the separating character is backslash, used in DOS.

In Unix for example, you will use a slash instead.)

If the file cannot be read, the compiler may search also in other

locations which are specific to your operating system.  In

particular, if an environment variable by the name BOOL exists, it

will be used as a search path.  For more details consult the specific

Bool User's Guide for the operating system that you are using.

Page 84

RUNNING THE PROGRAM

Two input data files are specified:  dt1.pla and dt2.pla, both can be

read inside the program by the built in function readpla.  dt1.pla is

assigned the number 1, and dt2.pla is assigned the number 2.

Page 85

CHAPTER 4: BUILT IN FUNCTIONS LIBRARY

4.1 GENERAL INFORMATION

The built in functions library provides services that are hard or

impossible to achieve through regular statements.  This includes:

accessing the minimizer or the simulator, handling files, generating

output etc.

Built in functions, unlike user defined functions are more flexible

in the number and types of actual parameters that they accept.

In addition, actual parameters that are scalar network elements

(input, output, node, or flip-flop) can be passed to a built in

function indirectly, through the $v macro, by specifying the

"Variable Identification Number" of that variable.

The Variable Identification Number is part of the Status Word of the

variable, and is an integer number that uniquely identifies the

variable.   The Status Word of a variable contains information on the

variable and can be retrieved and manipulated by several built in

functions.

This mechanism enables a program to reference variables by using

integer numbers instead of names, which makes some tasks easier.

(For example, the mechanism is used by the external interface

functions to generate output files for all network elements).

4.1.1. PARAMETERS AND RETURNED VALUE

Some built in functions may accept several different types for a

specific parameter.

The term 'any boolean class scalar'  means that the variable can be

of type input, output, node, flip-flop or boolean.

The term 'any boolean class group' or just 'any group type' means

that the variable can be any of:  input group, output group, node

group, flip-flop group or boolean group.

The term 'any boolean class' includes both, or in other words, it

stands for: any boolean class scalar or any boolean class group.

The term 'any scalar network element means any variable of type

input, output, node, or flip-flop.   Since by definition, network

elements are variables, a parameter that is specified as above may

not be an expression.

Page 86

BUILT IN FUNCTIONS LIBRARY

The term 'any scalar or group network element' means any variable of

type input, output, node, flip-flop, input group, output group, node

group or flip-flop group.   Note that here also, all parameters must

be variables.

The synopsis part may specify that the returned value from a function

is 'none'.   This does not mean that you will get an error message if

you try to use the function in an expression.   It only means that no

meaningful information is returned.  (Usually in this cases, a value

of integer 0 is returned).

Some functions require that the parameter passed to them is a

variable.   This is specified by the phrase:  "parameter xxx must be

a variable".   This means that the parameter must be a name of an

existing variable.   It may not be an expression.  (In other words,

it must be passed by reference).

Some functions accept a variable number of parameters.  This is

usually specified in form:

function_name(a,b,c,...);

This indicates that any number of parameters can be specified (Up to

32 which is the maximal number of parameters allowed by the

compiler).

4.1.2 STATUS WORD OF A VARIABLE

The Status Word of a variable is a 32 bit integer number that

contains information on a scalar boolean class variable.   Status

words can be retrieved and manipulated by several built in functions

which are described in the section: Accessing Information on Status

Word and Attributes of Variables.    Network elements (input, output,

node or flip-flop) are uniquely identified by their Status Word.

Boolean variables, however, are not.

The Status Word has several fields as follows:

Bits 0-15    are the index of the variable in the corresponding

universal group.   (e.g.  for an input variable, it is its index in

univ.inpgroup).  For boolean variables, this field is always 0.

Bits 16-19   indicate the type of the variable as follows:

0000  -  input

0001  -  output

0010  -  node

0011  -  flip-flop

0100  -  boolean

Bits 0-19  together form the Variable Identification Number, which is

an integer number that uniquely identifies network elements

(variables of type input, output, node or flip-flop).

Page 87

BUILT IN FUNCTIONS LIBRARY

Bit 20   -  Is 1 if the variable contains don't cares (has a non

empty don't care set) and 0 otherwise.

bit 21   -  Is 1 if the variable was minimized with the built in

minimizer, 0 otherwise.  (minimization through extmin will reset this

bit to 0)

bit 22   -  Is 1 if the variable should be used in negative polarity,

0 otherwise.  (This bit is set if negative polarity was chosen during

minimization).

bits 23-29  Reserved for future expansion.

bits 30,31  Always zero.

Any bits above bit 31 are ignored.  (This is applicable to

implementations that use more than 32 bits for an integer)

4.1.3 VARIABLE IDENTIFICATION NUMBER AND THE $v MACRO

If the Variable Identification Number (See definition above) of a

network element variable is known, the variable itself can be

retrieved very easily from the appropriate universal group.  An

easier way to retrieve the variable itself is through the use of the

$v macro.

If var-ident is an integer expression that evaluates to the Variable

Identification Number of a variable, then $v(var-ident)  is replaced

with the name of the variable.   However, this construct is legal

only when the macro appears as an actual parameter to a built in

function.  (This is the only case where type checking is done only

during execution).

The $v macro is using only bits 0-19 of var-ident.  Any other bits

are ignored.  This means that the complete Status Word can be used in

the $v macro, and there is no need to mask out the high order bits.

For example, if A is a flip-flop variable, then the statement:

minimize (A);

is equivalent to:

minimize ($v(get_status(A)) );

In this example, get_status(A) evaluates to the Status Word of A

(which contain as part of it the Variable Identification Number).

Therefore $v(get_status(A)) is replaced with A when minimize is

called.

Page 88

BUILT IN FUNCTIONS LIBRARY

4.1.4 ORIGINAL NAME OF A VARIABLE

Some built in functions such as print or getvarname make use of the

name of the variable that is passed to them as a parameter.  In that

case, they use what is called the "original name of the variable"

The original name of a variable is usually the name by which that

variable was declared.  However, in some cases, this name is only an

alias, and the original name is traced back.  For example, the name

of individual elements in an explicit group declaration are used

instead of the group's name.  Also, names of parameters of functions

are traced back if they were passed by reference. (This trace may go

through several levels of calls).

4.2 INPUT / OUTPUT HANDLING

This section contains functions that handle all operations associated

with files: opening, closing, reading writing and getting status

information on files.   This does not include the program source

files, the output list file or the trace file which are handled

automatically by the compiler.

BOOL allows the program to manipulate up to 8 input files and 8

output files at the same time in addition to the standard output (The

screen).

Currently, the only format supported for input data files is Berkeley

PLA format.  There is no such restriction on output files since they

can be formatted to any format by using formatted print statement

(printf function).

Opening and closing of files is usually done automatically by the

compiler.   Each file has a unique number associated with it, called

a file number.  There are 8 numbers for output files (1 to 8) and 8

numbers for input files (1 to 8).  Files can be specified in the

invocation line and are numbered according to their order of

appearance in the line.

For example, if BOOL is called to compile the source file des1.b with

the following command:

bool des1.b out1.o  out2.o  -Din1.dat -Din2.dat

Then out1.o, out2.o are output files with file numbers 1 and 2

respectively, and in1.dat, in2.dat  are input files with file numbers

1 and 2 respectively.

The file number is used by many I/O routines to identify the file.

Output file number 0 is reserved for the standard output.  Any output

sent to file number 0 will be written to the screen.

Page 89

BUILT IN FUNCTIONS LIBRARY

The mechanism described above can be overridden under program control

by calling the functions open_inpfile and open_outfile.   With these

functions files can be opened and closed for reading or writing and

attached to any file number (between 1 and 8) during program

execution.  Moreover, the same file number can be reused.

Any output sent to a file number that was not open for output (either

in the invocation line or by calling open_outfile, will be diverted

to the screen.

4.2.1 fileinfo    Accessing information on an output file

Synopsis:

val = fileinfo(filenum,charcnt,linecnt);

Type of parameter filenum:   integer

Type of parameters charcnt, linecnt:   integer (used for output)

Type of returned value:   integer.

fileinfo returns status information on an output file.

filenum is the file number of the output file.

If a specific file with that number was opened for output by the

invocation line, the function returns the value 1.  Otherwise, it

returns the value 0.  (If a value of 0 is returned it also means that

all output sent to this file is directed to the standard output

device which is usually the screen.) A value of 2 is returned if the

number filenum has an illegal value.

linecnt returns the number of lines that were already sent to this

output file.  (It counts the number of end of line characters that

were sent to the file).

charcnt returns the number of characters already sent to the file on

the current line.  (It is zero after writing an end of line

character.)

4.2.2 get_source_name    get name of source file

Synopsis:

s = get_source_name(mode);

Type of parameter mode:   integer.

Type of returned value:   string.

Returns a string which contains the name of the main source file (The

name specified on the command line).   mode can be either 0 or 1.

If mode is 0, the complete name is returned.

Page 90

BUILT IN FUNCTIONS LIBRARY

If mode is 1, any extension, if there is any, is stripped off the

name.  The extension is defined as any string that follows a dot.

The dot is also stripped off.

4.2.3 open_inpfile    Opening a file for input

Synopsis:

open_inpfile(filenum,filename);

Type of parameter filenum:   integer.

Type of parameter filename:   string.

Returned value:   none.

Closes any input file previously opened as file number filenum (if a

file was opened), and opens a new file for reading data.  The name of

the new file is specified by the string filename.  The file number of

the new file is specified by filenum.  If filename is an empty

string, the previous file is closed, but no new file is opened.

4.2.4 open_outfile    Opening a file for output

Synopsis:

open_outfile(filenum,filename);

Type of parameter filenum:   integer.

Type of parameter filename:   string.

Returned value:   none.

Closes any output file previously opened as file number filenum (if a

file was opened), and opens a new file for output.  The name of the

new file is specified by the string filename.  The file number of the

new file is specified by filenum.  If filename is an empty string,

the previous file is closed, but no new file is opened.

4.2.5 print    Printing equations in standard format

Synopsis:

print(a,b,c,d,...);

Type of parameters a,b,c,d,... : any type except integer array.

Returned value: none.

Comments: Each of the parameters:  a,b,c,d,...  must be a variable.

Prints the values of the variables to output file number 1.  Values

of integer variables are printed in hex and decimal.    Values of

boolean class variables are printed as boolean equations.

Specifying a group or array will cause all its elements to be printed

individually.

Page 91

BUILT IN FUNCTIONS LIBRARY

The original name of each parameter is printed with its value.  All

parameters must be variables: expressions cannot be passed as

parameters.   The printout has the format:

name = ....

If a boolean class variable is partially specified (Has don't cares),

its value is printed as two boolean equations, which specify both the

on-set and the off-set of the variable.  The on-set of the variable

is printed as:

1(name) = ...

The off-set is printed as:

0(name) = ...

name is the original name of the variable being printed.

If a variable was minimized with negative polarity (Or with

programmable polarity, and a negative polarity was chosen) then the

result is printed as:

!name = ....

The format of the printing can be changed with the setparam facility.

(You can change things like line size, boolean operators etc.)

4.2.6 printf    Formatted print facility

Synopsis:

printf(filenum,format_spec,a,b,c,...);

Type of parameter filenum:   integer

Type of parameter format_spec:   string

Type of parameters a,b,c,... :   any type except integer array.

Returned value: none.

printf performs formatted print to file number filenum according to

the format specifier format_spec.  The parameters that are printed

are a,b,c,...  which may be expressions or variables.

format_spec may contain plain characters which are sent as is to the

output file, and conversion specifiers which always start with the

character '%'.  Each parameter from the list a,b,c,... must match one

conversion specifier in format_spec.   The value of each parameter is

formatted according to the conversion specifier.   For boolean class

parameters, the simulation value is used and formatted as an integer

number (Except that for groups, it might contain a different number

of bits than simple integer variables).

The following conversion specifiers are available:

Page 92

BUILT IN FUNCTIONS LIBRARY

%b   binary conversion

%o   octal conversion

%d   signed decimal conversion

%u   unsigned decimal conversion

%h   hexadecimal conversion with leading zero fill

%x   hexadecimal conversion with leading zero suppression

%s   string conversion

%c   character conversion

Comments:

The specifier %s requires a string type parameter and vice versa.

The specifier %c requires an integer type parameter.  The integer

value is interpreted as an ASCII value of a character.  This enables

the user to print out any character by specifying its code.

A field size modifier consisting of one or two decimal digits which

specifies the number of characters in the destination field may be

inserted immediately to the right of the '%' (As in %12d).   The

result will be formatted into a field with the specified size and

padded with spaces (or with zeros as explained below.)

The difference between the %x and %h is significant only for integer

type parameters, and specify whether to suppress leading zeros or not

if the field size is greater than the size of the formatted number.

When writing a group, the number of non blank characters in the

output field will reflect the size of the group (leading zeros are

not suppressed).

If check_sim_dc was set to on with the setparam statement, then each

boolean class parameter is checked during the conversion for a don't

care value.   If the simulated value is a don't care, the character X

is used instead of 0 or 1 to indicate a don't care in binary

conversion.   When a group is printed out, the location of the X in

the output string indicates therefore, the exact element where the

don't care occurred.  In other conversions where each character in

the output represents several elements in a group, don't cares are

treated as zeros, and the character * is inserted to the right of the

field to indicate that a don't care was detected somewhere.

Page 93

BUILT IN FUNCTIONS LIBRARY

4.2.7 printsim    Simple output of simulation results

See also:  printf, sprintf

Synopsis:

printsim(a,b,c,...);

Type of parameters a,b,c,... :   any type except integer array and

string.

Returned value:   none.

Comment: a,b,c,...  must be variables.  Expressions are not

acceptable.

This function prints to output file number 1 the simulation value of

the parameters a,b,c,...

The simulation value of each parameter is printed on a separate line

in several formats.  Don't cares are treated the same as in the

printf function.

4.2.8 readpla    Reading input files in PLA format

Synopsis:

inpnum = readpla(filenum,inpgroup,outgroup);

Type of parameter filenum:   integer.

Type of parameter inpgroup:   input group.

Type of parameter outgroup:   boolean group. (used for output)

Type of returned value:   integer.

readpla reads the contents of an input data file.

filenum is the number of the input file.

The data file must be in PLA format.  (A definition of this format

appears in the appendix.)

The number of input elements specified in the data file should be

less than or equal to the size of inpgroup.

The function returns an integer value which is the number of input

elements that actually appear in the input file.

Input elements in the file are matched to the elements in inpgroup

right to left.  The rightmost input element in the file is matched to

element number 0 of inpgroup.  If inpgroup contains more elements

than declared in the data file, the higher elements are ignored.

Output elements are read from the file right (LSB) to left (MSB). If

outgroup is a variable size group, its size is adjusted to the number

of output elements in the file. If the number of output elements in

the file is different than the size of outgroup, and outgroup is not

a variable size boolean group, then its higher elements are either

Page 94

BUILT IN FUNCTIONS LIBRARY

cleared to global don't care, or the leftmost elements in the file

are discarded.

Before reading the file, outgroup is cleared. (All previous

assignments to outgroup are discarded.)

4.2.9 writepla    Generating PLA output file

Synopsis:

writepla(filenum,inpgroup,outgroup);

Type of parameter filenum:   integer.

Type of parameter inpgroup:   input group.

Type of parameter outgroup:   any boolean class group.

Returned value:   none.

writepla writes the contents of outgroup to an output file according

to the order of input variables in inpgroup.  inpgroup may also be a

boolean group, but in that case the elements of inpgroup must be

exact copies of input literals.

Elements are written to the file right (LSB) to left (MSB) in PLA

format. (A definition of this format appears in the appendix.)

The input group inpgroup is used to define the order of appearance of

input elements in the file.  The rightmost input element in the file

corresponds to element number 0 in inpgroup.

Input variables which do not appear in inpgroup are ignored even if

the value of outgroup depends on them.

This function is available for upward compatibility with previous

versions of Bool.   The function writestd provides similar

functionality but is easier to use.   The only advantage of writepla

on writestd is that writepla enables you to control the order of

input literals in the file.

4.2.10 writestd    Generating several standard formats

Synopsis:

writestd(filenum,mode,a,b,c,...);

Type of variable filenum : integer.

Type of variable mode : integer.

Type of variables a,b,c,... : any boolean class.

writestd prints the variables a,b,c,... to an output file specified

by the file number filenum.    The output format can be selected from

a set of several generic formats by an integer number specified by

mode.

Page 95

BUILT IN FUNCTIONS LIBRARY

writestd is used mainly as a basic formatting tool for printing

boolean class variables in formats that are hard to generate with

printf.  When used in conjunction with other functions, such as

printf and getvarname, it can emulate the functionality of functions

such as writepla and print.

Currently, writestd can generate various flavors of PLA tables and

boolean equations.   Further control on format details can be

achieved through the setparam facility. (Except mode=1 and mode=2

which are unaffected by setparam settings).

Parameters that can be controlled by setparam are the format of

boolean equations and names, and the way continuation lines are

formatted.  Note that the parameters chars_per_eqn_line and

continuation_line will affect PLA matrix lines as well as boolean

equations output.

In PLA formats, The input literals will appear in the input section

of the PLA matrix, sorted according to their serial number, from

right to left (The literal with the highest serial number will appear

at the leftmost location in the matrix).

Detailed information on PLA tables is contained in a separate

section.

The following formats are currently available:

mode = 0

This is the basic generator of boolean equations.  This format

accepts only a single variable (The list a,b,c,... specified in the

synopsis is limited to one variable).  The contents of that variable

are printed as a boolean expression (Without the name of the

variable, or end of equation mark).    Note that if the variable was

minimized with negative polarity (Or with programmable polarity, and

a negative polarity was chosen), the inverse of the value is printed.

In other words, the expression is printed in the selected polarity

(which might be the inverse of the value of the variable).

mode = 1

generates a PLA file which contains only the keywords .i .o .type and

.end  with their associated information.    The variables a,b,c,...

appear in the output file as output literals in the same order (left

to right, as in the parameter list.  Elements of groups are printed

with the most significant element at the leftmost position and the

least significant element at the rightmost position.  All and only

input literals that are relevant, will appear in the input literals

section of the PLA table.

mode = 2

is the same as mode=1, but contain also the .ilb and .ob keywords

with their associated data (symbolic names of literals).

Page 96

BUILT IN FUNCTIONS LIBRARY

mode = 3

is a basic PLA table of type fd.  It contains only the PLA matrix

without any keywords.

mode = 4

is a basic PLA table of type fr.  It contains only the PLA matrix

without any keywords.

mode = 5

is a space separated list of symbolic names of the input literals

that will appear in any of the PLA formats above (Formats 1 through

4) in the same order of appearance as in the PLA matrix (sorted by

their serial number).

4.3 MINIMIZATION

See also: minstat

BOOL will minimize a variable only if you tell it to do so, by

calling the appropriate minimization function.   Some minimization

functions accept additional parameters on the desired minimization

strategy.

Either a built in (internal) or an external minimizer can be used.

The built in minimizer is called through the functions: minimize,

lminimize, and pminimize.  Each of the functions give a different

level of control on the minimization process, although all of them

call the same minimization algorithms.   pminimize gives the highest

control on the minimization process.

The built in minimizer minimizes each boolean variable in the input

list individually.

The function extmin calls an external minimizer.  As a default,

extmin will call the Espresso minimizer that is supplied with BOOL as

a separate executable file.  Unlike the built in minimizer which

minimizes each variable separately, Espresso tries to minimize a

group of variables with product term sharing.

The built in minimizer can be controlled by the parameters:

minimization level and minimization polarity.  An explanation on

these parameters can be found in the tutorial, in the section:

Selecting The Proper Minimization Algorithm.

When calling the minimizer with a minimization level of -1, no

minimization is done, and the call does not have any effect (No

operation).

When specifying a minimization level of 0, no minimization is done,

but all don't cares are collapsed to zero.

Page 97

BUILT IN FUNCTIONS LIBRARY

When minimization level is greater than 0, the built in minimizer

operates according to the following principles:

During the minimization process, the don't care portion is

eliminated, and the original boolean value of each variable is

replaced with a new value which has the following properties:

It is 1 wherever the original value is defined as 1.

It is 0 wherever the original value is defined as 0.

No input literal can be deleted from any product term, and no product

term can be deleted without violating the first two properties.

Comment: A boolean function that has the last property is called

nonredundant.   The solutions of the built in minimizer are always

nonredundant whether an absolute solution was reached or not.

An attempt is made to find a function which has a minimal number of

product terms.  If there are several solutions with the same minimal

number of product terms, an attempt is made to choose the one with

the total minimal number of input literals.

The built in minimizer may or may not reach a solution with minimal

number of product terms and literals.  This depends on the

minimization level.  An indication on the quality of the results is

returned by the minimization functions.   Also, the function minstat

can be used to inquire about a variable after it has been minimized.

No information however is available on results from the external

minimizer.

Side effects:

When a fully specified variable is minimized with the built in

minimizer, (A variable that does not contain a don't care set), the

internal data base is searched for other variables that are exact

copies of that variable, and their value is also updated to reflect

the new minimized representation.   This helps making calculations

with these variables faster.

4.3.1 extmin    Calling an external minimizer

Synopsis:

extmin(a,b,c,...);

Type of parameters a,b,c,...:   any boolean class (except input and

input group).   All parameters must be variables.

Returned value:   none

extmin calls and executes an external minimization program.  All the

parameters, whether scalars or groups, are minimized by an external

minimizer through a protocol defined below.

The values of all the parameters are written in PLA format to a file

named booldat.bi.   The external minimizer is then called with the

Page 98

BUILT IN FUNCTIONS LIBRARY

single command extmin.  A program by that name must exist (And must

be recognized by the operating system).  When called, extmin reads

the file booldat.bi, minimizes it and writes the results, also in PLA

format, to the file booldat.bo, which is then read by Bool.  The

minimized results replace the old values in the input parameters.

The default external minimizer supplied with the Bool package is

Espresso.  For example, under IBM/DOS, there is a batch file by the

name extmin.bat which calls and executes Espresso.

4.3.2 hazard_free   Generating hazard free logic

Synopsis:

hazard_free(n,a,b,c,...);

Type of parameter n:  integer.

Type of parameters a,b,c,...:  any boolean class

The parameter n is required, and must have the value 0 or 1.

The parameters a,b,c,... are optional.  If present, each of a,b,c,...

must represent an input literal or a group of input literals. (e.g.

input, node or flip-flop).  A boolean variable (or group) can also be

used provided that its value (or the value of each of its elements in

case of a group) is a single uninverted input literal.

hazard_free controls the behavior of the minimizer in all subsequent

calls.

hazard_free(1) will turn hazard free minimization on. Any call to any

of the minimizer functions: minimize, lminimize or pminimize,

thereafter will produce minimized hazard free equations (with respect

to all inputs), by adding a minimal set of redundant product terms.

To turn the hazard free option off, execute the call:

hazard_free(0).

To generate minimized logic that is hazard free only with respect to

a set of inputs, you can add a list of inputs (a,b,c,...) to the

parameter list of hazard_free().

The interpretation of the list depends on the value of n:

If n = 1   Minimization results will be hazard free with respect to

all input literals except a,b,c,...

If n = 0   Minimization results will be hazard free only with respect

to input literals a,b,c,...

To turn off the hazard free option, execute the call: hazard_free(0);

Page 99

BUILT IN FUNCTIONS LIBRARY

For more information consult the tutorial on minimization and hazard

free logic starting on page 47.

4.3.3 lminimize    Calling the built in minimizer

Synopsis:

status = lminimize(minlevel,a,b,c,...);

Type of parameter minlevel:   integer.

Type of parameters a,b,c,.. :   any boolean class except input and

input group

Type of returned value:   integer.

Each variable in the parameter list: a,b,c,... is minimized using a

minimization level specified by minlevel, and the default

minimization polarity.

The returned value is as follows:

0 - The minimum number of product terms was not reached in at least

one of the variables.

1 - All variables reached the absolute minimum, which means a minimal

number of product terms.   If several of those exist, the chosen

solution is the one with the minimal number of literals.

2 - all variables reached a minimal number of product terms, but for

at least one variable, it is not guaranteed that the solution has the

minimal number of literals.

If you want to guarantee an absolute minimum number of product terms

and literals, the minimization level minlevel should be 63.  In that

case, you will either get the absolute minimum, or the program will

run out of memory.

4.3.4 minimize    Calling the built in minimizer

Synopsis:

status = minimize(a,b,c,...);

Type of parameters a,b,c,.. :  any boolean class, except input and

input group.

Type of returned value:   integer.

Each variable in the parameter list: a,b,c,... is minimized using the

default minimization level, and the default minimization polarity.

The value returned by minimize is the same as for the function

lminimize.

Page 100

BUILT IN FUNCTIONS LIBRARY

4.3.5 pminimize    Calling the built in minimizer

Synopsis:

status = pminimize(minlevel,minpol,a,b,c,...);

Type of parameter minlevel:   integer.

Type of parameter minpol:   integer.

Type of parameters a,b,c,.. :   any boolean class, except input and

input group.

Type of returned value:   integer.

Each variable in the parameter list: a,b,c,... is minimized using a

minimization level specified by minlevel, and a minimization polarity

specified by minpol.

minpol can have one of the following values:

minpol = 0  - minimize with positive polarity

minpol = 1  - minimize with negative polarity

minpol = 2  - minimize with programmable polarity: try both positive

and negative polarity and choose the better solution.

The value returned by pminimize is the same as for the function

lminimize.

4.4 SIMULATOR

See also: printf, sprintf, printsim.

The built in simulator is accessed by the values statement and by

calls to the built in functions: clockff, eval, printf, sprintf,

printsim.   Control flow is achieved by regular Bool statements such

as function calls and loops.

The function clockff directly controls the simulator, while printf

and printsim are aimed for printing simulation results.  The

functions eval and sprintf enable the user to retrieve simulation

results (as numbers or formatted strings) back into program

variables.

printf, sprintf and printsim are documented in other sections.

4.4.1 clockff    Simulating a clock trigger to flip-flops

Synopsis:

clockff(a,b,c,...);

Type of parameters a,b,c,... :   flip-flop, scalar or group.

Returned value   :   none.

Comment:  All the parameters: a,b,c,...  must be names of variables.

Page 101

BUILT IN FUNCTIONS LIBRARY

Calling the function clockff with a list of names of flip-flops will

cause the simulator to trigger those flip-flops (Simulate a clock

trigger).   The immediate effect is that the simulated value of the

output of each flip-flop is changed to the simulation value present

at its input.   Excluded are all flip-flops which appeared in the

last values statement.  Those are frozen in the state specified by

the values statement.   After the clock is applied, the simulator

continues to propagate the changes through any nodes, (using a unit

delay model to resolve any race conditions), until the network

reaches a stable state.  If a stable state cannot be reached within a

number of iterations specified by the setparam facility, an error

message is generated.

Calling clockff without any parameters will trigger all the flip-

flops in the design.

All flip-flops in the parameter list supplied to clockff are

triggered simultaneously.

4.4.2 eval    Accessing the simulated value of a variable

Synopsis:

s = eval(var);

Type of parameter var : any boolean class.

Type of returned value:   integer.

Evaluates and returns the simulated value of the parameter var.  If

var is a scalar, the returned value is either 0 or 1.  If var is a

group, each bit in the returned value represents the simulated value

of one element in the group.   If the group contains more elements

than the number of bits in an integer, the value of the high order

elements is lost.

4.5 ACCESSING AND MANIPULATING BOOLEAN DATA

This section describes functions which directly access the data

stored in variables.    Some functions just perform some

manipulations and return a value without disturbing the data in the

input parameters, while other functions modify the data in the input

parameters.

The functions gr, ffj, and ffk will typically be used by the designer

to design his circuit.  clr may also be useful in some complex design

cases, especially for iterative networks.  All the other functions in

this subsection, will not be used in a typical circuit design, but

only in specialized functions for generating special output formats.

Page 102

BUILT IN FUNCTIONS LIBRARY

Please note that the data represented by all boolean class variables

is kept as a sum of products.   Unless otherwise noted, built in

functions access the on-set of that representation.

The functions getpt and ptcomp, however, may access the product term

representation of either the on-set, or the off-set of the input

parameter as follows:   If the parameter is passed by value (e.g. it

is an expression), the on-set is always referenced.  If the parameter

is a variable that is passed by reference, then its on-set is

referenced unless the variable was minimized, and the minimizer chose

negative polarity for that variable, in which case the off-set is

used (This includes the case where the minimizer minimized the

variable using programmable polarity algorithm and chose negative

polarity as a result).

4.5.1 clr    Clearing variables to an initial state.

Synopsis:

clr(a,b,c,...);

Type of parameters a,b,c,... :   Any type except input.

Returned value:   none

Each variable in the parameter list is cleared to its virgin state.

The following is a summary of the virgin state of each data type:

Boolean class scalars  -  global don't care.

Fixed size groups  -  all elements global don't care.

Variable size boolean groups  -  one element which is global don't

care.

Integers and integer arrays  -  all zeros.

string variables -  empty string.

4.5.2 ffj and ffk    Converting a flip-flop from D type to JK

Synopsis:

j = ffj(ffname);

k = ffk(ffname);

Type of parameter ffname:   flip-flop, scalar or group

Type of returned value:   boolean, scalar or group depending on

ffname.

The two functions ffj and ffk convert a design with D flip-flops to a

design with JK flip-flops.

In the scalar case, ffname is the name of a flip-flop.  ffj returns

the boolean equation of the J input to an equivalent JK flip-flop,

while ffk returns the boolean equation of the K input.

Page 103

BUILT IN FUNCTIONS LIBRARY

If however ffname is the name of a group of flip-flops, ffj and ffk

return each a group value, where each element represents the J and K

inputs to each of the flip-flops.

For backward compatibility with previous versions, the ffj and ffk

functions can be called also in another form:

j = ffj(Q,D);

k = ffk(Q,D);

D represents the equation of the D input to the D flip-flop ("next

state"),  while Q is the output from the flip-flop ("present state").

In this form of the function, Q and D can have types input and

boolean.

Notice that if X is a name of a flip-flop, then the following

statement:

j = ffj(X);

is equivalent to:

j = ffj( (X), (X:D) );

4.5.3 getpt    Extracting a single product term

Synopsis:

p = getpt(p,b,n);

Type of parameter p : boolean scalar variable.  (used for output)

Type of parameter b : any boolean class scalar.

Type of parameter n : integer.

Type of returned value:   integer.

getpt extracts product term number n from the boolean function

represented by b, and puts it into the output parameter p.  The

previous contents of p are discarded.  After a successful return from

the function, p contains exactly one product term.  The returned

value is 0 for a successful return.  If the number of product terms

in b is less than or equal to n , the value 1 is returned.

getpt does not change the value of b.

NOTES:

1. The boolean function actually accessed by getpt can be either the

on-set or the off-set of b.  Please refer to the explanation at the

top of this subsection.

2. The first product term is counted as number 0.

3. If b is global one, then it is treated as one product term with a

value of global one.

4. If b is global zero, then it is treated as having no product

terms.

Page 104

BUILT IN FUNCTIONS LIBRARY

4.5.4 gr    Generating a group from a list of variables

Synopsis:

gvar = gr(n,a,b,c,...);

Type of parameter n:   integer.

Type of parameters a,b,c,... :   any boolean class or integer

Type of returned value:   boolean group.

The returned value is a group which includes as its elements all the

elements gathered from the variables a,b,c,...

The rightmost parameters are included as the elements with the lower

index, and the leftmost parameters are included as the high elements.

If a parameter is a group, all its elements are included.  Integer

parameters are converted to booleans (They should be either 0 or 1).

The parameter n controls the number of elements in the resulting

group.   If n is greater than 0, the resulting group will have

exactly n elements.  This means that if the total number of scalar

elements in all parameters (scalars and groups) is greater than n,

then only the lower (right) n elements are included in the result,

and the higher (left) elements above n are discarded.  If the total

number of elements in the input parameters are less than n, the high

order elements of the result are assigned the value global zero.

If n equals to 0, the resulting group will contain all the elements

gathered from the input parameters, without any additions or

deletions.

This function is useful for catenating groups to groups, and scalars

to groups, and also for trimming the number of elements in a group to

a desired number.

4.5.5 pass   pass value of parameter

Synopsis:

x = pass(y);

Type of parameter y:   any boolean class scalar.

Type of returned value:   boolean.

Comment:  y must be a variable.

pass acts as a no-operation function, by simply returning its

argument.  If y is a name of a variable, then using y or pass(y) in

an expression is equivalent.

The pass function is helpful when a variables cannot be accessed

directly because only its Variable Identification Number is known.

For example, if stat represents the Variable Identification Number of

a variable, then the variable can be accessed through the use of the

$v macro as in:  pass($v(stat)).

Page 105

BUILT IN FUNCTIONS LIBRARY

4.5.6 one    Extracting the on-set of a boolean

Synopsis:

var = one(b);

Type of parameter b:   any boolean class scalar.

Type of returned value:   boolean.

Returns the on-set of b which is defined as having the value 1

wherever the function represented by b has a value 1, and a value of

zero elsewhere.

If b is fully specified (has an empty don't care set), then by

definition, the returned value equals to b.

4.5.7 ptcomp    Comparing two product terms

Synopsis:

i = ptcomp(p1,p2);

Type of parameters p1, p2:   any boolean class scalar.

Type of returned value:   integer.

Compares two product terms according to the same key by which product

terms are sorted internally.  The first product term of p1 is

compared against the first product term of p2.

Returned value is:

1    p1 comes before p2.

0    p1 is equal to p2.

-1    p1 comes after p2.

NOTES:

1. The boolean function actually accessed by ptcomp can be either the

on-set or the off-set of each of p1 and p2.  Please refer to the

explanation at the top of this subsection.

2. Only the first product term in p1 and the first product term in p2

are used in the comparison.  All the other product terms are ignored.

3. The product terms in a boolean function are sorted according to an

internal representation:  product term no. 0 comes before product

term no. 1 (this is also the order by which they are printed).

4. If any of the operands p1 or p2 is global one or global zero, the

rules are: Global one comes before any other product term, and global

zero comes after any product term.

Page 106

BUILT IN FUNCTIONS LIBRARY

4.5.8 valueof   Retrieve value of a boolean class variables

Synopsis:

x = valueof(y);

Type of parameter y:   any boolean class.

Type of returned value:   boolean or boolean group depending on

input.

Comment: y must be a variable.

The returned value will be a scalar boolean if y is scalar.   If y is

a group, the returned value will be a boolean group with the same

number of elements as in y.

valueof retrieves the value that is associated with a variable.   If

y is an input, boolean or output variable, using valueof(y) in an

expression is equivalent to using the name y by itself.

However, if y is a node or flip-flop, then y and valueof(y) have

different meaning.  When y appears by itself in an expression, it

represents just an input literal, while valueof(y) represents the

value that was assigned to y during execution of previous statements.

(Which for a flip-flop, is the boolean function representing the next

state of the flip-flop).

Note that although the explanations above refer to y as a scalar, y

may also be a group.

4.5.9 zero    Extracting the off-set of a boolean

Synopsis:

var = zero(b);

Type of parameter b:   any boolean class scalar.

Type of returned value:   boolean.

Returns the off-set of b which is defined as having the value 1

wherever the function represented by b has a value 0, and a value of

zero elsewhere.

If b is a fully specified (has an empty don't care set), then by

definition, the returned value equals to the logical inverse of b.

4.6 ACCESSING INFORMATION ON VARIABLES

This section describes functions which return information on

variables, that is not directly related to the Status Word or the

attributes of a variable.

Page 107

BUILT IN FUNCTIONS LIBRARY

This section does not contain functions which directly manipulate the

data itself.  These are described in the section: Accessing And

Manipulating Boolean Data.

Please note that the data represented by all boolean class variables

is kept as a sum of products.   Unless otherwise noted, built in

functions access the on-set of that representation.

The functions lit, litn, lser, lstat and ptn, however, may access the

product term representation of either the on-set, or the off-set of

the input parameter as follows:   If the parameter is passed by value

(e.g. it is an expression), the on-set is always referenced.  If the

parameter is a variable that is passed by reference, then its on-set

is referenced unless the variable was minimized, and the minimizer

chose negative polarity for that variable, in which case the off-set

is used (This includes the case where the minimizer minimized the

variable using programmable polarity algorithm and chose negative

polarity as a result).

4.6.1 getvarname    Accessing the name of a variable as a string

Synopsis:

s = getvarname(var);

Type of parameter var:   any type  (must be a variable).

Type of returned value:   string

getvarname returns the original name of the variable var as a string.

4.6.2 isdc    Check if a boolean is global don't care

Synopsis:

n = isdc(var);

Type of parameter var  :   any boolean class scalar.

Type of returned value :   integer

Returns 1 if var has a value of global don't care, 0 otherwise.

4.6.3 isone    Check if a boolean is global one

Synopsis:

n = isone(var);

Type of parameter var  :   any boolean class scalar.

Type of returned value :   integer

Returns 1 if var has a value of global one,  otherwise it returns 0.

Page 108

BUILT IN FUNCTIONS LIBRARY

4.6.4 isused_input    Check for unused input literals

Synopsis:

n = isused_input(ivar);

Type of parameter ivar  :  input, node, or flip-flop

Type of returned value  :  integer

Checks if the input literal specified by ivar is used as input by any

other network element (node, flip-flop or output).

A returned value of 1 means that the literal is used.

A returned value of 0 indicates that the literal is not used by any

network element (Or in other words, it does not appear in any boolean

equation of any node, flip-flop or output variable).

4.6.5 iszero    Check if a boolean is global zero

Synopsis:

n = iszero(var);

Type of parameter var  :   any boolean class scalar.

Type of returned value :   integer

Returns 1 if var has a value of global zero, otherwise it returns 0.

4.6.6 lit    Number of input literals in a product term

Synopsis:

n = lit(var);

Type of parameter var:   any boolean class scalar.

Type of returned value:   integer.

Returns the number of input literals that compose the first product

term of the boolean function represented by var.

NOTE:

1. The boolean function actually accessed by lit can be either the

on-set or the off-set of var.  Please refer to the explanation at the

top of this subsection.

4.6.7 litn    Total number of input literals

Synopsis:

n = litn(a,b,c,...);

Type of parameters a,b,c,... :   any boolean class.

Type of returned value:  integer.

Page 109

BUILT IN FUNCTIONS LIBRARY

Returns the total number of input literals appearing in different

product terms in the input parameters.

All scalar parameters and all elements of all group parameters are

processed.   If the same product term appears more than once, then

the literals appearing in that product term are counted only once.

This function provides a quick (although inaccurate) way to estimate

the complexity of a design implemented by random logic (gates).

NOTES:

1. The boolean function actually accessed by litn, for any of the

input parameters, can be either the on-set or the off-set.  Please

refer to the explanation at the top of this subsection.

2. Global zero and global one are considered as having zero literals.

4.6.8 lser    Serial number of an input literal

Synopsis:

n = lser(b,m);

Type of parameter b:   any boolean class scalar

Type of parameter m:   integer.

Type of returned value:   integer

Returns the serial number of input literal number m in the first

product term of the boolean function represented by b.  If m is

greater or equal to the number of input literals in the first product

term of b, or if m is negative, then a -1 is returned.   By

definition, the serial number of an input literal is its index in the

universal group univ.igroup.

NOTES:

1. The boolean function actually accessed by lser can be either the

on-set or the off-set of b.  Please refer to the explanation at the

top of this subsection.

2. The first literal is counted as literal no. 0 .

3. Global zero and global one are considered as having zero literals.

4.6.9 lstat    Status of an input literal in a product term

Synopsis:

n = lstat(b,sernum);

Type of parameter b:   boolean or input.

Type of parameter sernum:   integer.

Type of returned value:   integer.

lstat checks the first product term of the boolean function b for the

appearance of an input literal whose serial number is sernum.  The

returned value is as follows:

Page 110

BUILT IN FUNCTIONS LIBRARY

0 - the input literal appears in an inverted form.

1 - the input literal appears in an uninverted form.

2 - the input literal does not appear at all.

NOTES:

1. The boolean function actually accessed by lstat can be either the

on-set or the off-set of b.  Please refer to the explanation at the

top of this subsection.

2. sernum must be in the proper range: less than the number of input

literals in the program, and greater or equal to zero.

4.6.10 minstat    Minimization status of a variable

Synopsis:

n = minstat(b);

Type of parameter b:   any boolean class scalar.

Type of returned value:   integer

minstat returns the minimization status of b.

The returned value is as follows:

n = -2    The variable was minimized and a solution with the minimal

number of product terms was reached.

n = -1    The variable was minimized and a solution with the minimal

number of product terms was reached.  If several solutions with

minimal number of product terms exist, this solution is also

guaranteed to be the one with the total minimal number of input

literals.

n = 0     The variable was not minimized at all.

63 > n > 0  The variable was minimized using a minimization level of

n, but the solution is not guarantied to have a minimal number of

product terms.

NOTE:

The information returned by this function is relevant only for

variables which were minimized with the built in minimizer.   For

variables which were minimized by an external minimizer (by calling

extmin) the function returns 0.

4.6.11 ptn    Total number of product terms

Synopsis:

n = ptn(a,b,c,...);

Type of parameters a,b,c,... :   any boolean class.

Type of returned value:   integer

Page 111

BUILT IN FUNCTIONS LIBRARY

Returns the total number of different product terms in the input

parameters.  If the same product term appears in more than one

variable, it is counted only once.  This function provides a

convenient way to estimate the complexity of a design which is

implemented by a PLA with product terms sharing.

NOTES:

1. The boolean function actually accessed by ptn, for any of the

input parameters, can be either the on-set or the off-set.  Please

refer to the explanation at the top of this subsection.

2. Global zero is considered as having zero product terms.

3. Global one is considered as having one product term.

4.6.12 size    Number of elements in a group or array

Synopsis:

n = size(g);

Type of parameter g :   any boolean class group or integer array.

Type of returned value :   integer.

size returns the number of elements in g (Size of g).

4.7 ACCESSING INFORMATION ON STATUS WORD AND ATTRIBUTES OF VARIABLES

Most of the functions described below make use of the Status Word

which is described in the section:  Status Word Of A Variable.

The functions get_status, get_attrib_str,  get_attrib_status, and

get_attrib_num retrieve Status Word and attribute information of a

single network element.

The functions get_list_status, get_literals_status,

add_literals_status, add_dependents_status, and add_status_by_attrib

give the ability to process attribute and Status Word information of

a set of network elements by manipulating a list of Status Words

formatted in an integer array.   The integer array contains in

element number 0 (First element) the number of entries in the list,

and all other elements are Status Words.   If the first element in

the array contains 0, it means that the list is empty.

A list of Status Words formatted as above is called Status Word List.

Since each Status Word represents a single scalar network element,

the Status Word List represents a collection of scalar network

elements.  Integer arrays can be passed as parameters to functions,

therefore, the Status Word List gives a convenient way to bundle and

process together dynamically, under the same symbolic name, network

elements of different types: inputs, nodes, outputs and flip-flops.

Page 112

BUILT IN FUNCTIONS LIBRARY

The Status Word List is used almost exclusively for generating

special output formats.

The functions general_attrib_info, general_attrib_str and

general_attrib_num return general information about an attribute,

that is not related to any specific network element, but only to how

the attribute itself was defined (e.g. in the ff_attrib statement).

4.7.1 add_dependents_status   Append to a Status Word List

Synopsis:

add_dependents_status(IB,IA,attname1,attname2,...);

Type of parameter IB:  integer array (used for output).

Type of parameter IA:  integer array.

Type of parameters attname1, attname2,...:   string

Returned value:   none.

IA and IB contain Status Word Lists.

For explanations on the Status Word List, see explanations at the

beginning of this subsection.

The function scans the list in IA.  For each of the network elements

that are represented in the list, the function retrieves into IB the

Status words of its 'dependents' (as defined below), with regards to

the attributes attname1, attname2,...

A dependent of a variable X with regard to an attribute att is simply

the variable pointed to by the attribute att.   (Where att is a

Variable Attribute).

In order for this definition to be valid, the string variables

attname1, attname2,... must contain names of attributes of type:

Variable Attribute.  If any of them contains a name of an attribute

of a different type, or if an attribute by that name does not exist,

it is ignored.

Any previous list stored in IB is not overwritten!  It is assumed

that upon entry to the function, IB[0] contains the number of entries

in the previous list, and the new list is appended to the end of that

list.  Upon exit from the function, the value in IB[0] will reflect

the size of the combined list.    IA and IB may even point to the

same integer array.

Status Words of variables that already appear in IA or IB are not

rewritten into IB.  Also, multiple occurrences will generate only one

entry in IB.

If no attributes are specified (meaning, the function is called only

with the two parameters: IB, IA), then all existing attributes of

type Variable Attribute are scanned.

Page 113

BUILT IN FUNCTIONS LIBRARY

Note that in order to create a new list in IB you have to reset IB[0]

to zero before calling add_dependents_status.

4.7.2 add_literals_status   Append to a Status Word List

Synopsis:

add_literals_status(IB,IA);

Type of parameter IB:  integer array (used for output).

Type of parameter IA:  integer array.

Returned value:   none.

IA and IB contain Status Word Lists.

For explanations on the Status Word List, see explanations at the

beginning of this subsection.

The function scans the list in IA and checks the value (which is a

boolean function) of each network element represented in the list.

The Status Words of all the input literals that appear in that value

are retrieves into IB (See exceptions below).   By definition, input

literals are either inputs, nodes or flip-flops.

Any previous list stored in IB is not overwritten!  It is assumed

that upon entry to the function, IB[0] contains the number of entries

in the previous list, and the new list is appended to the end of that

list.  Upon exit from the function, the value in IB[0] will reflect

the size of the combined list.    IA and IB may even point to the

same integer array.

Status Words of variables that already appear in IA or IB are not

rewritten into IB.  Also, multiple occurrences will generate only one

entry in IB.

Note that in order to create a new list in IB you have to reset IB[0]

to zero before calling add_literals_status.

4.7.3 add_status_by_attrib   Append to a Status Word List

Synopsis:

add_status_by_attrib(IB,IA,attr);

Type of parameter IB:  integer array (used for output).

Type of parameter IA:  integer array.

Type of parameter attr:  string.

Returned value:   none.

IA and IB contain Status Word Lists.

For explanations on the Status Word List, see explanations at the

beginning of this subsection.

Page 114

BUILT IN FUNCTIONS LIBRARY

The function copies from IA to IB all status words of variables that

were declared with the attribute attr.

Any previous list stored in IB is not overwritten!  It is assumed

that upon entry to the function, IB[0] contains the number of entries

in the previous list, and the new list is appended to the end of that

list.  Upon exit from the function, the value in IB[0] will reflect

the size of the combined list.

Status Words of variables that already appear in IB are not rewritten

into IB.

Note that in order to create a new list in IB you have to reset IB[0]

to zero before calling the function.

4.7.4 get_attrib_num   Get numeric value of an attribute

Synopsis:

n = get_attrib_num(attname,var);

Type of parameter attname:   string.

Type of parameter var:   any scalar network element.

Type of returned value:   integer.

Returns the value of an attribute of a variable coded as an integer

number.  The returned value depends on the type of the attribute as

specified below.

attname is a string containing the name of the attribute, and var is

the name of the variable whose attribute is being tested.

For Enumerated Type Attribute the function returns the numeric value

associated with the attribute value, which is actually the index in

the enumerated list of that attribute.

For String Attribute the returned value is always 0.

For Numeric Attribute, the function simply returns the value of the

attribute.

For Variable Attribute, the function returns the Status Word of the

variable pointed to by the attribute.

If an attribute as specified by attname is not declared for the

variable var, the function returns the value 0.

Page 115

BUILT IN FUNCTIONS LIBRARY

4.7.5 get_attrib_status   Status of an attribute

Synopsis:

n = get_attrib_status(attname,var);

Type of parameter attname:   string.

Type of parameter var:   any scalar network element.

Type of returned value:   integer.

Returns the status of an attribute for a specific variable.

attname is a string containing the name of the attribute, and var is

the name of the variable whose attribute is being tested.

The function returns 1 if var was declared with the specified

attribute and 0 if not.

4.7.6 get_attrib_str   Get string value of an attribute

Synopsis:

str = get_attrib_str(attname,var);

Type of parameter attname:   string.

Type of parameter var:   any scalar network element.

Type of returned value:   string.

Returns the value of an attribute of a variable coded as a string.

The returned value depends on the type of the attribute as specified

below.

attname is a string containing the name of the attribute, and var is

the name of the variable whose attribute is being tested.

For Enumerated Type Attribute, and for String Attribute, the function

simply returns the string representing the value of the attribute.

For Numeric Attribute, the function returns the numeric value coded

as a decimal string.

For Variable Attribute, the function returns the name of the variable

pointed to by the attribute, as a string.

If an attribute as specified by attname is not declared for the

variable var, an empty string is returned.

Page 116

BUILT IN FUNCTIONS LIBRARY

4.7.7 get_list_status   Prepare a Status Word List

Synopsis:

get_list_status(IA,a,b,c,...);

Type of parameter IA:   integer array. (Used for output)

Type of parameters a,b,c,... :   any scalar or group network

elements.

Returned value:   none.

This function retrieves the Status Words from the list of variables

a,b,c,... and formats them in the integer array IA, as a Status Word

List.   The Status Words are retrieved from all scalar variables and

all elements of groups.

For explanations on the Status Word List, see explanations at the

beginning of this subsection.

Upon successful return from the function, IA[0]  contains the total

number of different Status Words written into IA.  The Status Words

are each written into successive locations in IA Starting at IA[1].

The status Words are written according to the a predetermined order:

First, appear Status Words of all input variables, then output

variables, then node variables and last flip-flop variables.

The Status Words written into IA are all unique, which means that

multiple occurrences of the same variable in the input list will

produce only one entry in IA.

4.7.8 get_literals_status   Prepare a Status Word List

Synopsis:

get_literals_status(IA,a,b,c,...);

Type of parameter IA:   integer array. (Used for output)

Type of parameters a,b,c,... :   any boolean class.

Returned value:   none.

The function scans the variables a,b,c,... and tests the value (which

is a boolean function) of each variable.   The Status Words of all

the input literals that appear in that value are retrieves into IA as

a Status Word List.   By definition, input literals are either

inputs, nodes or flip-flops.

For explanations on the Status Word List, see explanations at the

beginning of this subsection.

Upon successful return, IA will contain a list of unique Status Words

of input literals starting at IA[1], and sorted by their serial

number.  IA[0] will contain the total number of Status Words that

were retrieved into IA.

Page 117

BUILT IN FUNCTIONS LIBRARY

4.7.9 get_status   Get Status Word of a Variable

Synopsis:

n = get_status(var);

Type of parameter var:   any boolean class scalar.

Type of returned value:   integer.

Returns the Status Word of var.

4.7.10 general_attrib_info   Get general information on an attribute

Synopsis:

n = general_attrib_info(attname,vartype);

Type of parameter attname:   string.

Type of parameter vartype:   integer.

Type of returned value:   integer

This function returns general information on an attribute with a

specified name and variable type.

attname is a string containing the name of a valid attribute, and

vartype contains a number between 0 and 3 specifying the type of

variables on which the attribute is defined, as follows:

vartype = 0       input

vartype = 1       output

vartype = 2       node

vartype = 3       flip-flop

(e.g.  vartype = 0  indicates that the attribute was defined with

inp_attrib statement)

The function returns the following information on that attribute:

n = -4      No such attribute with the specified type was defined.

n = -3      The specified attribute has type String Attribute

n = -2      The specified attribute has type Variable Attribute

n = -1      The specified attribute has type Numeric Attribute

n >  0      The specified attribute is an Enumerated Type Attribute.

n is the number of different values (strings) in the enumeration

list.

Page 118

BUILT IN FUNCTIONS LIBRARY

4.7.11 general_attrib_num   Get general information on an attribute

Synopsis:

n = general_attrib_num(attname,vartype,str);

Type of parameter attname:   string.

Type of parameter vartype:   integer.

Type of parameter str:   string.

Type of returned value:   integer

This function returns general information on an Enumerated Type

Attribute with a specified name and variable type.   It is actually

the inverse of the function general_attrib_str.

attname is a string containing the name of a valid attribute, and

vartype contains a number between 0 and 3 specifying the type of

variables on which the attribute is defined, as follows:

vartype = 0       input

vartype = 1       output

vartype = 2       node

vartype = 3       flip-flop

(e.g.  vartype = 0  indicates that the attribute was defined with

inp_attrib statement).

str specifies one of the values in the enumeration list.  The

function returns the index of that value in the enumeration list.

If the specified attribute (attname) is not defined, or the specified

value (str) does not exist, a value of 0 is returned.

For example, if an attribute by the name libname was defined by:

ff_attrib libname  "ff3x0" "ff3x1"  "ff3x2";

Then the following statement:

n = general_attrib_num("libname",3,"ff3x2");

Will write the value 2 into the variable n (which is the index of

"ff3x2" in the list).

4.7.12 general_attrib_str   Get general information on an attribute

Synopsis:

str = general_attrib_str(attname,vartype,num);

Type of parameter attname:   string.

Type of parameter vartype:   integer.

Type of parameter num:   integer.

Type of returned value:   string

Page 119

BUILT IN FUNCTIONS LIBRARY

This function returns general information on an Enumerated Type

Attribute with a specified name and variable type.

attname is a string containing the name of a valid attribute, and

vartype contains a number between 0 and 3 specifying the type of

variables on which the attribute is defined, as follows:

vartype = 0       input

vartype = 1       output

vartype = 2       node

vartype = 3       flip-flop

(e.g.  vartype = 0  indicates that the attribute was defined with

inp_attrib statement).

num specifies the index of the value (And must therefore be less than

the number of values in the enumeration list).  The function returns

a string representing that value.

An empty string is returned if no valid value can be returned for any

reason.

For example, if an attribute by the name libname was defined by:

ff_attrib libname  "ff3x0" "ff3x1"  "ff3x2";

Then the following statement:

str = general_attrib_str("libname",3,1);

Will write the string "ff3x1" into the variable str.

4.8 STRING HANDLING

The functions in this section offer a modest way of manipulating

strings with a C-like functionality.

4.8.1 sprintf   Formatted conversion

Synopsis:

sprintf(str,format_spec,a,b,c,...);

Type of parameter str:   string  (used for output)

Type of parameter format_spec:   string

Type of parameters a,b,c,... :   any type except integer array.

Returned value: none.

sprintf offers almost the same functionality as printf but instead of

printing the results into a file, the results are formatted into the

Page 120

BUILT IN FUNCTIONS LIBRARY

string variable str.  For a detailed description refer to the

explanations on the function printf.

4.8.2 strcat   Catenate two strings

Synopsis:

strcat(str1,str2);

Type of parameter str1:   string  (used for output).

Type of parameter str2:   string.

Returned value:   none

Comment:   str1 must be a variable.

The function catenates the string specified by str2 to the end of the

string that already exists in str1.

4.8.3 strcmp   Compare two strings

Synopsis:

n = strcmp(str1,str2);

Type of parameter str1, str2:   string.

Type of returned value:   integer.

Compares the strings str1 and str2 according to their ASCII values

and returns an integer value as follows:

n < 0     str1 < str2

n = 0     str1 == str2   (The two strings are identical)

n > 0     str1 > str2

4.8.4 strlen   Get length of a string

Synopsis:

n = strlen(str);

Type of parameter str:   string.

Type of returned value:   integer.

The function returns the number of characters in the string str.

Page 121

BUILT IN FUNCTIONS LIBRARY

4.9 MISCELLANEOUS

4.9.1 exit   Abnormal termination of a program

Synopsis:

exit(code);

Type of parameter code:   integer.

Returned value:   None.

Causes an unconditional immediate termination of the program.  code

is sent to the operating system (program termination code).

This function is intended for terminating a program under abnormal

conditions (e.g. when the program detects an error).

The exit function is not intended for normal termination.   Under

normal conditions, a program terminates when control is passed to the

end statement.

Please note that according to the Unix and DOS conventions a

termination code of 0 means normal termination, while any other value

indicates an error condition.  The termination code can later be

tested by the operating system.

4.10 OLD FUNCTIONS

The functions in this section exist for backward compatibility with

previous versions of BOOL.   However, their functionality can be

achieved by using other built in functions.

4.10.1 length   Get length of a name of a variable

Synopsis:

n = length(var);

Type of parameter var:   any type.

Type of returned value:   integer.

Returns the number of characters which compose the original name of

the variable var.   This function is equivalent to the following:

n = strlen(getvarname(var));

Page 122

BUILT IN FUNCTIONS LIBRARY

4.10.2 printvarname    Print name of a variable

Synopsis:

printvarname(filenum,var);

Type of parameter filenum:   integer.

Type of parameter var:   any type.

Returned value:   none.

Comments:  var must be a variable.  An expression is not acceptable.

This function prints the name of the variable var to the output file

specified by the file number filenum.  The original name of the

variable is used.  This function is equivalent to the following:

printf(filenum,"%s",getvarname(var));

Page 123

CHAPTER 5: ERROR AND WARNING MESSAGES

5.1 GENERAL INFORMATION ON ERROR AND WARNING MESSAGES

The compiler can generate several types of error or warning messages.

The first type is a fatal error message which causes immediate

termination of the program.  This can happen for example, if the

input file does not exist, or if there is not enough memory.  These

errors are not related to syntax or logical errors in the program and

therefore the error message includes only a brief description of the

problem.

The second type are issued by the preprocessor (e.g. nonexistent

include file).  The preprocessor issues a message with a line number

specification, and if possible, continues to scan the input file.  If

fatal errors are discovered, the program terminates at the end of the

preprocessor phase.

The third type are issued during the first pass (Which is actually

the second phase).  During this phase, the input is scanned for

syntax errors.  Any error found in this phase is reported with a file

name, line number and a column number, showing the exact location of

the error.  If fatal errors are found in this phase, the compiler

continues to scan the input for more errors, but does not proceed

into the next phase, which is the execution phase.  A simple error

recovery mechanism is used here: If an error is found in a statement,

all the rest of the statement is ignored.

The fourth type of errors are fatal errors discovered during the

execution phase (Second pass).  These errors are typically caused by

illegal values of integer variables, such as divide by zero.  These

type of errors cause immediate termination of the program, with an

error message which includes file name, line number and column

number, showing the exact location of the error.

The fifth type are warning messages.  These warn the user about

questionable situations such as values conflict in an assignment

statement.  Warnings are also reported with the exact location,

including file name, line number and column number.

The sixth type are errors detected when reading input data files, and

are caused by input which does not conform to the PLA format (which

is expected in this case by the built in functions readpla and

extmin).  This type is fatal and causes immediate termination.

There are also internal errors which are discussed later.

Page 124

ERROR AND WARNING MESSAGES

In the following sections we distinguish only between three main

categories: warnings, data file errors and all other errors,

regardless of the phase in which they are detected.   The messages

are sorted alphabetically, which makes it easier to locate them in

the text.

The compiler keeps track of the exact location of each statement in

the input.  Since the program may exist on several different files

(thanks to the #include directive), the location contains not only

the line and column number, but also the file name.  Several errors

may be detected in the same file.  Therefore the file name is

reported only once for each group of errors from the same file.

The column number report assumes that any character, except tab and

end of line, takes exactly one column.   Horizontal tab locations are

assumed at 8 columns increments.  The first character in the line

(immediately following the end of line character) is called column

number 1.  The first line in the file is called line number 1.

If an error is detected in a string which was substituted as a result

of a previous #define directive, an attempt is made to point out the

two locations:  the location of the error in the definition of the

string, and the location of the symbol which was substituted by the

string.

Note that errors detected in input data files (by readpla or extmin)

are reported with the name of the data file, and the location in the

data file where the error was detected.  (The source line is not

indicated in this case).

All error reports are sent to the list file which is the standard

output device unless declared otherwise in the command line.

If warnings or errors are detected, the single message:

*** warnings detected ***

or

*** errors detected ***

is sent to the screen whether the list file is directed to the screen

or not.

Example of an error report:

*** file:  FILE1

** error ** line: 20  col:  5  unrecognizable statement

*** file:  FILE2

** error ** line:  3  col:  7  illegal use of keyword

** error ** line:  9  col:  1  duplicate parameter name

In this example an error was detected in FILE1 and then two other

errors were detected in FILE2.

Page 125

ERROR AND WARNING MESSAGES

The compiler performs extensive checking on the consistency of

internal data structures.  If a problem is found, the compiler aborts

immediately with a message:

** Internal error no. <xxxx> ** execution aborted **

This type of message indicates an internal bug in the compiler, or a

machine malfunction.    Please notify us if you get this message, and

you think the problem is not hardware related.

5.2 WARNING MESSAGES

Warnings are given in the format:

** warning ** line: <xxx>  col: <yyy>  <message>

<message> is a short text explaining the problem.

<xxx>  is the line number where the error was found.

<yyy>  is the column number where the error was found.

Some warning messages may not produce column number information.

Following is a list of the warning messages that can be generated by

the compiler.  Only <message> is printed here.

clockff statement evaluated under a logical condition

An attempt was made to execute a clockff statement under a condition

that depends on values of input literals (nested inside an if, else

or .switch statement).   Although this is syntactically permitted, it

does not make much sense, Since clockff is a simulator statement

while conditions that depend on values of input literals are used

only to define logic.   Therefore it is most likely that this message

indicates an unintentional error.   The conditions that are used

during simulation would normally be based only on integer variables.

Page 126

ERROR AND WARNING MESSAGES

element no. <nn> : new value overrides old value

In a simple assignment to a group, using an assignment statement or a

JUMP or STAY statements, a conflict was detected between the old and

the newly assigned value of element no. <nn> of the group.  This is

only a warning and the new value overrides the old value.  If this is

the desired operation, you can avoid the warning by changing the

statement to an overriding statement:

Instead of = use := , instead of JUMP use XJUMP and instead of STAY

use XSTAY.

int to bool conversion - integer value is not 0 or 1

When converting an integer value to a boolean, the compiler expects

the integer value to be either 0 or 1.   Any other value is treated

the same as the value 1, but will generate this warning message.

new value overrides old value

In a simple assignment statement to a boolean variable, a conflict

was detected between the old and the new value.  This is only a

warning and the new value overrides the old value.  If this is the

desired operation, you can avoid the warning by using the :=

assignment instead of the = assignment.

redefinition of symbol

This warning message is produced by the preprocessor when a #define

directive redefines a symbol that was already defined earlier by a

previous #define.  To avoid the warning message you should use #undef

to undefine the symbol before you redefine it.

simulated value of variable <name> is don't care

This message indicates that the simulated value of the variable

<name> is don't care.   In most cases, don't cares should not happen

during simulation to any network element.  However, since in some

cases the value of some signals can indeed be ignored, this warning

message can be turned on and off by the setparam statement.

Page 127

ERROR AND WARNING MESSAGES

values statement evaluated under a logical condition

An attempt was made to execute a values statement under a condition

that depends on values of input literals (nested inside an if, else

or .switch statement).   Although this is syntactically permitted, it

does not make much sense, Since values is a simulator statement while

conditions that depend on values of input literals are used only to

define logic.   Therefore it is most likely that this message

indicates an unintentional error.   The conditions that are used

during simulation would normally be based only on integer variables.

5.3 DATA FILE ERROR MESSAGES

This class of messages appears in the following format:

file <filename>:

**  data file error  **  <message>  <location>

This form is used to report problems when reading input data files,

by the built in functions readpla and extmin.  Since these functions

expect the data in the file to be in PLA format, any deviation from

it is reported.

<message> is a short text which explains briefly the problem.

<location> is a location indicator which indicates the location in

the data file. (not in the source file.)  The location indicator may

or may not appear, depending on the type of problem, and may contain

either line number or line and column number.

The different error messages are listed below

could not find .i or .o parameter until line=<nn>

The indicated parameters were not found until line number <nn> which

is assumed to be the first data line.

extra characters

The indicated line contains more characters than are needed to define

one product term.

Page 128

ERROR AND WARNING MESSAGES

.i parameter has illegal value

The number specified with the .i (number of inputs) parameter is too

large or not strictly positive.

illegal character

An illegal character was detected at the indicated location.

incorrect number of symbols

The number of symbols found after the .ilb or .ob keywords does not

match the numbers specified by the .i or .o parameters.

missing characters

The indicated line does not contain enough characters to construct a

product term.

.o parameter has illegal value

The number specified with the .o (number of outputs) parameter is too

large or not strictly positive.

on-set/off-set conflict, output element no. <nn>

A problem was detected in the data definition of output element

number <nn>.  The on-set and the off-set as specified in the file

have common minterms.

parameter <xxx> not supported

A parameter with the name <xxx> (which is not supported by BOOL) was

detected in the input data file.

.type parameter has illegal value

The character string following the keyword .type is not one of the

strings recognized by BOOL.

Page 129

ERROR AND WARNING MESSAGES

5.4 OTHER ERROR MESSAGES

All other error messages have the following form:

**  error  ** line: <xxx>  col: <yyy>  <message>

<message> is a short text which explains briefly the problem.

<xxx>  is the line number where the error was found.

<yyy>  is the column number where the error was found.

In some messages, the column number information, or both column

number and line number information is missing.  This depends mainly

on the type of the problem and the phase in which the problem was

discovered.

The following is a list of all the error messages with a short

explanation on each message.  The messages are sorted alphabetically

and only the <message> part is printed here.

<char>  expected

or:<char1> or <char2> expected

The compiler expected to find the specified character(s).

a label is expected (CASE or @) inside a .switch statement

The first statement inside a .switch block (immediately following the

opening '{' ) must be preceded by a label.  (A label is the keyword

case (or the character @) followed by an integer expression and

ending with a colon).

accepted values are only YES or NO

The parameter specified in a setparam statement may accept only two

possible values which are the strings: YES or NO.

attribute <attr> is already defined. cannot redefine

The specified attribute <attr> was already defined for the same type

of variable.  You may not define the same attribute more than once

for each type of variable.

Page 130

ERROR AND WARNING MESSAGES

boolean type expected

A boolean type expression or variable is needed.

cannot find end of comment or string up to end of file

The ending " or */ of a string or comment was not found.

constant integer value expected

The indicated expression may not contain names of variables or

function calls.  It may contain only numbers and operators.

control string does not match no. of parameters

The number of conversion specifiers in the format specifier of a

printf or sprintf functions, is different from the number of

parameters to be printed.

declaration of a non-existing parameter

The function header includes a declaration of a parameter which does

not appear in the list of parameters.

destination array size is too small

This message is reported by built in functions that write their

results into an integer array (e.g. get_list_status), if the size of

the array is not large enough.  You have to increase the size of the

array (number of elements).

different group sizes

The .switch statement, when specified with two groups, as in

.switch(in -> out)  requires that the number of elements in both

groups, in and out should be the same.

Page 131

ERROR AND WARNING MESSAGES

divide overflow

An attempt was made to divide a number by zero, or to divide the

maximal possible negative number by -1.

duplicate parameter name

The same name appears more than once in the parameters list.

duplicate symbol name

An attempt to declare a symbol which was already declared by a

previous declaration.  Check previous declarations for a symbol with

the same name.

element no. <nnn> of parameter does not represent an input

The indicated variable must be either an input literal or a boolean

value which represents an input literal (If written as a boolean

expression, it should contain only one non inverted literal).

#else not associated with #if

The preprocessor encountered an #else directive but could not match

it to any previous #if directive.

#endif not associated with #if

The preprocessor encountered an #endif directive but could not match

it to any previous #if directive.

error trying to load and execute extmin

The built in function extmin failed to load and execute the system

command extmin.

Check that an executable file with the proper name exists.

error trying to open booldat.bi for output

The built in function extmin failed to open for output the file

booldat.bi .

Page 132

ERROR AND WARNING MESSAGES

This can happen for example if you have a write protected file (or a

subdirectory) with that name.

error trying to open booldat.bo for input

This indicates that the built in function extmin failed to open the

data file booldat.bo for input, after calling an external minimizer.

This can happen if the external minimizer failed to create this file.

error trying to open data file <name> for input

The compiler failed to open the file with the indicated name for

input.  Check that a file by that name exists.

error trying to open file <name>

The compiler failed to open for input the main source file as

specified in the command line.

error trying to open file <name> for output

The compiler failed to open for output the file with the indicated

name.  This may happen if you already have a write protected file or

a directory with that name.

file number has an illegal value

The file number, specified in one of the built in functions (e.g.

printf) is negative or too large.

file number <n> is not open

An attempt was made to read from a file number that was not open.

You must open a file for input before you can read from it either

through the command line, or through the function open_inpfile.

function body empty

No executable statements were found in the body of a user defined

function.

Page 133

ERROR AND WARNING MESSAGES

function reference to a non function symbol

A symbol was accessed illegally in the form of a function call.  If

fname is not a keyword, then the form fname(...) can be used only for

calling functions.  If that is your intention, check that fname is

written correctly, and that a function by that name is defined.

group type expected

The .switch statement requires group variables as parameters.

illegal assignment

The type of variable in the assignment is illegal (integer array, or

input) , or its type does not match the expression assigned to it.

(For example, a value of type group cannot be assigned to a scalar

boolean).

illegal character constant

The indicated character constant is illegal.  Check for balanced

quotes and correct number of characters.

illegal conversion specifier '<c>'

In a printf or sprintf statement, the character appearing after the %

is not recognized as a legal conversion specifier.

illegal declaration statement

This usually indicates that the syntax used in the declaration

statement is not legal in that context.  For example, trying to

define attributes for formal parameters of a function.

illegal definition of attribute value

In an attribute definition, the only legal values that follow the

name of the attribute are either a list of strings or one of the

specifiers: %n, %v or %s.

Page 134

ERROR AND WARNING MESSAGES

illegal dimension size

The dimension specified for a group or integer array is either zero,

negative or too large.

illegal dimension specifier

The dimension specifier (The number inside the [] in the declaration

of a group or an integer array) should be a constant integer

expression.  Any other type is illegal.

illegal expression

A syntax error was detected while evaluating an expression.  Check

for the appearance of an illegal character, or a missing or misplaced

operator.

illegal extension string after a ':'

The next state of a flip-flop (scalar or group) may be accessed by

appending an extension of :D or :d.  This is currently the only legal

extension.

illegal mix of variables

In an explicit group declaration, different types of variables cannot

be intermixed in the same group.  The only exception is that the

digits 0 and 1 are allowed inside an explicit boolean group

declaration to indicate private elements that are initialized to 0 or

1..

illegal nesting of function declaration

Declaration of functions cannot be nested.  Check that each function

is properly ended with a '}'.

Page 135

ERROR AND WARNING MESSAGES

illegal number of elements detected in booldat.bo

This error message is produced by the built in function extmin when

the number of input variables, or output variables returned from the

external minimizer in the file booldat.bo does not match the number

of elements that were sent to it in the file booldat.bi.

illegal number of parameters

The number of parameters in a function call is incorrect.  Check the

definition of the function.

illegal operation

The types of operands involved in this operation are not legal.  For

example, trying to compare a boolean variable with a group variable

with the equal to (==) operator.  Consult the section on operators

for a list of legal combinations of operands for each operator.

illegal option <c>

An unknown option <c> was specified in the command line.

illegal recursive call to a function

Recursive calls are not allowed: A function is not allowed to call

itself directly, or through nested calls to other functions.

illegal reference to array

An attempt was made to use the name of an integer array without the

[...] as a simple variable.  You must specify a specific element by

using an index in square brackets.

illegal reference to function name

A function name was mentioned without the parentheses.  All function

calls must have parentheses after the function name, even if the

function has no parameters.

Page 136

ERROR AND WARNING MESSAGES

illegal reference

This error is produced when a symbol is not referenced correctly.

for example, when a variable which is not a group or an integer

array, is referenced with square brackets, or with an incorrect

sequence (e.g. name:D[5] instead of name[5]:D ).

illegal structure

An illegal syntactical structure was detected in a declaration

statement.  It usually indicates a missing number inside the square

brackets when declaring a global integer array, or the appearance of

a number inside the square brackets when they are supposed to be

empty (As in the declaration of a group as a function parameter.), or

the appearance of square brackets where it is not allowed by the

syntax.

illegal symbol

The indicated character string cannot be used as a symbol.  Check the

number of characters, and the legality of the first character, as

well as other characters in the string.

illegal type or value for attribute <attr>

This message indicates that the value used for an attribute in a

declaration statement is illegal.  For example, trying to specify for

an Enumerated Type Attribute a string that does not appear in the

definition of the attribute.

illegal type

The type of variable or expression used in the statement does not

match any of the types allowed for this statement.  A function name

appearing without parentheses, an integer array name without an

index, or the form name:D  where only name is expected can also

produce this error message.

illegal use of a reserved keyword

An attempt was made to use a reserved keyword in a way inconsistent

with its definition.  It usually happens when you try to declare a

variable, a preprocessor macro, or a function with a name that is a

reserved keyword.

Page 137

ERROR AND WARNING MESSAGES

illegal use of function parameter

An attempt is made to use one of the parameters of a user defined

function as an element in an explicit group declaration inside that

function.

illegal use of macro

The macro $v() may be used as a replacement to a parameter of a built

in function, but not for user defined functions.

illegal use of statement

An attempt was made to use a statement which is not allowed in that

context.  For example, an attempt to use one of the statements: JUMP

or XJUMP or STAY or XSTAY inside a .switch block, while the header of

the .switch statement does not allow it.   To allow these statements

inside a .switch block, the group specified in the .switch header

(inside the parenthesis) must either be a flip-flop group, or two

distinct groups which are specified as in:

.switch(ingroup -> outgroup) ...

illegal value

This error message indicates that an integer number or expression has

a value which is not legal for the intended use.  For example, the

message is generated when the left operand in a @ operation is either

negative or larger than the maximal number of elements in a group.  A

hex number, showing the offending value may sometimes be specified

with the message.

include file open error

The preprocessor could not open the file mentioned in the include

directive.  Check that the file exist, and that the correct path is

specified.

Page 138

ERROR AND WARNING MESSAGES

include nesting depth exceeded limit

The nesting depth of the include files exceeded the limit.  This will

typically happen when there is a recursive include sequence (meaning

that a file includes itself).

index out of range

The value of the index in a reference to a group or integer array is

too large or negative.

input file open error

The input file specified in the command line could not be opened.

Check that a file by that name exists.

insufficient memory

This implies that a request for memory allocation was denied by the

operating system.  It usually happens with very complex designs

during minimization, although it may also happen at any other point

in the program.  Try to specify a smaller minimization level.  If

nothing helps, you need to run your design on a machine with more

memory.

integer type expected

The variable or expression used does not have the correct type.  Only

an integer can be used.

line too long

An input line which exceeded the maximal allowed length was detected

in the input file during the preprocessor phase.  Check that the end

of line sequence in the source file is consistent with the operating

system definitions.

Page 139

ERROR AND WARNING MESSAGES

left index must be greater than right index

In an explicit group declaration, when specifying a range of elements

from another group, the right number is the beginning of the range,

and the left number is the end of the range.  For example:

group x = [a[7,3]];

M param error

The value specified in the command line with the -M option

(minimization level)  is illegal.

mismatch between conversion specifier and parameter type

The type of the parameter to be printed in printf or sprintf does not

match the corresponding conversion specifier (e.g. trying to print a

string with a %d specifier).

mismatch between returned value and function type

The return statement in a user defined function has an expression

with a type that is inconsistent with the definition of the function.

The expression in the return statement must have the same type as the

function (Unless a conversion is allowed, such as from integer to

boolean).

missing #endif to balance previous #if

The preprocessor could not find (until the end of the program) an

#endif directive to match the #if directive.

missing end statement

The compiler could not find an end; statement at the end of the

source file.  Make sure that your program ends with an end statement.

This message can also be generated if you did not end properly a

compound statement, but in that case, you will also get other error

messages.

missing file name

The preprocessor could not find a legal file name in an #include

directive.

Page 140

ERROR AND WARNING MESSAGES

missing parameters

The preprocessor could not find any parameters in a #define, #undef

or #ifdef directives.

missing source file name

The name of a main source file name was missing in the command line.

missing "

The filename in an #include directive must be surrounded by double

quotes.

negative shift count

The shift count specified in a shift operation is negative, which is

illegal.

nesting exceeded maximal allowed value

The compiler limits the total amount of lexical nesting of some

commands such as if conditions, loops, function calls etc.  The

limitation is high enough to be practically considered as non

existent.  However, it can happen in extreme cases.    For example,

if you have a nested if-else-if-else-if sequence that spans several

hundred levels, you may exceed the limit.  If you get this error, try

to convert the sequence of if-else-if-else-if into a simple sequence

of if statements, or at least break this sequence into two or more

unrelated sequences.   Converting the sequence into a .switch

statement should also solve the problem.

network did not stabilize after <nnn> steps

This message is produced by the simulator when it cannot resolve

signal propagation due to an asynchronous feedback.   After each

values or clockff statement,  the simulator propagates the signals

through the nodes in the system until there are no more changes.  The

simulator assumes a unit delay for each node.   <nnn> steps is

therefore the amount of delay units that will take an event to pass

through <nnn> nodes.

Page 141

ERROR AND WARNING MESSAGES

Check whether your design contains any asynchronous feedbacks

(Through nodes) that may cause the circuit to oscillate.

If you want to change the limit <nnn> you can do it with the setparam

statement.

number is too large

The number represented by the string of digits is too large to fit

into an integer data type.

parameter does not represent an input

The indicated variable must be either an input literal or a boolean

value which represents an input literal (If written as a boolean

expression, it should contain only one non inverted literal).

parameter has illegal type

The indicated parameter in a function call has a type that does not

match the type required by the function.  Note that automatic type

conversion is done only from input or integer to boolean.

parameter has illegal value

An integer parameter sent to a built in function has a value too high

or too low.

parameter has no declaration

During parsing of the function head, no declaration was found for one

of the parameters.  The syntax requires that all parameters in the

list should be declared, and the declaration must appear before the

"{" that opens the function body.

parameter is not a variable

This error message is generated by some built in functions which

require that certain parameters be sent by reference and not by

value.

Page 142

ERROR AND WARNING MESSAGES

premature end statement

The end statement was found inside another statement (e.g. a compound

statement).  The end statement must appear only at the end of the

program at the outer level.  Check that all your compound statements

are properly ended with a "}".

same attribute appears more than once

The same attribute appears more than once in the same declaration

statement (which is illegal).

same variable assigned more than once inside values statement

Each input literal may be assigned a simulation value only once in a

values statement.  If you can't see any literal that appears more

than once, check for groups that have overlapping elements.

shift count is greater than number of bits in left operand

The shift count specified in a shift operation may not be greater

than the number of bits (In case of shifting an integer) or the

number of elements (in case of shifting a group) of the operand being

shifted.

specified format accepts only a single scalar variable

Some formats of the writestd function accept only a single scalar

variable.   Check that the format number you specified is the correct

one.

statement out of context

The specific statement appears in an incorrect context.  For example,

a return statement in the main program or a jump statement not inside

a .switch statement.

string size exceeds implementation limits

The compiler imposes a limit on the actual size of strings that may

be held in a string variable.   Check that the size of the string is

not exceeding the limit.

Page 143

ERROR AND WARNING MESSAGES

string too long

The indicated string is too long to be a symbol or a number.

symbol already used as parameter

A local variable of a user defined function is declared with a name

that is already used by one of the function parameters.

this type is not accepted for a local variable

Network elements (input, output, node, flip-flop and their groups)

may not be declared as local variables in a function.

too many attribute strings

The compiler has a limitation on the number of strings (values) that

are allowed in the definition of an enumerated type attribute.

too many data files

The number of data files specified in the command line exceeds the

limit.

too many elements

Indicates that the total number of scalar elements accumulated from

all parameters (groups and scalars) exceeded internal limitations.

too many elements inside []

More than the maximal allowed number of elements appear in an

explicit group declaration.

too many output files

The command line contains too many output files.

Page 144

ERROR AND WARNING MESSAGES

too many parameters

The number of parameters in a function call exceeds the maximal

allowed number of parameters.

total number of inputs, nodes and flip-flops exceeds limit

The compiler has an internal limitation on the maximal number of

input literals that it can handle in one design.  Try to split your

design into several pieces, each with a smaller number of input

literals.

type of parameters does not match

Some built in functions require that the types of parameters obey

certain rules within themselves.  For example, the ffj function when

called with two parameters, requires that if one parameter is a group

than the other must also be a group.  Consult the definition of the

specific function.

types do not match

Inside a values statement, the type of the variable should match the

type of the expression assigned to it.  If the variable has type

input group then the expression should also be a group.  If the

variable has type input then the expression should have type boolean,

input or integer.

undefined attribute <attr>

A reference was done in a declaration statement to an attribute that

was not defined previously for that type of variable.

undefined function

A function reference in the form fname() was done to a symbol which

was not declared in any way.

Page 145

ERROR AND WARNING MESSAGES

undefined symbol

The symbol that was specified in the #undef directive is not defined.

undefined variable

A reference was made to a variable which was not declared.

unexpected character

An unexpected character was detected by the compiler.  Check that the

file does not contain any non printable characters.

unknown parameter

The name that appears as a parameter name in a setparam statement is

unknown.  Check that the name is spelled correctly.

unrecognizable statement

The character string does not match any known statement.  Check that

the file does not contain any non printable characters.

variable is not a simple integer

The for statement requires that the loop index should be a simple

integer variable.  (Not even an element of an integer array)

variable length boolean group is not allowed as element

Variable length boolean groups may not be specified as elements in an

explicit group declaration.

zero increment

The increment parameter specified in a for statement has a value of

zero, which is illegal.

Page 146

CHAPTER 6: EXTERNAL LIBRARY

6.1 GENERAL

The external library is a collection of handy functions arranged in

several source files which can be used by your program by including

the appropriate file with an #include directive.

The external library is supplied in Bool source code and can be

edited and modified by the user.   Since requirements change from one

user to the other, and from one design to another, you are actually

encouraged to modify these functions according to your needs.   You

should therefore consider the external library functions as examples,

or templates.

These files have an extension of .INC. A brief summary is included

below.  For more details as well as explanations on how to use the

functions are included as comments in each file.

6.1.1 GENERAL UTILITY FILES

file: ARITH.INC

This file contains several arithmetic functions.  All the functions

in this file accept groups as input parameters and return a group as

an output.  You can even find in the file an example of a function

that adds two groups.  Although this is not very useful (because you

can use the + operation), it does give an example of how this

operator can be built from simpler operations.  Among the functions

in this file you will find functions that can perform signed and

unsigned multiplication of groups.

file: OUTP.INC

This file contains several functions which can write the results in

different formats.

One of the formats in this file which cannot be generated by built in

functions is the INTEL HEX86 which enables you to down load results

directly into a PROM PROGRAMMER.  It contains also another function

which writes results in an equation format, similar to the print

function.  Although this is not very useful as is, it can be used as

an example for generating output results in non standard formats.

file: LOGIC.INC

This file contains a collection of several utilities for logic

manipulations.

Page 147

EXTERNAL LIBRARY

CHAPTER 7: APPENDIX

7.1 PLA TABLE FORMAT

Compatibility With Berkeley PLA Format        Compatibility With Berkeley PLA Format

The following text defines the PLA format used by BOOL to communicate

or interface with other programs.    The format is compatible with

the one used by Berkeley's Espresso minimizer.  It uses a subset of

the Berkeley format, and the differences are listed below:

1. Each product term must fit on a single text line.

2. The only parameters (keywords) that are used by BOOL are:    .i .o

.ilb .ob .type .end (or .e).

3. BOOL will ignore the keywords: .pair .na .p, and all other

keywords will be flagged as errors.

PLA Table Format Definition        PLA Table Format Definition

The PLA data file format is a two level description of a multi-

variable boolean function. It is described as a character matrix

where each line in the matrix describes one product term.

The first few lines in the file contain keywords (or parameters)

which define general information such as the size of the matrix and

the type of function defined in the matrix.

Following these keywords is the matrix.  The lines in the matrix are

called data lines.

Following the matrix is the keyword .e or .end which signals end of

data.

White space (horizontal tab or spaces) anywhere in the file are

ignored except when used as delimiters.   Comments are allowed in the

file by placing a pound sign (#) as the first character in the line.

The keywords should appear each on a separate line. They are listed

below:

.i <d>      Specifies the number of input variables in the matrix.

<d> is a decimal number.

Page 148

APPENDIX

.o <d>      Specifies the number of output variables in the matrix.

<d> is a decimal number.

.ilb token1 token2 ...     Gives the names of the input variables in

the same order as in the matrix.   This must come after .i and .o.

The number of tokens following the keyword must be the same as

specified by .i <d>.  The tokens may be specified on several lines if

one line is not enough.

.ob token1 token2 ...   Gives the names of the output variables in

the same order as in the matrix.   This must come after .i and .o,

and the number of tokens following the keyword must be the same as

specified by .o <d>.  The tokens may be specified on several lines if

one line is not enough.

.type <s>   Specifies the type of function described in the matrix.

<s> is a string of one to three characters and can be any one of the

following: f, fd, fr, fdr.

.e or .end  Marks the end of data.

The keywords .i .o and .e (or .end) are mandatory.   All other

keywords are optional.  If .type does not appear, the default value

fd is used.

The matrix can describe either the on set, the off-set or the don't

care-set of the function (or a combination of the above).  This is

specified by the <s> string following the .type parameter.  Following

is a description of each of the possible types:

f    Only the on-set is specified.

fd   The on-set and the don't care set are specified. (default)

fr   The on-set and the off-set are specified.

fdr  The on-set, the off-set and the don't care set are specified

If the type is f, it is assumed that the boolean functions are fully

specified.  The off-set is calculated as the inverse of the on-set,

and the don't care set is empty.

In all other cases, the functions are assumed to be partially

specified, and in the case of fd or fr, the missing function out of

the three (on-set, off-set or don't care set) is calculated from the

other two.

The on-set and the off-set as specified in the file, are not allowed

to have overlapping minterms.   However, the description of the don't

care set is allowed to have overlapping minterms with either the on-

set, or the off-set.  In that case, the overlapping minterms are

considered as don't care minterms.

The number of lines in the matrix equals the number of different

product terms in the description.   Each line in the matrix contains

n+m non white space characters, where n is the number of inputs and m

is the number of outputs.   The first n characters from the left are

Page 149

APPENDIX

called the input section.  The input section describe the product

term, and each of its characters corresponds to one input variable.

The remaining m characters in the line relate to each of the output

literals, and is called the output section.

The Matrix Input Section        The Matrix Input Section

The input section may contain the following characters:

1     means that the input variable appears in positive polarity (not

inverted).

0     means that the input variable appears in negative polarity

(inverted).

-     means that the input variable does not appear in that product

term.

The characters x, X and 2 are allowed as a synonym for -.

The Matrix Output Section        The Matrix Output Section

Each non white space character in the output section corresponds to

one output variable.    The characters allowed in the output section

are: 0,1,-,~.   The way the characters are interpreted depends on the

value supplied with the .type keyword.  The following describes the

meaning of each character for each of the types:

type f

1 means that the product term belongs to the on-set.

0, - or ~  mean that the product term has no meaning for the value of

this output variable.

type fd

1 means that the product term belongs to the on-set.

- means that the product term belongs to the don't care set.

0 or ~  means that the product term has no meaning for the value of

this output variable.

type fr

1 means that the product term belongs to the on-set.

0 means that the product term belongs to the off-set.

- or ~ means that the product term has no meaning for the value of

this output variable.

type fdr

1 means that the product term belongs to the on-set.

0 means that the product term belongs to the off-set.

- means that the product term belongs to the don't care set.

Page 150

APPENDIX

~ means that the product term has no meaning for the value of this

output variable.

In the output section, the characters x, X or 2 are allowed as a

synonym for -, 3 is allowed as a synonym for ~, and 4 is allowed as a

synonym for 1.

7.2 LIST OF RESERVED WORDS

The following list contains keywords which have special meaning to

the compiler and therefore may not be declared as names.  All the

keywords in the list are reserved in upper and lower case.

.else  .if  .switch  bool  boolean  break  case  continue  else  end

ff  flipflop  for  group  if  inp  input  int  integer  jump

lminimize  minimize  node  out output  print  printf  printsim

return  setparam  stay  univ.ffgroup  univ.igroup  univ.inpgroup

univ.ndgroup  univ.outgroup  v1.break  v1.continue  v1.return  values

while  xjump  xstay

Notice that some of the names in the list belong to built in

functions that cannot be overridden by user defined names.   Also

note that some keywords in the list are not documented and are

reserved for backward compatibility with previous versions.

Page 151

INDEX

!  10, 35, 60

!=  15, 34, 60

#define  63

#else  64

#endif  64

#ifdef  64

#ifndef  64

#include  64

#undef  64

&  10, 60

*  16, 60

+  15, 60

-  15, 60

/  16, 60

<  34, 60

<<  16, 18, 60

<=  15, 34

==  34, 60

>  34, 60

>=  15, 34, 60

>>  16, 18, 60

@  16, 60

^  15, 60

|  10, 60

~  16, 35, 60

absolute minimum  47

actual parameter  38, 71

add_dependents_status function  113

add_literals_status function  114

add_status_by_attrib function  114

ALU  17

AND operatoror  10

and_operator parameter  79

anyout_attrib  66

Arithmetic minus operator  15

Arithmetic plus operator  15

ASCII value  56

assign_operator parameter  79

assignment statement  11, 72

attribute of a variable  66, 67

Berkeley PLA format  51

Binary numbers  56

binary operators  16, 60

bitwise inverse  16

block  10

BOOL keyword  27

bool.def  82

boolean class  86

boolean class group  86

boolean class scalar  86

Page 152

boolean class variable  7, 58

boolean data type  7, 57

boolean equation  10

boolean function  10

boolean keyword  27

boolean variable  7, 27, 58

break statement  35, 75

built in function  52, 69

Case of a character  7

chars_per_eqn_line parameter  79

check_sim_dc parameter  79

clear and assign statement  74

clockff function  45, 101

clr function  103

command line  82

command line switches  83

Comments  55

compound statement  10, 72

conflicts during an assignment  11

constant values  58

continuation_line parameter  79

continue statement  35, 75

conversion specifier  44, 92

data file  83

Decimal numbers  56

declaration statement  64

default minimization level  84

default_minlevel parameter  80

default_polarity parameter  80

divide operator  16

don't cares  21

don't cares during simulation  46

element of a group  13

end statement  6, 81

endeqn_mark parameter  79

Enumerated Type Attribute  67, 68

eval function  102

exit function  122

explicit group declaration  14, 65

expression  35, 62

extmin function  98

Feedback loops  45

ff keyword  22

ff_attrib  66

ff_extension parameter  79

ffj function  26, 103

ffk function  26, 103

field size modifier  93

file number  83, 89

fileinfo function  90

first pass  82

flip_flop variable  7, 58

for statement  31, 32, 76

formal parameter  38, 70, 71

Page 153

format specifier  43

fully specified  21

Function  38, 69

general_attrib_info function  118

general_attrib_num function  119

general_attrib_str function  119

get_attrib_num function  115

get_attrib_status function  116

get_attrib_str function  116

get_list_status function  117

get_literals_status function  117

get_source_name function  90

get_status function  118

getpt function  104

getvarname function  108

global don't care  57

global one  57

global variables  7, 59

global zero  57

gr function  18, 105

Greater equal operator  15

group  13, 58

grouping function  18

grouping operator  16, 18

hazard free  49

hazard_free function  47, 99

Hexadecimal numbers  56

if statement  74

if-else-if structure  24

Implicit group declaration  65

include path  83

inp keyword  9

input literal  10

Input variable  7, 58

input_attrib  66

Integer array  35, 59

integer data type  57

Integer variable  34, 58

internal error  126

inverse operator  16

isdc function  108

isone function  108

isused_input function  109

iszero function  109

JK flip flop  25

jump statement  22, 78

length function  122

Less equal operator  15

list file  84

lit function  109

literal  10

litn function  109

lminimize function  100

local variable  41, 59, 70

Page 154

lser function  110

lstat function  110

main source file  82

max_sim_iterations parameter  79

minimization level  47, 97

minimization polarity  46

minimize function  46, 100

minstat function  111

multiply operator  16

name_index_prefix parameter  79

name_index_suffix parameter  79

network element  7, 58, 86

next state  22

node variable  7, 58

node_attrib  66

nonredundant boolean function  98

Not equal operator  15

NOT operatoror  10

not_operator parameter  78

Numeric Attribute  67, 68

Numeric constants  29, 56

Octal numbers  56

one function  106

one_value parameter  79

open_inpfile function  91

open_outfile function  91

operators  60

OR operator  10

or_operator parameter  79

original name of a variable  89

out keyword  9

output file  83

output variable  7, 58

output_attrib  66

overriding assignment  12, 73

Parameter of a function  38

Parentheses  16

partially specified  21

pass by reference  38, 71

pass by value  38, 71

pass function  105

pminimize function  47, 101

precedence of operators  16, 63

preprocessor  63, 82

present state  22

print function  91

printf function  43, 92

printsim function  94

printvarname function  123

priority encoder  36

product term  9

ptcomp function  106

ptn function  111

readpla function  94

Page 155

redundant product term  49

reformat_extension parameter  79

return statement  71

RS flip flop  28

scalar variable  13

second pass  82

serial number of an input literal  110

setparam statement  46, 47, 51, 78

shift left operator  16, 18

shift right operator  16, 18

simulator  42, 79, 80, 101

size conversion  73

size function  39, 112

size of a group  13, 57, 58

size of an integer array  59

sprintf function  120

state machine  21

state transitions  22

state variable  22

Status Word  87

Status Word List  112

stay statement  22, 78

strcat function  121

strcmp function  121

String Attribute  67, 68

string data type  57

string variable  44, 58

strlen function  121

sum of product terms  9

.switch statement  77

symbolic names  7, 55

symbols  55

trace file  84

type of a function  69

unary operators  16, 60

undefined value  42

univ.ffgroup  59

univ.igroup  59

univ.inpgroup  59

univ.ndgroup  59

univ.outgroup  59

universal groups  59

unknown value  41

user defined function  69

valueof function  107

values statement  43, 45, 80

Variable Attribute  67, 68

Variable Identification Number  87

variable size boolean group  58

verbose command line switch  84

virgin state  59, 103

while statement  32, 75

White space  55

writepla function  95

Page 156

writestd function  51, 95

xjump statement  25, 78

XOR operator  15

xstay statement  25, 78

zero function  107

zero_value parameter  79

Page 157