Program GPSSYNC

GPS Synchronization and Test Program

For use with GPS Receivers

(C) Copyright David H. Ransom, Jr., 1998

All rights reserved.

Version  9838

September, 1998

David H. Ransom, Jr.

240 Bristlecone Pines Road

Sedona, Arizona  86336 USA

e-mail: rans7500@spacelink.nasa.gov

WWW Home Page

-------------

http://www.dransom.com

http://tie.jpl.nasa.gov/dransom/

Bulletin Board System

---------------------

RPV ASTRONOMY BBS

(520) 282-5559 @ 2400-14400 Baud

Program GPSSYNC Synchronization and Test Program                 Page 1

INTRODUCTION

------------

Program GPSSYNC is shareware and there are no warranties of any kind.

Registration is NOT required but would be appreciated to encourage me to

continue program development and testing. Donations in any amount may be

sent to the address shown on the title page. If requested, I will send a

3.5" disk with the current release of the program for donations of US$15.00

or more! See file README for registration information.

Program GPSSYNC is a DOS-based program for IBM-compatible personal

computers. It has only been tested on Pentium and higher systems but should

execute without difficulty on 386 systems or better. The program should be

compatible with DOS 5.0 and above (DOS 6.22 is recommended), Windows 3.1x

(using the MS-DOS prompt), and Windows 95/98 (in a DOS window). No tests

have been performed with Windows NT. GPSSYNC is intended to test the

compatibility of a GPS Receiver using the NMEA-0183 specification and to

allow the display of received GPS time and position information. The

program may also be used to synchronize the computer clock to UTC

(Coordinated Universal Time, also sometimes referred to as GMT or Greenwich

Mean Time, a designation which is now considered obsolete in most

quarters).

As of Version 9838, program GPSSYNC continues to undergo enhancement,

testing and validation. Reports on other GPS receivers with which the

program executes satisfactorily are requested. Comments, suggestions, and

bug reports are invited. A known bug occurs at the end of the year (31

December local date only) with the dates when UTC date is a different year

from the local date; this minor bug will be corrected at a future release.

GPSSYNC is Year 2000 compliant (to the extent that the connected GPS

receiver is also Year 2000 compliant). Among features being considered for

future releases is additional data logging to record the "raw" position and

time information for further analysis. Other requested features will be

considered.

GPSSYNC has been tested with the following GPS receivers using NMEA-

0183 format data at 4800 baud:

Delorme Tripmate

Garmin GPS III

Garmin GPS II+

Garmin GPS 12

Except for recognizing the chipsets for the two brands of GPS receivers

tested to date, no special code is included which should restrict operation

of GPSSYNC with other models and makes of GPS receivers. Test reports on

other GPS receivers are requested (see my mailing address and e-mail

address on the title page).

GPSSYNC is a test program which has been written as a first step

toward including GPS receiver capability in my other software (such as

STSPLUS and ASTROCLK) for time-setting and position functions. At the same

time, it is hopefully a useful program on its own merits for the testing of

GPS receivers and for obtaining high accuracy position information

(especially from GPS receivers such as the Delorme Tripmate which have no

Program GPSSYNC Synchronization and Test Program                 Page 2

display capability). Comments and suggestions are invited.

Special thanks to my friend Paul Traufler for his very considerable

help in deciphering the NMEA-0183 messages generated by a GPS receiver. I

might never have tumbled to some of the more subtle messages without his

assistance and encouragement!

Thanks also to Ken Ernandes for his invaluable assistance and

suggestions during the testing and validation of GPSSYNC!

Dave Ransom

Program GPSSYNC Synchronization and Test Program                 Page 3

GPSSYNC OPERATION

-----------------

GPSSYNC requires a free serial communications port, COM1 through COM4,

and the ability to execute a standard DOS (MS-DOS) program. When executed,

GPSSYNC looks for a file GPSSYNC.INI which contains specific configuration

information. If the file is not found, default parameters for Sedona, AZ,

are automatically used and the file is created when the program is exited.

To begin operation of GPSSYNC, enter the following command at the DOS

prompt:

GPSSYNC

The GPSSYNC setup screen will appear:

---------------------------------------------------------------------------

GPS Synchronization Program, Version 9838

Copyright (C) 1998 by David H. Ransom, Jr.

Current GPSSYNC Settings:

Serial Port:       COM1

Serial Rate:       4800

UTC Offset:        -7.0 hrs

Daylight Time:        0

Local Zone Abbr:   MST

Location:          Sedona, AZ

Press SPACE to change, ENTER to continue, or ESC to quit:  _

---------------------------------------------------------------------------

NOTE: If the program is being restarted and prior AVERAGE position data are

available, an additional command is available and shown in the prompt

above, "TAB to resume":

Press SPACE to change, ENTER to continue, TAB to resume, or ESC to quit:  _

If TAB is pressed, the average data accumulated during the previous

execution(s) of the program are displayed and restored. Subsequent average

position and elevation calculations use the prior data.

********************

*  IMPORTANT NOTE  *

********************

Users are reminded to always press ENTER to start a new set of

position calculations (or to press "C" to clear the AVERAGE data if TAB has

been pressed) when executing the program from a new location. Failure to do

so will yield AVERAGE data which is in error and meaningless!

If the displayed information is correct, press ENTER (or TAB, if

available) to continue with program execution. Press ESC to quit GPSSYNC

and return to the DOS prompt. Always press ENTER to start a new set of

position calculations (and NOT use the prior averages). If some or all of

the displayed information needs to be changed, press SPACE and enter the

Program GPSSYNC Synchronization and Test Program                 Page 4

appropriate information as prompted. Press ENTER to retain the current

setting for a particular value. Note that only 4800 baud is supported at

this time. "UTC Offset" is the time difference from UTC/GMT in hours

WITHOUT any correction for Daylight/Summer time (which is accounted for

separately). In the United States, Daylight Time is in effect in most

areas, not including Arizona and Indiana, from the first Sunday in April

until the last Sunday in October. The change is made at 02:00 local time.

The dates and times may vary in other countries.

GPSSYNC requires that the GPS receiver be configured for NMEA-0183,

Version 1.5 or 2.0, serial output at 4800 baud. Although the details may

vary from receiver to receiver, NMEA compatibility should assure that

GPSSYNC will operate with almost any GPS receiver. There are, however, so-

called "proprietary messages" which are generated by the various GPS

receiver chipsets to identify special functions as well as the

manufacturer. The GPS receivers tested to date are identified as to chipset

manufacturer as follows:

Delorme Tripmate         RWI Zodiac Chipset

Garmin                   Garmin Chipset

"RWI" stands for Rockwell International, who provide the chipset for the

Delorme unit. Other chipset manufacturers doubtless have their own

proprietary messages but I have no information on those messages at this

time.

The initial messages from a given chipset vary slightly. The RWI

Zodiac Cipset, for example, requires that several messages be exchanged

between the GPS receiver and the PC. This sequence is signalled by several

"beeps" from the PC. For both chipsets tested, unique proprietary messages

are sent from the GPS receiver which permit GPSSYNC to identify the chipset

at the upper right of the program display. This field will be left blank

for unidentified chipsets. The Garmin chipset also identifies the current

Map Datum in use by the receiver; the default is "WGS-84".

When first started, GPSSYNC monitors the serial communications line to

the GPS receiver for messages. If no messages are received within 15

seconds, an error message is displayed:

---------------------------------------------------------------------------

GPS Synchronization

NO GPS RESPONSE FOR 15 SECONDS ... Check batteries!

Press ENTER to quit ...

---------------------------------------------------------------------------

The serial communications line to the GPS receiver is continuously

monitored throughout program operation. Since messages will cease upon

discharge of the receiver's battery, that condition will be reported. If

the error message is the only activity reported, check the initial

conditions for the correct COM port, check all cables, verify that the GPS

receiver is configured for NMEA output at 4800 baud, and that fresh

batteries are installed in the receiver.

When GPSSYNC has established communications with the receiver and

operation begins, the program awaits data messages from the GPS receiver:

Program GPSSYNC Synchronization and Test Program                 Page 5

---------------------------------------------------------------------------

GPS Synchronization                    RWI Zodiac Chipset

GPS NOT VALID

UTC Time:  INVALID

MST Time:  INVALID

DOS Time: 10:05:13  02 APR 1998

Run Time:  0:00:09

---------------------------------------------------------------------------

Depending upon the type of receiver being used, considerable time may

elapse (up to 5 minutes or more) before any further activity is reported.

In the example shown above for the Delorme Tripmate, the presence of "RWI

Zodiac Chipset" indicates that communications with the that receiver have

been established.

During this time, the receiver is attempting to establish

communications with a GPS satellite to determine the current UTC time and

date and to obtain ephemeris data (orbital positions) for the satellites in

the GPS constellation. Typically, up to 12 or more GPS satellites may be

"in view" from a user's location. Which satellite(s) may be receivable

depends upon the specific characteristics of the user's location. Best

reception is obtained with the receiver outside with a clear horizon in all

directions, a situation which may not always be practical. Buildings,

trees, chimneys, steel structures, and other obstructions may restrict the

areas of the sky from which signals may be received. Use of a GPS receiver

indoors may significantly degrade the signals although I have generally had

success with the receiver placed near a window in a single story building.

Throughout this period, GPSSYNC shows the GPS as "NOT VALID" along with the

DOS time. UTC and local time (MST Time in the example above) may appear or

may be shown as "INVALID", depending upon the receiver.

When the first GPS signal is received, GPSSYNC displays that

information:

---------------------------------------------------------------------------

GPS Synchronization

GPS NOT VALID

UTC Time:  INVALID

MST Time:  INVALID

DOS Time: 10:08:12  02 APR 1998

Run Time:  0:01:52

PRN ELV  AZM  SIG

26            34   =========

---------------------------------------------------------------------------

In this example, the current DOS time is 10:08:12 on 02 APR 1998, 1:52 has

elapsed since the program began monitoring the GPS receiver, and a signal

is being received from GPS satellite PRN (Pseudo Random Number) 26 with a

signal strength of 34. The bar graph to the right of the signal strength is

proportional to the signal strength. At this time, the GPS receiver does

not yet know for certain the location (ELV=Elevation above level horizon,

AZM=Azimuth or direction in the sense NESW) of the particular GPS satellite

Program GPSSYNC Synchronization and Test Program                 Page 6

being received. Among other things, display of this first GPS signal

verifies that the GPS receiver and GPSSYNC are operating properly together.

During the next several minutes, additional GPS satellites should be

detected and their signal strengths displayed. During this time the GPS

receiver will obtain the orbital data for satellites supposedly in view (if

the current position is correct) and initialize its internal data. Failure

to proceed beyond this point in, say, five or ten minutes may indicate an

unsuitable location for the GPS receiver. As additional GPS satellite

signals are received, the GPS receiver makes a preliminary internal

position determination and displays the probable location of the GPS

satellites which should be "in view" from that location:

---------------------------------------------------------------------------

GPS Synchronization

GPS NOT VALID

UTC Time:  INVALID

LCL Time:  INVALID

DOS Time: 10:08:38  02 APR 1998

Run Time:  0:02:18

PRN ELV  AZM  SIG

27  80   055

02  61   312

13  49   220

19  45   118

07  45   204

15  20   092

26  19   315  33   =========

31  17   042

18  07   169

10       250

23       351

04       190

---------------------------------------------------------------------------

Different GPS receivers may order the satellites differently. The RWI

Zodiac Chipset orders the satellites by elevation and the Garmin Chipset

orders the satellites by PRN number. As a general rule and ignoring

possible signal obstructions, the higher the elevation the more likely that

the signal from that GPS satellite will be successfully received. In the

example shown, 12 GPS satellites are considered candidates for reception

and (given the preliminary position determination) 9 of those should be

above the horizon.

As signals from more GPS satellites are received and displayed, the

receiver's estimate of the current position improves and satellites which

are judged to be below the horizon will be dropped from the list displayed.

Most GPS receivers assume that the current position is no more than

approximately 500 miles from the previous position and this is the initial

position used for satellite position calculations. If the receiver has

moved a greater distance, considerable time may elapse before the receiver

determines where it may be, up to 15 minutes or more. Similar delays may

occur in poor signal conditions or when the geometry of the GPS satellites

relative to the user's location is unfavorable for an accurate position

Program GPSSYNC Synchronization and Test Program                 Page 7

determination.

Sooner or later, hopefully, the GPS receiver will receive three or

four GPS satellites of sufficient signal quality and with adequately

favorable geometry that a position determination can be made. Two types of

position determinations are available: 2D and 3D. "2D" means two

dimensional with only position (latitude and longitude) reported; "3D"

means three dimensional with position and altitude (above mean sea level)

reported. 3D is typically more accurate than 2D. When the position

determination can be made, the display is updated. Examples of both modes

are shown on the following page.

Program GPSSYNC Synchronization and Test Program                 Page 8

2D DISPLAY EXAMPLE

---------------------------------------------------------------------------

GPS Synchronization                    Garmin Chipset

Map Datum = WGS 84

GPS VALID, 2D

UTC Time: 17:14:39  02 APR 1998        Location:    Sedona, AZ

MST Time: 10:14:39  02 APR 1998        Satellites:  3 / 9

DOS Time: 10:14:35  02 APR 1998        HDOP:        5.10

Run Time:  0:01:10

PRN ELV  AZM  SIG                                  Current           Average

02  62   316  42 * ============        LAT:   34 51.5320'N      34 51.5320'N

07  47   204  41 * ============        LON: -111 49.6510'W    -111 49.6513'W

13  46   217                           ALT:       0.0 ft            0.0 ft

15  21   089

18  04   168                                # Samples  Position:         3

19  43   119                                           Altitude:         0

26  20   313  46 * =============

27  79   068

31  14   041

Press:   T=Set TIME   C=Clear AVERAGES   ENTER=QUIT   ESC=CANCEL

---------------------------------------------------------------------------

3D DISPLAY EXAMPLE

---------------------------------------------------------------------------

GPS Synchronization                    Garmin Chipset

Map Datum = WGS 84

GPS VALID, 3D

UTC Time: 17:16:19  02 APR 1998        Location:    Sedona, AZ

MST Time: 10:16:19  02 APR 1998        Satellites:  4 / 9

DOS Time: 10:16:16  02 APR 1998        HDOP:        2.00

Run Time:  0:02:50

PRN ELV  AZM  SIG                                  Current           Average

02  62   317  39 * ===========         LAT:   34 51.5540'N      34 51.5445'N

07  48   205  41 * ============        LON: -111 49.5950'W    -111 49.5988'W

13  46   217                           ALT:    4493.8 ft         4464.2 ft

15  21   089

18  03   168                                # Samples  Position:        46

19  42   120  30   ========                            Altitude:         6

26  20   313  48 * ==============

27  79   073  34   =========

31  14   041  31 * ========

Press:   T=Set TIME   C=Clear AVERAGES   ENTER=QUIT   ESC=CANCEL

---------------------------------------------------------------------------

Program GPSSYNC Synchronization and Test Program                 Page 9

For both examples, the "*" next to the signal strength indicates a

satellite which is being used by the GPS receiver in its position

determination calculations. Which satellites are used versus not used are

determined by both signal strength and geometry. A minimum of FOUR

satellites is required for 3D position determination; THREE satellites are

required for 2D position determination. Note that the various data messages

from which the display information is derived are not necessarily sent

coincidently and there may therefore be a lag of from one to two seconds

before all information displayed is properly coordinated.

The "Satellites" information indicates the number of satellites

currently being used in position determination calculations versus the

number of satellites currently "in view" or calculated to be at or above

the user's horizon. The horizon calculations do NOT take into account any

possible obstructions, such as mountains, nor do they account for locations

at high elevations with a horizon below true horizontal. Because of their

orbital altitude at some 10,000 miles, the GPS satellites are "visible" to

slightly more than one hemisphere of the Earth. With 24 or more operational

GPS satellites in orbit, at least 12 could theoretically be in view at any

given point in time and most current GPS receivers have 12 active channels.

In practice, fewer satellites are usually in view with useable signals

because of variations in orbital position, coincidence, and obstructions.

"HDOP" stands for "Horizontal Dillution of Precision", a figure of

merit to describe the accuracy of the position determination calculations.

A number under 1.00 is usually considered ideal and higher numbers

proportionately decrease the probable accuracy of the position

calculations.

Some receivers may require an initial 3D position determination (FOUR

GPS satellites being received satisfactorily) before sending valid data.

Only after the initial 3D calculation has been achieved will 2D position

determination data be generated. Under poor signal conditions and/or

satellite geometry, this can cause extended delays especially for receivers

having less sensitivity. On rare occasions with the Delorme Tripmate,

initial 3D position determination has taken in excess of 15 minutes. The

Garmin GPS III has seldom required more than 2 or 3 minutes to acquire at

least 2D position even under unfavorable conditions; from this I conclude

that the Garmin unit is both more sensitive and uses a more effective

satellite selection algorithm.

The NMEA-0183 standard requires position, latitude and longitude to be

reported as degrees, minutes, and decimal fractions of a minute. I have

reformatted the data to separate the degrees from the minutes (and the

degree symbol which is present on GPSSYNC's display has been deleted from

this document because it may not print correctly on some systems). Two sets

of data are reported: the CURRENT position and the calculated AVERAGE

position. The CURRENT position is the latest reported by the receiver and

may vary considerably depending upon signal conditions, satellite geometry,

and the presence or absence of S/A (Selective Availability), the means that

the Department of Defense uses to "dither" or degrade the accuracy of the

GPS signals for reasons of national security. The AVERAGE position is

calculated using a simple arithmetic average; a more sophisticated

calculation scheme might yield superior results but I have not had an

opportunity to investigate such schemes. The number of samples used in

calculating the AVERAGE is also displayed.

Calculation of the user's altitude requires 3D position determination.

That number may be zero (if no valid altitude determination has yet been

performed) and is decreased in intensity when 2D position determination is

Program GPSSYNC Synchronization and Test Program                 Page 10

in effect. The altitude data are NOT updated during 2D position

determinations and the AVERAGE is calculated independently. The accuracy of

the altitude calculation is substantially less than for the latitude and

longitude. At my location in Sedona, Arizona, the altitude may vary some

hundreds of feet in either direction from the true altitude. For reasons of

personal preference, I have converted the altitude from meters, the units

required by NMEA-0183, to feet; the standard conversion factor is: 1 foot

equals 0.3048 meters.

Several keys are active while the data are being updated. A reminder

of these active keys is displayed at the bottom of the screen.

ENTER     Press to quit GPS calculations and return to the initial

GPSSYNC menu. The AVERAGE values are retained.

ESC       Press to cancel the GPS calculations in progress and discard

the current AVERAGE values. The AVERAGE values are re-

initialized from file GPSSYNC.INI (if present) to those from

the prior execution of the program.

T         Press to cause GPSSYNC to set the computer's time and date

to the GPS time and date displayed for the local time zone

(MST or Mountain Standard Time in the examples).

CAUTION: If the UTC Offset and/or Daylight are incorrectly

set during program initialization, an incorrect local time

will be displayed and used with the "T" command. Verify that

these settings and the displayed times and dates are correct

BEFORE using the "T" command!

C         Press to clear (reset) the AVERAGE position and altitude and

sample counts. The prior data are lost. (See also ESC above)

GPS Synchronization from within Program STSPLUS

-----------------------------------------------

Program GPSSYNC may be directly executed from Program STSPLUS by

pressing ALT-F2 from the STSPLUS Main Menu, Version 9838 or later. This

feature allows the STSPLUS user to dynamically update the STSPLUS primary

location, UTC Offset, and Daylight/Summer Time setting directly from the

GPSSYNC initialization information and the AVERAGE GPS position data

calculated by GPSSYNC. Operation of GPSSYNC is identical to stand-alone

operation except that when GPSSYNC is exited the following special messages

are displayed:

---------------------------------------------------------------------------

Automatic STSPLUS Update from GPSSYNC

GPSSYNC can set the current primary location in STSPLUS to the

AVERAGE position calculated with the GPS receiver. In addition,

GPSSYNC can append the AVERAGE position to the STSPLUS city file.

Do you wish GPSSYNC to update the STSPLUS location data (Y,n):

Do you wish GPSSYNC to update the STSPLUS city file (Y,n):

---------------------------------------------------------------------------

Program GPSSYNC Synchronization and Test Program                 Page 11

The AVERAGE latitude, longitude, and elevation calculated by GPSSYNC

may be substituted for the present primary location in STSPLUS by

responding "Y", "y" or ENTER to the first prompt. This also updates the

STSPLUS UTC Offset and Daylight/Summer Time settings in STSPLUS. The new

data are written to file STSPLUS.INI directly and will be used in all

subsequent executions of STSPLUS (until changed by the user). Respond "N"

or "n" if you do NOT wish the data to be updated. The second prompt, which

only appears if you update the data with the first prompt, allows you to

append the location name and AVERAGE latitude, longitude, and elevation to

the current city file in use by STSPLUS. When the prompt(s) have been

responded to, execution returns to the STSPLUS Main Menu.

********************

*  IMPORTANT NOTE  *

********************

In order for this automatic update feature to operate properly, the

two files GPSSYNC.EXE and GPSSYNC.INI must be located in the STSPLUS

directory. This allows STSPLUS to "find" GPSSYNC and the other files

required for the update process. If the GPSSYNC files are not present,

pressing ALT-F2 from the STSPLUS Main Menu will have no effect.

Received NMEA-0183 Message Logging

----------------------------------

As a convenience during testing and for the curious, I have included a

special command line option to cause GPSSYNC to log all received NMEA-0183

messages in a special file, GPSSYNC.MSG. This feature is enabled by

executing GPSSYNC with the following command:

GPSSYNC /NMEA

Note that file GPSSYNC.MSG is opened in the APPEND mode and, unless the

file is deleted after each test, it will accumulate messages from multiple

executions of GPSSYNC. The file may become quite large since multiple

messages are sent by the GPS receiver every second or two.

Users experiencing difficulty with the operation of GPSSYNC with a

previously untested brand or model of GPS receiver may send me a copy of

files GPSSYNC.INI and GPSSYNC.MSG via e-mail or on disk (preferably as a

ZIP file to reduce the file size) for their receiver and I will attempt to

determine what's going on as time permits. I recommend that the file

consist of all messages from the time program GPSSYNC is started until the

GPS receiver has obtained 3D lock and accurate positions are displayed. No

guarantees, of course!

File GPSSYNC.INI

----------------

Program GPSSYNC saves various required data in file GPSSYNC.INI. This

file is read each time the program is started and updated each time the

program is exited. The following sample file includes comments to the right

explaining each line of the file:

Program GPSSYNC Synchronization and Test Program                 Page 12

9838                     Current GPSSYNC Version Number

-7                       UTC Offset (hours)

0                       Daylight Flag (0=OFF, 1=ON)

MST                      Local Time Zone Abbreviation

1                       COM port assigned (1 to 4)

"Sedona, AZ"             Location

627476.767 , 300        Latitude, sum and # samples

-2012901.208 , 300       Longitude, sum and # samples

414753.8 , 300          Elevation, sum and # samples

34.8598203888889        Current AVERAGE Latitude (degrees)

-111.827844888889        Current AVERAGE Longitude (degrees)

4535.80271216098        Current AVERAGE Elevation (feet)

The Current AVERAGE latitude, longitude, and elevation data included

in file GPSSYNC.INI are the same as those displayed during program

operation. Even with Selective Availability enabled (as it usually is), a

sufficient number of position calculations should yield a reasonably

accurate position over time (better than the specified instantaneous

accuracy of approximately 30 meters horizontally and 100 meters

vertically). Users may find these data helpful for other applications but

should remember that the data are not accurate to the full precision shown.

Program GPSSYNC Synchronization and Test Program                 Page 13

GPSSYNC Revision History

------------------------

Each released version of GPSSYNC uses a four digit revision code such

as 9838. The first two digits indicate the year and the second two digits

indicate the week of the year. In some cases, an additional letter suffix

is added to distinguish changes occurring within the same week or to

identify special versions. A partial week at the beginning or end of the

year is counted as a full week. Using this method, a year will typically

have 53 weeks although it is possible to have 54 weeks in a leap year (1972

is an example). The current year-week revision code is shown on the Julian

Date display, Display Mode 7, in my program ASTROCLK.

This file records the revision history of program GPSSYNC through all

of the minor twists and turns that usually accompany the evolution of such

a program. These notes may also be helpful to users who are upgrading from

one version to another to find out what has changed.

David H. Ransom, Jr.

e-mail: rans7500@spacelink.nasa.gov

Home Page: http://www.dransom.com

http://tie.jpl.nasa.gov/dransom/

Version 9838 -- September 1998

------------------------------

-This is a MAJOR UPDATE and incorporates a wide variety of changes and

improvements (including those made in Beta Version 9837 which was not

released publicly). Review this documentation for details of the updates

and changes.

-The structure of file GPSSYNC.INI has changed substantially from Version

9817.

*************

*  CAUTION  *

*************

If you have used GPSSYNC Version 9817, please delete

file GPSSYNC.INI prior to executing Version 9838; a new

file will automatically be created.

Version 9837 -- September 1998

------------------------------

-Beta Version for testing, not released publicly.

Version 9817 -- April 1998

--------------------------

-Initial public release