Vector to Two-Line Elements

(VEC2TLE) Software

User Manual

Version 9648

22 November 1996

Kenneth J. Ernandes

29-34 170 Street

Flushing, NY 11358

USA

Kenneth J. Ernandes

29-34 170 Street

Flushing, NY 11358

USA

Vector to Two-Line Elements (VEC2TLE) Software User Manual

This publication is a Technical Reference Manual for the VEC2TLE software.

A step-by-step operating guide is provided in the User Instructions.

Copyright c 1993-1996 by Kenneth J. Ernandes.  This publication may be

reproduced and distributed as long as the final product is a complete,

unaltered reproduction of the original document.  All other rights are

reserved.

IBM is a registered trademark of International Business Machines Corporation.  Microsoft, Windows, and MS-DOS are registered trademarks of Microsoft Corporation.

TABLE OF CONTENTS

1.0     INTRODUCTION  .............................................       1

1.1     Vector Types Supported ..............................     2

1.2     Keplerian Orbital Elements ..........................     3

1.3     Orbit Propagation Models ............................     4

2.0     INSTALLATION ..............................................       6

2.1     Upgrading From a Previous Version of VEC2TLE ........     8

3.0     USER INTERFACE DESCRIPTION ................................       9

3.1     Menu Activation .....................................     9

3.2     Menu Item Selection .................................     9

3.3     Dialog Boxes ........................................     10

3.4     Windows .............................................     12

3.5     Status Line Help ....................................     13

4.0     OPERATING INSTRUCTIONS ....................................       14

4.1     Startup .............................................     14

4.2     File ................................................     14

4.3     Settings ............................................     17

4.4     Launch Date/Time ....................................     19

4.5     Drag Multiplier .....................................     19

4.6     Compute TLE .........................................     20

4.7     Re-Epoch TLE at Asc Node ............................     21

4.8     Impulsive Delta-V ...................................     22

4.9     Compute Date ........................................     23

4.10    Compute Day of Year .................................     23

4.10    Program Termination .................................     23

5.0     REFERENCES ................................................       24

6.0     ACKNOWLEDGMENTS ...........................................       25

7.0     LICENSE STATEMENT .........................................       26

7.1     Registration ........................................     26

7.2     Distribution ........................................     27

7.3     Limited Warranty ....................................     27

7.4     Governing Law and General Provision..................     28

APPENDIX A     --  HOT KEY SUMMARY .................................      29

APPENDIX B      --  SEQUENTIAL DAY OF THE YEAR ......................     31

APPENDIX C      --  ACRONYMS AND ABBREVIATIONS ......................     33

APPENDIX D      --  VECTOR FILE FORMATS .............................     35

APPENDIX E      --  SOURCES OF STATE VECTORS ........................     39

APPENDIX F      --  GENERAL INFORMATION .............................     43

APPENDIX G      --  ABOUT THE AUTHOR ................................     46

PAGE 1:

1.0		INTRODUCTION

The Vector to Two-Line Elements (VEC2TLE) software provides the user with the

capability to convert position/velocity/time state vectors of a variety of

formats to AMSAT Keplerian and/or National Aeronautics and Space

Administration (NASA) Compatible Keplerian Two-Line Element (TLE) sets.

These Keplerian elements contain orbit descriptions compatible for

propagation with the Simplified General Perturbations (SGP) and SGP Version 4

(SGP4) orbit theories developed for use by the North American Aerospace

Defense Command (NORAD) and United States Space Command (USSPACECOM).

Both AMSAT and Two-Line Keplerian element formats are popular sources of

orbital data.  This is because one or both these formats are generally

available for Earth-orbiting satellites of interest.  As a result, the

popular satellite tracking software such as STSPLUS and TRAKSAT use TLEs

and/or AMSAT Keplerian elements as their primary sources of input data along

with the SGP4 orbit propagator.

Keplerian element data is available for most Earth-orbiting satellites from

several sources.  VEC2TLE is not intended to replace these as primary sources

of orbital data.  Rather, its primary purpose is for circumstances in which

this data may not be available in a timely fashion.  A prime example is the

Space Transportation System (STS) in which position and velocity state

vectors may be the only orbital information available in near real time.  In

the past, users of the tracking software had little choice but to wait (hours

or days) until the Keplerian Element data was available following thruster

firings that changed the Shuttle's orbit.  Often, the Keplerian Element data

is out of date by the time it is received due to subsequent thruster firings.  It is this  situation that resulted in the creation of VEC2TLE.  The Keplerian Elements computed by VEC2TLE are fully compatible with SGP or SGP4 as true and accurate renditions of the orbit specified by the input state vectors.

VEC2TLE is also useful as a pre-launch planning tool.  The software has the

ability to use time-independent, Earth-fixed state vectors.  With time-

independent vectors, the only missing information for computing Keplerian

elements is the launch date and time.  Once a scheduled or actual launch time

is provided, VEC2TLE readily computes the corresponding Keplerian elements.

Pre-launch nominal time-independent vectors are provided for each Space

Shuttle launch and uploaded to a variety of Internet and other public

electronic Bulletin Board Systems (BBSs) -- see Appendix E.  Additionally,

pre-launch nominal vectors were used to estimate Keplerian elements for

several new amateur radio satellites including: Radio Sputnik 15 (RS-15),

JAS-2 (a.k.a. FO-29), and UNAMSAT-B (a.k.a. MO-30).  The Keplerian elements

estimated for these new amateur radio satellites have been very useful

preliminary data, since the official NASA/Space Command data is usually not

available until a few days after launch.

Sections 1.1, 1.2, and 1.3 provide an overview of the technical features of

VEC2TLE.  Section 1.1 discusses the various vector types that may be input to

compute Keplerian Elements.  Section 1.2 is a brief description of Keplerian

PAGE 2:

orbital elements.  Section 1.3 is an overview of the SGP and SGP4 propagation

models.  It should be noted, however, that VEC2TLE takes care of all of the

intricate details, requiring the user only to know the specific type of input

vector if it's manually entered and nothing if it's read automatically.

Thus, this information is just provided for the benefit of the interested

reader.

1.1		Vector Types Supported

VEC2TLE supports a variety of vector types in the computation of Keplerian

Elements.  The two basic vector types are Earth-Centered Inertial (ECI) and

Earth-Fixed Greenwich (EFG).  Both ECI and EFG represent right-handed

orthogonal Cartesian reference frames with the origin at the Earth's center

of mass.

1.1.1		Earth-Centered Inertial

The ECI coordinate frame is considered an inertial frame since its

orientation remains virtually fixed in space.  The primary axis (X-axis) is

in the equatorial plane and points in the direction of the Vernal Equinox.

The Y-axis is also in the equatorial plane, and points 90 degrees to the East

of the Vernal Equinox.  The Z-axis is perpendicular to the equatorial plane,

and points through the North Pole.

The ECI coordinate frame, in truth, is not inertial.  The Vernal Equinox

precessing very slowly due to gravitational torque's imparted on the Earth's

rotational axis by the Moon, Sun, and other planets in the solar system.

Precession is a long-term effect.  The Earth's equator/poles also do not

maintain a fixed obliquity to the ecliptic.  The Moon and Sun also impart a

short-term periodic wobble on the Earth's rotational axis called nutation.

The result is a change in the orientation of the X-, Y-, and Z-axes over

time.  Reference 1 has a detailed discussion of precession and nutation.

VEC2TLE provides two standard systems for specifying the orientation of the

ECI coordinate axes:  1) True Equator, True Equinox of Date and 2) Mean

Equator, Mean Equinox of 1950 (M50).  The former, as its name implies,

orients the X-, Y-, and Z-axes to the actual directions of the Vernal Equinox

and polar axes at the applicable time of the vector.  The latter orients the

axes to the respective directions of the equinox and pole at the beginning of

the Besselian Year 1950.

1.1.2		Earth-Fixed Greenwich (EFG)

The EFG coordinate frame rotates with the Earth.  The primary axis (E-axis)

is in the equatorial plane and is directed through the Greenwich Prime

Meridian (GPM).  The F-axis is also in the equatorial plane and points 90

degrees to the East of the GPM.  The G-axis is perpendicular to the true

equator, and points through the North Pole.  The EFG system is also known as

the True of Day Rotating (TDR).

PAGE 3:

The EFG coordinate frame is particularly useful for launch planning because

the components of the position/velocity state vectors remain fixed with

respect to the Earth's surface.  The applicable times for each state vector

are generally referenced as an offset from launch time.  Using the predicted

or actual launch time, Keplerian Elements for all orbital segments can be

computed with VEC2TLE by entering the vectors the launch time.  VEC2TLE

allows the user to reference the time since launch as Mission Elapsed Time

(MET), to which the program can automatically add the launch time and date.

1.1.3		Vector Units

VEC2TLE can compute Keplerian Elements from state vectors in any one of the

following sets of units:

- kilometers (km), kilometers per second (km/s)

- nautical miles (nm), nautical miles per second (nm/s)

- feet (ft), feet per second (ft/s)

1.2		Keplerian Orbital Elements

Keplerian orbital elements provide a geometric description of the size,

shape, orientation, and in-plane phasing of a satellite orbit.  The following

is a brief description of the Keplerian elements.  For further details, see

references 2 through 5.

The size and shape of an elliptical orbit are described by the semimajor axis

and eccentricity respectively.  These parameters also specify the orbital

perigee (closest distance to Earth) and apogee (farthest distance from

Earth).

The semimajor axis is half the longest distance across the orbital ellipse.

The size of the semimajor axis determines the orbital period, which is the

time it takes for the satellite to make one complete revolution or orbit.

The period is sometimes used instead of semimajor axis.  The mean motion,

which is the orbital frequency (the reciprocal of the period), is also

commonly used in place of semimajor axis (as is the case with Keplerian

Elements).

The eccentricity describes how much the orbit deviates from circular.  A

perfectly circular orbit has a zero eccentricity.  As the orbit becomes more

elongated, the eccentricity increases.  Eccentricity must remain less than

one for an orbit to be elliptical (remain closed).  An eccentricity value

greater than or equal to one describes an escape trajectory (parabolic/

hyperbolic).

The orientation of the orbital plane is described by the inclination and the

right ascension of the ascending node (RAAN).

PAGE 4:

The inclination describes the tilt of the orbital plane with respect to the

equatorial plane.  The inclination specifies the northern most and southern

most latitudes over which the satellite will directly pass.  An inclination

of zero degrees is an equatorial orbit; an inclination of 90 degrees is a

polar orbit.  Inclinations greater than 90 degrees describe orbits moving

against the rotational direction of the Earth (retrograde).

The ascending node of an orbit is its south to north equatorial crossing

point.  In astronomical terms, right ascension is the angle measured eastward

from the Vernal Equinox to the point of interest.  Thus the RAAN is the angle

measured eastward from the Vernal Equinox to the orbit's ascending node.

The final orientation angle is the argument of perigee.  This is the angle

measured from the ascending node to the perigee point, in the direction of

satellite motion.

The in-plane phasing is specified by the true anomaly.  This is the angle

measured from the perigee point to the position of the satellite at the epoch

time of the element set.  Since satellites in elliptical orbits do not travel

at a constant speed, true anomaly has a time domain counterpart called mean

anomaly, which describes the angle from perigee to the satellite were the

satellite in a circular orbit.  There is a standard mathematical conversion

between mean and true anomaly, based on the eccentricity of the orbit.  Mean

anomaly is often used in place of true anomaly (as is the case with Keplerian

Elements).

1.3		Orbit Propagation Models

This section provides a general discussion of the SGP and SGP4 orbit

propagation models.  A detailed discussion of the mathematics involved is

beyond the scope of this manual.

Orbital elements representing the exact physical Keplerian parameters are

referred to as "osculating" elements.  The data in Keplerian Element format

represent fictitious "mean" values for the various orbital elements.  This

was done to accommodate both speed and convenience in the process of

computing positions and velocities of the satellites.  Thus the introduction

of osculating elements for propagation in SGP or SGP4 would result in

erroneous predictions.

The procedure to compute position and velocity state vectors for any desired

time using SGP or SGP4 is well understood and detailed in reference 6.  The

basic perturbations that cause a satellite's path to deviate from an ideal

Keplerian orbit result from 1) the non-spherical mass distribution of the

Earth and 2) atmospheric drag.  SGP and SGP4 apply these perturbational

effects to orbits by the technique known as variation of parameters, where

the parameters being changed are the orbital elements.  If these effects were

ignored and the orbit were propagated using 2-body (i.e., Keplerian) orbit

theory, the error in the predictions would be obvious in the span of 2-3

hours.

PAGE 5:

1.3.1		Earth Gravitational Modeling

SGP uses a 3rd order geopotential model to describe the mass distribution of

the Earth.  This includes the equatorial bulge (2nd order) and the greater

amount of mass in the southern hemisphere (3rd order) that describes the

characteristic "pear" shape of the Earth.  SGP4 uses a 4th order geopotential

model that includes an additional mass deviation that is smaller than the

second and third order deviations.

The geopotential deviations from an "ideal" spherical mass distribution

result in predictable changes to the orbit.  The primary gravitational

perturbational effects are on the orbital plane and the orientation of the

orbit's apogee-perigee (or apsidal) line.  The primary effects are "secular"

in nature as they represent constant drift rates for the ascending node and

the apsidal line as a function of time.  The constant drift rates are a

function of the semimajor axis, eccentricity, and inclination of the orbit.

The secondary effects are periodic in nature and consist of both long- and

short-term effects.  The long term periodic terms are superposed on the

secular effects.  The short-term periodic terms, in turn, are superposed on

the long-term periodic effects.

1.3.2		Atmospheric Drag Modeling

Both SGP and SGP4 use static methods to model the effects of atmospheric drag

on satellite orbits.  SGP assumes a quadratic variation of the mean motion as

a function of time.  The quadratic coefficients are one sixth the second

derivative of mean motion with respect to time (nddot/6) for time squared and

one half the derivative of mean motion with respect to time (ndot/2) for

time.  SGP4 models the density of the Earth's upper atmosphere using the

fourth power of the orbital altitude.  SGP4 applies drag effects to the orbit

using a pseudo ballistic coefficient (Bstar), normalized to the orbital

altitude and current atmospheric density profile.  For both SGP and SGP4, the

drag coefficients (n dot/2, nddot/6 or Bstar) are usually empirically fit

(based on long-term behavior) in the orbit determination process.

PAGE 6:

2.0		INSTALLATION

The VEC2TLE software is designed to operate on an International Business

Machines (IBM) compatible Personal Computer (PC) using the Microsoft Disk

Operating System (MS-DOS) version 3.2 or higher.  The software also operates

in the MS Windows and Windows 95 environments.  The computer must be equipped

with at least 640 K or RAM and a video display compatible with one of the

following standard PC formats: CGA, Hercules, EGA, MCGA, or VGA.  The VEC2TLE

software automatically detects the display adapter format and displays at the

highest possible resolution.  A hard disk and a math coprocessor are

recommended.

The software must first be installed on either a floppy diskette or the

computer's fixed (hard) disk.  You may install VEC2TLE automatically by

placing the distribution diskette in a floppy drive and setting that floppy

as the default drive.  Automatic installation is accomplished by initiating

the INSTALL.BAT batch file by typing:

INSTALL [drive:][path]

The default drive and path for INSTALL.BAT is C:\VEC2TLE.  (You may specify

an alternate destination drive and path as shown above in the optional

command line parameters.)

The installation procedure may also be accomplished manually.  Assume an

installation from floppy disk A: to hard disk C:

Place the software floppy diskette into drive A:

Set drive C: as the default drive:

C: [ENTER]

Create a directory for VEC2TLE:

MD C:\VEC2TLE [ENTER]

Set the VEC2TLE directory as the default path:

CD \VEC2TLE [ENTER]

Copy all files to the default directory:

COPY A:[path]*.* [ENTER]

PAGE 7:

Note that the manual installation procedure may be modified to accommodate a

different destination drive letter, a different installation drive letter, or

a different VEC2TLE directory name as desired.  This is accomplished by

making the appropriate changes to the above procedure as follows:

a) DESTINATION DRIVE - substitute (for C:) the actual destination

drive letter (followed by    a colon)

b) INSTALLATION DRIVE - substitute (for A:) the actual installation

drive letter (followed by a colon)

c) PATH - substitute the actual directory names (paths) for \VEC2TLE

(or the installation path)

If a mouse is to be used with the software, it must have its driver installed

prior to initiating the VEC2TLE software.  For further information, consult

the documentation provided with your mouse.

When posted on a BBS, VEC2TLE and its associated files are compressed into a

.ZIP format file.  The files name is indicative of the version of VEC2TLE

contained within.  The file naming convention is "V2Lyyww.ZIP".  The V2L is

the prefix indicating that this is a compressed version of VEC2TLE.  the yy

digits indicate the year in which the current version was created.  The ww

digits indicate the week (1-54) in which the current version was created.

This is consistent with the version number indicated on the copyright notice

display.  For example, the current version of VEC2TLE (9425) would be found

in a file named V2L9435.ZIP when posted on a BBS.  This file naming

convention provides a convenient mechanism by which users can determine

whether or not they have the most current version of the software.

The files contained in the compressed (i.e., .ZIP) file are as follows:

INSTALL.BAT             Installation Batch File

VEC2TLE.EXE		Executable program file

VEC2TLE.CFG             Program configuration file

VEC2TLE.DOC             Software user manual

QUICKV.DOC		Quick start user instructions

VEC2TLE.ICO             Icon suitable for use with Microsoft Windows

REGISTER.FRM            Registration Form

STS-80.VIF              Sample State Vector message from STS-80 in VEC2TLE-

readable format

VEC2TLE.NEW		Changes since the previous release

CONVERT.BAT		Batch file for use with VEC2VIF

VEC2VIF.EXE		Vector to VIF conversion utility

VEC2VIF.DOC		VEC2VIF documentation

STS.VEC                 NASA GSFC OIG RAID RBBS example vectors

PAGE 8:

2.1		Upgrading From a Previous Version of VEC2TLE

Upgrading from a previous version of VEC2TLE may be accomplished by using the

standard installation procedure.  If the manual installation procedure is

used, you should always delete the old program configuration file

(VEC2TLE.CFG).  This is usually necessary because when new features are added

to the software, there are often new configuration items added as well.  If

the old configuration file is not compatible with the new software, VEC2TLE

could lock-up or experience an abnormal termination.

When VEC2TLE goes through its start-up procedure, it checks for the presence

of the configuration file.  If the file exists, it opens it and loads to

program default information.  If the configuration file does not exist,

VEC2TLE will create a new one with fresh defaults.  The program defaults may

be changed at any time by the user under the Settings menu.  The program

output settings may also be changed at any time under the File menu (Data

Output Options and the TLE, AMSAT, and Vector Output File selections).

PAGE 9:

3.0		USER INTERFACE DESCRIPTION

The user interface is designed to be both intuitive and functionally

efficient.

The intuitive characteristics of the interface include its use of many de

facto standards for PC graphic user interfaces (GUIs) as well as liberal use

of labels and comments.  Accesses to different screens flow in a consistent

and logical (e.g. top-down or left-to-right) fashion.  The "look and feel" of

the GUI is intended to follow the conventions for Microsoft Windows

applications.  The purpose is to minimize the need for the new user to refer

to this user manual or to the user instructions.

The author also recognizes that experienced users prefer to have "short cuts"

rather than be required to perform a tedious series of actions for an

operation.  Thus the functionally efficient aspect of the user interface is

that many operations can be accessed by single action "hot keys."

Other features of this interface also include the ability to access an

operation by several different methods and the ability to gracefully exit

from any operation as necessary.

Much of the user interface may be driven with a Microsoft-compatible mouse.

The left button of the mouse activates the mouse cursor at its present

position on the screen.

3.1		Menu Activation

Individual menus are identified by their title labels on the menu bar at the

top of the screen.  Each menu title label has a highlighted activation

character.  Menus may be activated by either the mouse or the keyboard.  When

a menu is activated, its functional selections appear below the menu title in

the work screen area.

To activate a menu by the mouse, place the mouse cursor over the menu title

(on the top menu bar) and press the left mouse button.

To activate a menu by the keyboard, simultaneously press the [ALT] key plus

the highlighted character's key in the desired menu title (e.g. press ALT-F

to activate the "File" menu).

3.2		Menu Item Selection

All menu items are identified by a descriptive label.  Additionally, some

menu items also have a "hot key" (listed to the right of the label).  The

menu item labels have a highlighted selection character.  One menu item is

always highlighted in reverse video as the default for selection whenever a

menu is active.  A menu item may be selected by any one of the following

methods:

PAGE 10:

Place the mouse cursor on the label of the menu item and press the left mouse

button.  OR:

Move the highlighting selection (i.e. reverse video) bar with the "up" or

"down" arrow keys to the desired menu item and press the [ENTER] key.  OR:

Press the key corresponding to the menu item label's highlighted selection

character.  OR:

Press the "hot key" associated with that menu item.  (Note that the

corresponding menu need not be active to select a menu item by the

"hot key.")

3.3		Dialog Boxes

The dialog boxes (or data input screens) are designed to facilitate data

entry, provide a degree of error checking, and in some cases prevent the

entry of erroneous data.  All dialog boxes consist of a rectangular window

with a double line border and a drop shadow.  On the top bar of the window is

a short title descriptive of the data to be input in the dialog box.

3.3.1		Data Items

Data items in any dialog box may be selected by either the keyboard or the

mouse.  The [TAB] key is the selection device to be used for the keyboard.

Pressing the [TAB] key selects the data items in sequential order.  Note that

when the last item is selected, pressing the [TAB] key will select the first

data item on the list.  When using the mouse, place the mouse cursor on the

desired data item and press the left mouse button.

3.3.1.1	Text Items

Text items may be entered or edited via the keyboard.  Text items are

identified by a rectangular input box.  Some text fields have a label that is

highlighted when the text box is selected.  When working with non-blank text

items, it is important to know whether you are in the "entry" or the "edit"

mode.

The text item is in the "entry" mode if the data presently in the input line

is highlighted (i.e., in reverse video).  (The cursor must also appear as a

blinking underscore character.)  In this mode, the old value of the text

input line will be deleted and replaced with the characters typed from the

keyboard when the first character key is pressed.  The "entry" mode may be

activated by moving the cursor to the input line by the Tab key.  You may

also enter this mode by placing the mouse cursor on the input box and

pressing the left mouse button twice in rapid succession (i.e., double-

clicking).

The text item is in the "edit" mode if the data in the input box is not

highlighted and/or the cursor appears as a blinking rectangle.  In this mode,

the data may be edited in a fashion similar to that for most word processor

software (i.e., backspace, delete, right arrow, left arrow, etc.).  This mode

PAGE 11:

may be entered by changing the shape of the mouse cursor with the [INSERT]

key, or by a single press of the left mouse button while the cursor is on the

input box.  It should be noted that the shape of the cursor indicates whether

new text will be inserted in front of the cursor or will overwrite the

existing text.  If the cursor is the blinking underscore, new text will be

inserted in front of the cursor; if the cursor is the blinking rectangle,

then new text will overwrite the existing text.  Note also that the size of

all text fields is limited and when the maximum size is reached, no further

characters may be inserted.

3.3.1.2	Buttons

Most dialog boxes have two buttons: OK and Cancel.  These buttons control the

manner in which processing is to be performed using the data from the input

screen.  When a dialog box is in a sequence, it may have a Prev button that

allows the user to return to the previous dialog box.  These buttons may be

accessed by either the keyboard or the mouse.

3.3.1.2.1 	OK Button  Activating the OK button instructs the software to

process the input screen data.  The OK button is activated by either of the

following methods:

Move the cursor (with the [TAB] key) to any item except the Cancel button and

the press the [ENTER] key.  OR:

Place the mouse cursor on the OK button and press the left mouse button.

3.3.1.2.2 	Cancel Button  Pressing the Cancel button instructs the

software to abandon the current operation.  This has the effect of ignoring

any data input.  The Cancel button is activated by any one of the following

methods:

Move the cursor (with the [TAB] key) to the Cancel button and press the

[ENTER] key.  OR:

Press the keyboard [ESCAPE] key.  OR:

Place the mouse cursor on the Cancel button and press the left mouse button.

OR:

Closing the input screen window (ALT-F3 on the keyboard or selecting the

button in the upper left corner of the window with the mouse).

3.3.1.2.3 	Prev Button  Pressing the Prev button instructs the software

to return to the previous dialog box in a sequence.  The Prev button is

activated by either of the following methods:

Move the cursor (with the [TAB] key) to the Prev button and press the [ENTER] key.

OR:

Place the mouse cursor over the Prev button and press the left mouse button.

PAGE 12:

3.3.1.3   	Radio Buttons

Radio buttons are helpful data input items in that they promote speed of data

entry and do not allow the input of "illegal" values.  A radio button is used

to select one and only one item from a set (cluster) of possible values.  A

radio button always has two or more items and one (and only one) of these

items is always selected.  Radio buttons are delimited by parenthesis ().

The selected item in a radio button cluster may be changed by the up and down arrow keys or by placing the mouse cursor on the item to be selected and pressing the left mouse button.  Note that radio buttons have labels with highlighted characters.  In such cases, an item may also be selected by pressing the key corresponding to the highlighted character.

3.3.2     	Other Features

The dialog boxes are constructed to appear at the center of the work screen

area. Should the dialog box be blocking needed data in another open window,

it is possible to move the input window out of the way with the mouse.  This

is accomplished by placing the mouse cursor on the top bar then holding the

left mouse button down while moving the window by mouse action.

Dialog boxes are "modal" -- it is not possible to have two dialog boxes open

at the same time.

3.4       	Windows

Windows provide a means of visual information input and output for VEC2TLE.

The dialog boxes and error messages fall into the general category of

windows.  As such, the information generally applies to these windows, but

more importantly applies to the data output windows displaying the computed

Keplerian Elements.

All windows can be moved around the desk top by activating the cursor on the

top bar and dragging the window.  In the upper left corner of the window is a

"close" button that will destroy the window.  The currently active window may

also be closed by an Alt-F3 hot key action as well as by selecting the Close

item from the Window menu.

The TLE output windows stack on top of each other in the desk top.  A new

window assumes its position in the desk top stack with its order being

immediately after the currently active window.  The user may move through the

stack of windows in the desk top to view previously computed data.  This is

accomplished by the Pg Up (previous window) or Pg Dn (next window) hot key

actions or by selecting the corresponding entries under the Window menu.

PAGE 13:

An output window may also be shrunk down to only a small fraction of the desk

top size by dragging the lower right corner with the mouse.  However, this

action is not generally recommended since the window will obtain the data

from the last Keplerian Element computation when the window is restored to

fit the full size of the desk top.  Restoring and/or centering the active

window to the full desk top may be accomplished by the activating the mouse

cursor on the arrow in the upper right corner of the window, by the Alt-Z hot

key action, or selecting the Zoom option from the Window menu.

3.5		Status Line Help

Context-sensitive status line help has been implemented to give a short on-

line description of the function(s) performed by each menu item.  The help

status consists of a phrase or sentence that replaces the standard status

line at the bottom of the screen when a menu is active.  The phrase is

applicable to the highlighted menu item and thus changes appropriately as the

user scrolls through the menu or moves from one menu to another.

PAGE 14:

4.0		OPERATING INSTRUCTIONS

This section provides the operating instructions for the VEC2TLE software

including any applicable background information.  Note that a detailed

explanation of each input option is not provided as it would be redundant.

Rather, the operator should either have a functional understanding of the

user interface or consult the quick start user instructions before attempting

any serious operation of the software.

These operating instructions are on the single function level.  They provide

a detailed background on each of the various menu items and their associated

dialog boxes.  Users interested in the proper sequence of menu actions

required to get VEC2TLE to perform a particular function should refer to the

quick start user instructions (see file QUICKV.DOC).

4.1       	Startup

The software is initialized by setting the Disk Operating System (DOS)

default drive and directory (path) to that where software resides.  For the

purposes of these instructions, it is assumed that the software resides on

hard drive C: in directory \VEC2TLE.  (If this assumption is incorrect, the

information below needs to be modified accordingly.)

Change to the default drive:

C: [ENTER]

Change the directory to the program-resident directory:

CD \VEC2TLE [ENTER]

Start the program:

VEC2TLE [ENTER]

Note that VEC2TLE may also be initiated from the Microsoft Windows or Windows

95 operating environment.

4.2       	File

This section provides a description of the functions listed in the menu under

the File heading.  As the name implies, these functions are concerned with

actions associated with input and output files.  Files are selected using a

list selector in a dialog box.  The desired path and file name may be entered

directly or a "wildcard" may be entered altering the names appearing in the

scrollable list box.  Alternatively, the desired file may be selected as a

PAGE 15:

pick from the list box.  A short history of wildcard entries is accessible

from the down arrow to the right of the wildcard entry area.

4.2.1     	Vector Input File

The Vector Input File function allows the user to identify a text file from

which state vectors may be read for conversion to Keplerian Elements.  The

default wildcard is *.VIF for the list box when selecting a vector input

file.  If entering the path and file name directly, the identified file must

exist.  If the file does not exist, VEC2TLE will present you with a dialog

box with the following message:

Error  Could not open file: [filename]

VEC2TLE searches the file sequentially for the next formatted state vector.

If VEC2TLE detects an unknown file format, it will present you with a dialog

box with the following message:

Error  Unknown vector format in file: [filename]

An explanation of the vector file formats appears in Appendix D.  Appendix D

also includes examples of the file headers as well as each of the three basic

formats.  There is no default vector input file.

4.2.2		TLE Input File

The TLE Input File function allows the user to identify a text file from

which TLE may be read for delta-V application.  The default wildcard is *.TLE

for the list box when selecting a vector input file.  If entering the path

and file name directly, the identified file must exist.  If the file does not

exist, VEC2TLE will present you with a dialog box with the following message:

Error  Could not open file: [filename]

4.2.3     	TLE Output File

The TLE Output File allows the user to specify the file to which TLEs are to

be written.  The default wildcard is *.TLE for the list box when selecting a

TLE output file.  If the specified output file name is entered directly and

the output file does not exist, it will be created.  If the file does exist,

any TLEs written will be output to the existing file.  If the file does not

exist and cannot be created, VEC2TLE will present you with a dialog box with

the following message:

PAGE 16:

Error  Could not open file: [filename]

When a TLE output file is selected, its path and name are saved to the

configuration file (VEC2TLE.CFG).  This path and name are used to identify

the default output TLE file, which is opened at program startup.  If no

configuration file exists, or the default configuration file is used,

OUTPUT.TLE will receive the TLE updates.

4.2.4     	AMSAT Output File

The TLE Output File allows the user to specify the file to which AMSAT format

Keplerian elements are to be written.  The default wildcard is *.AMS for the

list box when selecting a AMSAT output file.  If the specified output file

name is entered directly and the output file does not exist, it will be

created.  If the file does exist, any Keplerian elements written will be

output to the existing file.  If the file does not exist and cannot be

created, VEC2TLE will present you with a dialog box with the following

message:

Error  Could not open file: [filename]

When a AMSAT output file is selected, its path and name are saved to the

configuration file (VEC2TLE.CFG).  This path and name are used to identify

the default output AMSAT file, which is opened at program startup.  If no

configuration file exists, or the default configuration file is used,

OUTPUT.AMS will receive the AMSAT Keplerian updates.

4.2.5     	TLE Output File

The TLE Output File allows the user to specify the file to which formatted

state vectors are to be written.  The default wildcard is *.VIF for the list

box when selecting a Vector output file.  If the specified output file name

is entered directly and the output file does not exist, it will be created.

If the file does exist, any vectors written will be output to the existing

file.  If the file does not exist and cannot be created, VEC2TLE will present

you with a dialog box with the following message:

Error  Could not open file: [filename]

When a vector output file is selected, its path and name are saved to the

configuration file (VEC2TLE.CFG).  This path and name are used to identify

the default output vector file, which is opened at program startup.  If no

configuration file exists, or the default configuration file is used,

OUTPUT.VIF will receive the new formatted vector.

4.2.6		Launch Date/Time File

The launch date/time file provides VEC2TLE with a satellite-by-satellite

reference of launch dates and times.  Reference to this file is required if

vectors are to use a Mission Elapsed Time (MET) epoch reference (also called

PAGE 17:

Ground Elapsed Time [GET]) or if VEC2TLE is commanded to estimate the

revolution number of an input state vector.

Once selected, the launch date/time file path and name are automatically

saved in the configuration file (VEC2TLE.CFG).

The launch date/time file is text-based and is structurally identical to the

STSORBIT PLUS launch time/date file (STSPLUS.LTD).  Users of STSORBIT PLUS

are encouraged to use the STSPLUS.LTD file as the launch time reference.

4.2.7    	Data Output Options

The Data Output Options menu item is a toggle control that allows the user to

control the output of the computed TLEs, AMSAT Keplerian elements, and

formatted State vectors to the output file(s).  Options include replacing

existing data for a satellite, appending the new data, or not outputting the

new data.  These settings may be changed any number of times during the

program execution.  The default settings are to replace data to the [default]

output data file(s).

4.3       	Settings

The selections made under the Settings heading apply to the manual entry of

position and velocity state vectors.  These settings are format descriptors

for the state vectors and their associated times.

4.3.1     	Defaults

The defaults are the settings saved in the program configuration file

(VEC2TLE.CFG).  If this file is not found at program startup, a standard

configuration file will be created.  Any changes made under this menu item

will be saved to the configuration file once the OK button is activated or

ENTER is selected.  All configuration items are controlled by radio buttons.

The following is a list of the configuration items:

INERTIAL REFERENCE.  This setting applies only to ECI vector types and

selects the applicable inertial reference frame.  The choices are:

-- True equator, true equinox of date

-- Mean equator, mean equinox of 1950

TYPE.  Two Cartesian vector types are supported:

-- Earth-Centered Inertial (ECI)

-- Earth-Fixed Greenwich (EFG) / True of Day Rotating (TDR)

PAGE 18:

VECTOR UNITS.  Three sets of vector dimensional units are supported:

-- km, km/s

-- nm, nm/s

-- ft, ft/s

TIME FORMAT.  Two UTC date and time formats are supported:

-- YYYYDDDHHMMSS.SSS  (Year, Day, Hour, Min, Sec, Fractional Sec)

-- YYYYDDD.DDDDDDDDD  (Year, Day, Fractional Day)

Additionally, two MET date and time formats are also supported:

-- DDDDDDDHHMMSS.SSS  (Day, Hour Min, Sec, Decimal Sec)

-- DDDDDDD.DDDDDDDDD  (Day, Decimal Day)

Note that VEC2TLE allows for implicit two-digit years to be entered from 1950

to 2018 (inclusively).  If the first two digits of the year are 00-18, the

years 2000-2018 are assumed; if the first two digits are from 50-99, the

years 1950-1999 are assumed.

EPOCH CONTROL.  The epoch time of the computed Keplerian Elements may be one

of two times:

-- The time of the State Vector

-- The time of the Ascending Node (south to north equatorial

crossing) prior to the 	State Vector epoch.

Note that setting epoch of the Keplerian Elements to the time of the State

Vector is somewhat more accurate and is thus the preferred method when there

is a choice.  The magnitude of the difference between the two depends mainly

on the accuracy of the drag terms.

VECTOR TIME REFERENCE  The epoch time of input vectors may be referenced two

ways:

-- Coordinated Universal Time (UTC)

-- Mission Elapsed Time (MET)

4.3.2     	Inertial Reference

This menu item allows the user to override the default setting for inertial

reference without changing the value in the configuration file.

4.3.3     	Vector Units

This menu item allows the user to override the default setting for vector

units without changing the value in the configuration file.

PAGE 19:

4.3.4		Epoch Control

This menu item allows the user to override the default setting for the

Keplerian Elements epoch control without changing the value in the

configuration file.

4.3.5		Vector Time Reference

This menu item allows the user to override the default setting for the vector

time reference without changing the value in the configuration file.

4.4		Launch Date/Time

This menu item allows the user to set (or change) the launch date/time for

any satellite in the launch date/time reference file.  The launch date/time

is stored for each satellite in units of Julian Days and fractions of days.

Julian days are the time elapsed (in days) since 0.5 Jan 4713 BC.

4.5		Drag Multiplier

This function allows the user to manually set the drag multiplier for any

satellite in the drag reference file (VEC2TLE.DRG).  The drag multiplier is

used by the drag model to compensate for differences between the actual and

the model's "standard" spacecraft ballistic coefficient and the actual and

the model's  "standard" atmospheric density profile.

The drag multiplier is maintained/updated automatically by VEC2TLE whenever

the ndot/2 and/or Bstar drag parameters are entered with a state vector to

compute Keplerian elements.  The current multiplier is updated by exponential

smoothing with what the multiplier should have been in order for the drag

model to compute the exact drag parameter(s) which was/were entered.  The

rationale for the exponential smoothing is so that future drag estimations

will be based on the history of measured drag values.  This also allows

empirical measurements by different methods to all be assimilated into the

drag multiplier.

One method to empirically estimate the ndot/2 drag parameter is by measuring

the change in the mean motion between two sets of Keplerian elements as

follows:

ndot/2 = 0.5 * ( n(2) - n(1) ) / dt

where:

-- n(1) is the mean motion (rev/day) of one Keplerian element set

-- n(2) is the mean motion (rev/day) of the other Keplerian element

set

-- dt is the time difference (days) between the epoch times (Te) of

the two Keplerian element sets (i.e., dt = Te(2) - Te(1) ).

PAGE 20:

In order for the drag estimation method above to work properly, the two

Keplerian Elements should have a reasonable time separation (preferably 24

hours or more) and there should be no orbit-changing maneuvers occurring

between the two Keplerian Elements.

4.6       	Compute TLE

The Compute TLE menu is where VEC2TLE performs its computational function.

The vectors may be input manually or read from vector input files.  For both

cases, a reasonability check is run to try to ensure that the input state

vector represents a closed trajectory (circular, elliptical), that the apogee

of the orbit is above the surface of the Earth, and the epoch time of the

vector is not zero.  If any if these conditions are violated, an error

message is displayed and the user is given the option to edit the state

vector or cancel the operation.  Note that passing the reasonability check

does not guarantee that Keplerian elements can be computed from the input

state vector.

4.6.1     	Input State Vector

The Input State Vector menu item allows the manual input of position and

velocity state vectors and associated data in order to compute Keplerian

Elements.  The following is a summary of the fields in the input dialog box:

Satellite Name or ID          	- 12 characters (max.) free-form

Catalog Number                  - 5 digit integer USSPACECOM ID number

International Designator      	- International designator*

Position Vector Components    	- 16 Character floating point or scientific

notation

Velocity Vector Components    	- 16 Character floating point or scientific

notation

Vector Type                   	- Radio Button

Time Format                   	- Radio Button

Orbit Theory                  	- Radio Button

ndot/2                        	- 10 Character floating point or scientific

notation

nddot/6                       	- 10 Character floating point or scientific

notation

Bstar                         	- 10 Character floating point or scientific

notation

Element Set #                 	- 3 digit integer

Rev #                         	- 4 digit integer revolution number

Numeric fields are truncated when the first digit in the input character

string is not a legal part of an integer or floating point number as

applicable.  All characters after the first illegal character are ignored.

Blank fields or those beginning with an illegal character are interpreted as

zero.

Note that when drag parameters or the revolution number is assigned a value

of negative one (-1), they will be automatically estimated by VEC2TLE.  In

order for VEC2TLE to estimate a revolution number, it must have a launch time

PAGE 21:

and date (LTD) file assigned with a record for that satellite.

*The International designator has three components: year, sequential launch

of the year, and the sequential piece designator for that launch.  The year

is invariably a two-digit number; the sequential launch is invariably a

three-digit number.  The piece designator is one to three characters in

length, with legal characters being A-Z, excluding the letters "I" and "O".

The International designator is generally right-justified with a space

between the three components if the sequential piece designator is a single

letter.

4.6.2     	Read Vector File

The Read Vector File menu item searches a text file for formatted state

vectors.  This option reads forward from the current position in the file

until it encounters a vector or reaches the end of the file.  Thus with

selection of this option, VEC2TLE reads the next sequential vector from the

input vector file and otherwise processes the data with the same procedure

for Input State Vector menu item.  If no vector file has been selected, the

Vector Input File dialog box (from the File menu) will be displayed initiated

automatically to allow the user to select the vector file at that time.

If the end of file (EOF) is reached when attempting to read the state vector,

the following error message is displayed:

Error  End of File reached for: [filename]

See appendix D for vector file formats and examples.

4.7		Re-Epoch TLE at Asc Node

VEC2TLE has the ability to move the epoch time on an existing TLE to the time

of the ascending node prior to the TLE's current epoch time.  Note that this

feature is slightly less accurate than deosculating a state vector directly

at the ascending node.

The source for the Two-Line Elements is the TLE input file.  VEC2TLE reads

the TLEs from this file in a sequential fashion.  If the user wishes to skip

to a TLE later in the file, the 'Cancel' button should be activated until the

desired TLE is loaded.  If the user wishes to use a TLE earlier in the file,

the file should be re-opened under the TLE Input File option.  If no TLE

input file has been designated, the TLE Input File dialog box (from the File

menu) will be initiated automatically to allow the user to select the element

file at that time.

PAGE 22:

4.8		Impulsive Delta-V

VEC2TLE has the ability to impart an impulsive or quasi-impulsive delta-V to

a TLE, computing the appropriate changes to the TLE.  This feature

approximates the post-burn state vector and is somewhat less accurate that a

direct state vector deosculation.

The source for the Two-Line Elements is the TLE input file.  VEC2TLE reads

the TLEs from this file in a sequential fashion.  If the user wishes to skip

to a TLE later in the file, the 'Cancel' button should be activated until the

desired TLE is loaded.  If the user wishes to use a TLE earlier in the file,

the file should be re-opened under the TLE Input File option.  If no TLE

input file has been designated, the TLE Input File dialog box (from the File

menu) will be initiated automatically to allow the user to select the element

file at that time.

VEC2TLE performs some error detection and/or correction on the TLE.  VEC2TLE

cannot correct for: negative or zero mean motion, eccentricity of one or

more, or date/time errors in the TLE.

Note that the element number and the revolution number will be automatically

incremented by VEC2TLE while it is computing the post-burn TLE.

The delta-V burn data is entered in a dialog box following the TLE input

dialog.  This data must be entered manually.  The user may return from this

dialog box to the TLE dialog box by the Prev button.

VEC2TLE will perform an impulsive (instantaneous) delta-V at the burn ignition time if the burn duration, initial mass, and final mass are all zero.  Otherwise, a quasi-impulsive burn is assumed.  The difference between an impulsive and quasi-impulsive delta-V is the effective time of the delta-V application.  The quasi-impulsive delta-V applies an impulsive burn at the middle time of the burn.

If the delta-V is null (the three components are zero), VEC2TLE will compute

a new TLE representative of the orbit at the effective time of the delta-V

application.  This feature is useful when the epoch on an element set needs

to be moved to a different time.

When a quasi-impulsive delta-V is requested, the mass/weight units of the

spacecraft and thruster propellant flow rate are at the discretion of the

user; the only restriction is that they must be common units.  The total mass

of the spacecraft must be greater than the consumed propellant mass (initial

mass must be greater than the product of the burn duration and the propellant

consumption rate).

VEC2TLE cannot correct for the following in the delta-V data: date/time

errors, propellant consumption exceeding spacecraft mass, or delta-V placing

the spacecraft in an escape trajectory.

PAGE 23:

4.9		Compute Date

This menu item is a date computation utility that is an on-line complement to

Appendix B of this User Manual.  This function computes the month and day

corresponding to a date input in year/day-of-year format.  (The year/day-of-

year format is standard for space operations.)  An abbreviation for the day

of the week (Sun, Mon, Tue, etc.) is also provided.

4.10		Compute Day of Year

This menu item is a day-of year computation utility that is an on-line

complement to Appendix B of this User Manual.  This function computes the

sequential day-of year (1-366) corresponding to a date input in MM-DD-YY

format.  (The year/day-of-year format is standard for space operations.)  An

abbreviation for the day of the week (Sun, Mon, Tue, etc.) is also provided.

4.11		Program Termination

The program may be exited by either menu selection or by "hot key."

To terminate the program by menu selection, activate the "Exit" menu item in

the "File" menu.

To exit the program by "hot key," depress ALT-X.

PAGE 24:

5.0		REFERENCES

1.	Spherical Astronomy, Robin M. Green, Cambridge University Press,

1985.

2.	Fundamentals of Astrodynamics, Roger R. Bate et al, Dover

Publications, Inc., 1971.

3.	Orbital Mechanics, Vladimir A. Chobotov, American Institute of

Aeronautics and Astronautics, 1991.

4.	Orbital Motion, Third Edition, A.E. Roy, Adam Hilger, 1988.

5.	Methods of Orbit Determination, Pedro Ramon Escobal, Robert E.

Krieger Publishing Company, 1976.

6.	PROJECT SPACE TRACK, Models for Propagation of NORAD Element Sets,

Felix R. Hoots  and Ronald L. Roehrich, Spacetrack Report No. 3,

ADC/DO6, December 1980.

7.    Computertalk Space Object Catalog, Joel Runes, 1992.

8.	Vector to Two-Line Elements (VEC2TLE) Orbit Computation Software, K.

Ernandes, The AMSAT Journal, Volume 17, No. 2, pp. 26-29, March/April

1994.

PAGE 25:

6.0		ACKNOWLEDGMENTS

The author would like to thank Dave Ransom (Sysop RPV Astronomy BBS and

author of STSPLUS) and Joel Runes (Journalist and Computer Consultant) for

their encouragement and enthusiasm during the initial creation of VEC2TLE.

Without their timely and constructive comments, VEC2TLE would be a less

useful program.

Both Dave and Joel also provided valuable testing and validation support

during the STS-55, STS-56, and STS-57 missions as well as key contributions

to this User Manual.

The author would also like thank Richard Rhodes, whose feedback led directly

to the creation of the quick start user instructions.  The ensuing "thread"

in the CompuServe HamNet forum led to some other valuable feedback from other

users.  In the end, the author hopes to have made VEC2TLE easier for the non-

aerospace engineer to use (but additional constructive comments are always

welcome).

PAGE 26:

7.0		LICENSE STATEMENT

This software is protected by copyright law of the United States of America.

This software is licensed as "Shareware".  This software may be evaluated for

a period of up to thirty (30) days.  If you wish to continue using this

software after the initial 30 day trial period you must register.  You may

not modify, decompile, disassemble, or reverse-engineer the software, and you

may not remove or obscure the copyright notices.   Each registered copy of

this software may be used on only one computer or one computer network at a

time.

7.1		Registration

Registration of this software is accomplished by completing the registration

form (REGISTER.FRM) and remitting the $10.00 (U.S.) registration fee as

applicable in this section.  The registration fee is applicable and required

for all professional and commercial users, unless the software has been

purchased from AMSAT-NA.  For the purposes of this section, a professional or

commercial user is defined as an individual or organization who uses the data

generated by the software in the conduct, support, or pursuit of their

profession or business.

Distribution of VEC2TLE by an Electronic BBS or by other commercial shareware

distribution methods is not considered professional or commercial use of this

software.  Distributions of VEC2TLE-generated data (i.e., Keplerian Elements)

are considered professional or commercial use of this software.  Registration

of a previous release of VEC2TLE for professional use does not constitute

registration of any future releases.

The registration fee is not required (but would be appreciated) for those

using the software in support of a hobby.  For the purposes of this section,

a hobby is considered to be an activity or pursuit for entertainment or

relaxation purposes that is not supporting professional or commercial

activities.

Users who remit the $10.00 registration fee to the author (or who purchase

VEC2TLE from AMSAT-NA) shall be licensed to use the software for professional

and/or commercial purposes.  Those licensed to use the software for

professional and/or commercial purposes shall receive a distribution diskette

from the author or AMSAT-NA, as applicable.  This diskette contains all of

the files for the most current version of VEC2TLE.  Additionally, the

distribution diskette also contains a licensed copy of the V2KEP program,

which is a command-line version of VEC2TLE.  V2KEP may be used for quick

vector file conversions from the MS-DOS command line and is also suitable for

use within batch files.

Users who remit $20.00 shall receive a license to use the software for

professional and/or commercial purposes as well as a printed version of the

user manual and quick start user instructions  found in the VEC2TLE.DOC and

QUICKV.DOC files (included with all distributions of the software).

PAGE 27:

7.2		Distribution

The VEC2TLE software may be posted on Electronic BBS or otherwise distributed

(by CD-ROM or other media) to individuals interested in its capabilities.

This distribution is to provide individuals with the opportunity to evaluate

its capabilities under the above shareware restrictions.  When this software

is distributed, all original distribution files must be included.  The

following is a complete list of the original distribution files:

INSTALL.BAT             Installation Batch File

VEC2TLE.EXE		Executable program file

VEC2TLE.CFG             Program configuration file

VEC2TLE.DOC             Software user manual

QUICKV.DOC		Quick start user instructions

VEC2TLE.ICO             Icon suitable for use with Microsoft Windows

REGISTER.FRM            Registration Form

STS-80.VIF              Sample State Vector message from STS-80 in VEC2TLE-

readable format

VEC2TLE.NEW		Changes since the previous release

CONVERT.BAT		Batch file for use with VEC2VIF

VEC2VIF.EXE		Vector to VIF conversion utility

VEC2VIF.DOC		VEC2VIF documentation

STS.VEC                 NASA GSFC OIG RAID RBBS example vectors

Except as stated in this section, you may not otherwise distribute, lend,

transfer, rent, lease, sub-license, or time-share the software, diskettes, or

documentation.

The V2KEP software provided to professional users is not shareware and

therefore shall not be posted or otherwise distributed along with VEC2TLE

without permission in writing from the publisher.  All other distribution of

V2KEP is a violation of the software copyright and is expressly forbidden.

7.3		Limited Warranty

The publisher (Kenneth J. Ernandes) warrants the physical diskette(s) and

documentation provided to registered users (as applicable) to be free of

defects in materials and workmanship for a period of ninety days (90) from

the registration date.  If the publisher is notified of defective materials

or workmanship during the warranty period, and such notification is

determined to be valid by the publisher, the publisher shall replace the

defective diskette(s) or documentation.

The entire and exclusive liability and remedy for breach of this Limited

Warranty shall be limited to replacement of defective diskette(s) or

documentation and shall not include or

PAGE 28:

extend to any claim for or right to recover any other damages, including but

not limited to, loss of profit, data, or use of the software, or special,

incidental, or consequential damages or other similar claims, even if the

publisher has been specifically advised of the possibility such damages.  In

no event will the publisher's liability for any damages to you or any other

party ever exceed the registration fee paid for the license to use the

software, regardless of any form of the claim.

KENNETH J. ERNANDES SPECIFICALLY DISCLAIMS ALL OTHER WARRANTIES, EXPRESSED OR

IMPLIED, INCLUDING BUT NOT LIMITED TO, ANY IMPLIED WARRANTY OF

MERCHANTABILITY OR SUITABILITY FOR A PARTICULAR PURPOSE.  Specifically, the

publisher makes no representation or warranty that the software is suitable

for any particular purpose and any implied warranty that the software is

suitable for any particular purpose and any implied warranty of

merchantability is limited to the ninety-day duration of the Limited Warranty

covering the physical diskette(s) and documentation only (and not the

software) and is otherwise expressly and specifically disclaimed.

This limited warranty gives you specific legal rights; you may have others

that vary from state to state.  Some states do not allow the exclusion of

incidental damages or consequential damages, or the limitation on how long an

implied warranty lasts, so some of the above restrictions may not apply to

you.

7.4		Governing Law and General Provisions

This license statement shall be construed, interpreted, and governed by the

laws of the State of New York.  If any provision of this statement is found

void, invalid, or unenforceable, it will not affect the validity of the

balance of this statement, which shall remain valid and enforceable according

to its terms.  If any remedy provided is determined to have failed of its

essential purpose, all limitations of liability and exclusion of damages set

forth in the Limited Warranty shall remain in full force and effect.  The

terms of this statement may only be modified in writing signed by you and an

authorized officer of the publisher.  All rights not specifically granted in

this statement are reserved by Kenneth J. Ernandes.

PAGE 29:

APPENDIX A -- HOT KEY SUMMARY

Menu Selection:

F10					Menu Bar Selection

Alt-space				=

Alt-F					File

Alt-S					Settings

Alt-C					Compute TLE

Alt-W					Window

Menu Item Selection:

=

About...				N/A

File

Vector Input File			Alt-V

TLE Input File                          Alt-E

TLE Output File                         Alt-T

AMSAT Output File			Alt-M

Vector Output File                      Alt-P

Launch Date/Time File                   Alt-L

Data Output Options                     Alt-O

Exit					Alt-X

Settings

Defaults				F2

Inertial Reference                      F3

Vector Units                            F4

Epoch Control                           F5

Vector Time Ref.			F6

Launch Date/Time			F7

Drag Multiplier                         F8

Compute TLE

Input State Vector                      Alt-I

Read Vector File			Alt-R

Re-Epoch TLE at Asc Node                ALT-N

Impulsive Delta-V			Alt-D

Compute Date                            Alt-A

PAGE 30:

Compute Day of Year                     Alt-Y

Window

Close					Alt-F3

Next					PgDn

Previous				PgUp

Zoom					Alt-Z

PAGE 31:

APPENDIX B -- SEQUENTIAL DAY OF THE YEAR

The tables in this appendix represent the sequential day of the year

corresponding to any date for both conventional years and leap years.  The

year and sequential day of the year is the most common date format used for

orbital applications.  The following tables provide a look-up reference for

conversion.

Table B.1  Day of the Year Calendar -- Non-Leap Years

Day   Jan   Feb   Mar   Apr   May   Jun   Jul   Aug   Sep   Oct   Nov   Dec

1    001   032   060   091   121   152   182   213   244   274   305   335

2    002   033   061   092   122   153   183   214   245   275   306   336

3    003   034   062   093   123   154   184   215   246   276   307   337

4    004   035   063   094   124   155   185   216   247   277   308   338

5    005   036   064   095   125   156   186   217   248   278   309   339

6    006   037   065   096   126   157   187   218   249   279   310   340

7    007   038   066   097   127   158   188   219   250   280   311   341

8    008   039   067   098   128   159   189   220   251   281   312   342

9    009   040   068   099   129   160   190   221   252   282   313   343

10    010   041   069   100   130   161   191   222   253   283   314   344

11    011   042   070   101   131   162   192   223   254   284   315   345

12    012   043   071   102   132   163   193   224   255   285   316   346

13    013   044   072   103   133   164   194   225   256   286   317   347

14    014   045   073   104   134   165   195   226   257   287   318   348

15    015   046   074   105   135   166   196   227   258   288   319   349

16    016   047   075   106   136   167   197   228   259   289   320   350

17    017   048   076   107   137   168   198   229   260   290   321   351

18    018   049   077   108   138   169   199   230   261   291   322   352

19    019   050   078   109   139   170   200   231   262   292   323   353

20    020   051   079   110   140   171   201   232   263   293   324   354

21    021   052   080   111   141   172   202   233   264   294   325   355

22    022   053   081   112   142   173   203   234   265   295   326   356

23    023   054   082   113   143   174   204   235   266   296   327   357

24    024   055   083   114   144   175   205   236   267   297   328   358

25    025   056   084   115   145   176   206   237   268   298   329   359

26    026   057   085   116   146   177   207   238   269   299   330   360

27    027   058   086   117   147   178   208   239   270   300   331   361

28    028   059   087   118   148   179   209   240   271   301   332   362

29    029         088   119   149   180   210   241   272   302   333   363

30    030         089   120   150   181   211   242   273   303   334   364

31    031         090         151         212   243         304         365

PAGE 32:

Table B.2  Day of the Year Calendar -- Leap Years

Day   Jan   Feb   Mar   Apr   May   Jun   Jul   Aug   Sep   Oct   Nov   Dec

1    001   032   061   092   122   153   183   214   245   275   306   336

2    002   033   062   093   123   154   184   215   246   276   307   337

3    003   034   063   094   124   155   185   216   247   277   308   338

4    004   035   064   095   125   156   186   217   248   278   309   339

5    005   036   065   096   126   157   187   218   249   279   310   340

6    006   037   064   097   127   158   188   219   250   280   311   341

7    007   038   067   098   128   159   189   220   251   281   312   342

8    008   039   068   099   129   160   190   221   252   282   313   343

9    009   040   069   100   130   161   191   222   253   283   314   344

10    010   041   070   101   131   162   192   223   254   284   315   345

11    011   042   071   102   132   163   193   224   255   285   316   346

12    012   043   072   103   133   164   194   225   256   286   317   347

13    013   044   073   104   134   165   195   226   257   287   318   348

14    014   045   074   105   135   166   196   227   258   288   319   349

15    015   046   075   106   136   167   197   228   259   289   320   350

16    016   047   076   107   137   168   198   229   260   290   321   351

17    017   048   077   108   138   169   199   230   261   291   322   352

18    018   049   078   109   139   170   200   231   262   292   323   353

19    019   050   079   110   140   171   201   232   263   293   324   354

20    020   051   080   111   141   172   202   233   264   294   325   355

21    021   052   081   112   142   173   203   234   265   295   326   356

22    022   053   082   113   143   174   204   235   266   296   327   357

23    023   054   083   114   144   175   205   236   267   297   328   358

24    024   055   084   115   145   176   206   237   268   298   329   359

25    025   056   085   116   146   177   207   238   269   299   330   360

26    026   057   086   117   147   178   208   239   270   300   331   361

27    027   058   087   118   148   179   209   240   271   301   332   362

28    028   059   088   119   149   180   210   241   272   302   333   363

29    029   060   089   120   150   181   211   242   273   303   334   364

30    030         090   121   151   182   212   243   274   304   335   365

31    031         091         152         213   244         305         366

PAGE 33:

APPENDIX C  -- ACRONYMS AND ABBREVIATIONS

ASCII          	American Standard Code for Information Interchange

AMSAT           Radio Amateur Satellite Corporation

AMSAT-NA        AMSAT North America

BBS            	Bulletin Board Service

CD ROM		Compact Disk Read Only Memory

CGA             Color Graphics Adapter

ECI            	Earth-Centered Inertial

EFG            	Earth-Fixed Greenwich (see also TDR)

EGA             Enhanced Graphics Adapter

E-Mail		Electronic Mail

EOF            	End Of File

ft              feet

FTP             File Transfer Protocol

ft/s            feet per second

GET             Ground Elapsed Time (see also MET)

GSFC            Goddard Space Flight Center

GPM             Greenwich Prime Meridian

GUI             Graphic User Interface

http            Hyper Text Transfer Protocol

IBM             International Business Machines

J2000           Equinox and Equator of Epoch 2000

km              kilometers

km/s            kilometers per second

M50            	Mean Equator, Mean Equinox of 1950

MET             Mission Elapsed Time (see also GET)

MS-DOS		Microsoft Disk Operating System

NASA            National Aeronautics and Space Administration

nm              nautical miles

nm/s            nautical miles per second

NORAD           North American Aerospace Defense Command

PAGE 34:

NPOE            Numerical Prediction of Orbital Events

OIG            	Orbital Information Group

PC              Personal Computer

RAAN           	Right Ascension of the Ascending Node

RAID           	Reports and Information Dissemination

RBBS           	Remote Bulletin Board System

RPV BBS        	Rancho Palos Verdes Astronomy BBS

SGP             Simplified General Perturbations

SGP4            Simplified General Perturbations Version 4

STS            	Space Transportation System

STSPLUS        	STS Orbit Plus

TDR            	True of Day Rotating (see also EFG)

TLE             Two-Line Elements

URL             Uniform Resource Locator

USSPACECOM      United States Space Command

UTC            	Coordinated Universal Time

VEC2TLE		Vector to Two-Line Elements

VGA             Video Graphics Array

VIF             Vector Input Format

WWW             World-Wide Web

PAGE 35:

APPENDIX D -- VECTOR FILE FORMATS

This section outlines the vector file formats accepted by VEC2TLE.  It should

be noted that these formats are the same formats output by Dave Ransom's

STSPLUS software.  The formats were coordinated with Dave and were made such

that they were generalized without impacting VEC2TLE's ability to read an

STSPLUS state vector.

The vectors read in by VEC2TLE are in standard PC text format or American

Standard Code for Information Interchange (ASCII).  The specification for

these files is not unnecessarily rigid since VEC2TLE is intended to be

reasonably flexible in terms of ability to read vector files.

D.1		Header Line

The header line defines the parameter definitions of the vector.  This is the

input vector file's counterpart to the configuration parameters. VEC2TLE

searches for an integer value following the string "Vector format = ".  Once

a digit is detected, up to the next three characters are read until a non-

digit character is read.  VEC2TLE forms up to a 4-digit integer that

specifies the parameters applicable to all vectors that follow in the file.

Note that leading zeros are implied and need not be included.

The significance of each digit is listed below:

TEN THOUSANDS DIGIT	- VECTOR TIME REFERENCE

0	Coordinated Universal Time (UTC)

1	Mission Elapsed Time (MET)

THOUSANDS DIGIT     	- INERTIAL REFERENCE

0     True equator, true equinox of date or ECI

1     Mean equator, mean equinox of 1950

HUNDREDS DIGIT      	- VECTOR TYPE

0     Earth-Centered Inertial (ECI/XYZ)

1     Earth-Fixed Greenwich (EFG)

PAGE 36:

TENS DIGIT          	- VECTOR UNITS

0     km, km/s

1     ft, ft/s

2     nm, nm/s

ONES DIGIT          	- DATA MODE FORMAT

4     Labeled data, one item per line

5     Two-Line space delimited data

6     Single-line comma delimited data

7     Labeled data, one item per line

Note that a '4' or a '7' for the data mode format indicates the same vector

file format.  This was done to accommodate vector data output from STSPLUS.

(Data mode '4' in STSPLUS is identical to data mode '7' except that state

vectors are only written to the output file immediately after the satellite

has crossed the ascending node.)

D.2		Vector Formats

This section specifies the three vector formats that VEC2TLE is capable of

interpreting.  Each format has an example, provided courtesy of Dave Ransom,

using True equator/equinox ECI vectors in units of ft and ft/s.  All vector

formats have a date/time format with a two-digit year and decimal day.

D.2.1		Two-Line Space Delimited

The two line space delimited format has the following data on the first line:

satellite catalog number, vector date/time format, and the three components

of the position vector.  The second line has the three components of the

velocity vector.  Each line may be up to 80 characters long.

The example below has two state vectors in it.  Note that some of the spaces

have been removed in the interest of easily fitting the data within the

margins.

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 15

20580 93110.045081 13656864.66720 17514322.54968 -5452252.42794

-16168.27686974290  15789.75251859515  10248.33566657315

20580 93110.045139 13575822.39276 17593013.13238 -5400930.14914

-16248.48336702945  15686.35334359047  10280.38786725583

D.2.2		Single-Line Comma Delimited

The single-line comma delimited format has data in the following order: epoch

flag (single digit -- ignored), units flag (single digit -- ignored), date/

time, the three components of the position vector, and the three components

of the velocity vector.

PAGE 37:

Each vector line may be up to 130 characters long.

The example below has two state vectors in it.  Note that the vectors have

been placed on multiple lines in the interest of easily fitting the data

within the margins.

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 16

0,1,20580,93110.0482060185,8771281.06140276,20969911.6226162,

-2491608.1429704,-19762.0576284838,9623.02409449012,

11524.6952898439

0,1,20580,93110.0482638889,8672341.7437806,21017717.5456987,

-2433947.77946384,-19813.4647395816,9499.28476766938,

11539.2506632381

D.2.3		Labeled Data, One Item Per Line

The labeled data, one item per line format has data in the following order:

satellite name or ID, satellite catalog number and International designator,

date/time, descriptive date time (ignored), the components of the position

vector, the components of the velocity vector, ndot/2, nddot/6, and Bstar.

Each line may be up to 80 characters long, but characters with ASCII codes

outside the range 21-127 are ignored.  VEC2TLE will also ignore any data in

columns 1-24 on the satellite name or ID line and will also ignore data in

columns 1-17 on the remaining lines.  The International designator must be in

column 32, following the satellite catalog number.

The example below has one state vector in it.

Vector format = 1017

Satellite Name:         STS-57

Catalog Number:         22684  93 37  A

Epoch Date/Time:        93180.63736111111

06/29/1993  15:17:47.000 UTC

ECI X:                     8024498.723185 ft

M50 Y:                   -19652913.243254 ft

Z:                     6620097.066346 ft

Xdot:                     20418.75391 ft/s

Ydot:                     11591.42188 ft/s

Zdot:                      9232.20312 ft/s

ndot/2 (drag):              0.00002834000 rev/day^2

nddt/6:                       0.00000E+00 rev/day^3

Bstar:                        5.17270E-05 1/Earth Radii

Elset #:                               71

Rev @ Epoch:              126.10967365930

Note that the Labeled Data, One Item Per Line format requires rigid labels

for VEC2TLE to recognize it as an existing vector in an output file.  Put

simply, the labels listed above (with a few exceptions) must be present in

the "old" vector before VEC2TLE will over-write it.  The exceptions are on

the position velocity vector components where the EFG labels and the true

PAGE 38:

equator and equinox of date labels are also acceptable.  If VEC2TLE cannot

recognize the format of the "old" vector, the "new" vector will be appended

to the file and the "old" vector will remain.

PAGE 39:

APPENDIX E -- SOURCES OF STATE VECTORS

This section provides sources of state vector information.  The order in

which the sources are presented was selected by the author's subjective

assessment of usefulness in terms of: ease of use of the data, general

accessibility, and the timeliness and frequency of updates.

E.1		NASA STSTLE Mailing List

NASA Spacelink (see E.3 below) at the Marshall Space Flight Center,

Huntsville, AL has set up the Space Transportation System Two Line Elements

(STSTLE) Internet mailing list.  The purpose of this list is to provide NASA

agencies and interested persons or organizations from the general public with

current Space Shuttle orbital information.  The data generally includes a

NASA State Vector and the corresponding AMSAT and Two-Line Keplerian elements

(as computed by VEC2TLE).  The data is typically updated three (3) times per

day during active missions and is provided to all subscribers automatically

by an Internet Electronic Mail (E-Mail) message.  Those considering

subscribing to this service should consider that they will typically receive

three E-Mail updates for the Shuttle Orbiter (more during periods of

significant orbital changes) and also updates for any other free-flying

satellites associated with that Shuttle mission.  If these many updates are

undesirable, you should consider getting updates from another source.

While the STSTLE mailing list already provides the output products of the

orbital State Vector (the AMSAT and Two-Line Keplerian elements), VEC2TLE

still provides the user a utility service.  The STSTLE mail message can be

copied directly to a text file with no need for the user to do any editing.

VEC2TLE can then read the mail message from the text file, find the State

Vector, compute the Keplerian elements, and automatically update the user's

Keplerian elements file(s).  This process is accomplished quickly and

efficiently with only a few keyboard and/or mouse actions.

To subscribe to STSTLE, send an E-Mail message to the following:

listproc@spacelink.msfc.nasa.gov

The text of the E-Mail message should read as follows:

subscribe STSTLE "Your Name"

The "Your Name" entry should actually be your name and not your E-Mail

address.  Shortly after receipt of your message, the list manager will add

your return E-Mail address and name to the list of subscribers.

PAGE 40:

E.2 		AMSAT World Wide Web Page

The Radio Amateur Satellite Corporation (AMSAT) of North America (AMSAT-NA)

maintains Space Shuttle Orbital Data on its World Wide Web (WWW) page.  Note

that the updates to the AMSAT-NA Shuttle Orbital Data are coordinated with

the STSTLE mailing list, so the updates are nearly coincident.  The data is

separated in different files by State Vector, AMSAT Keplerian Elements, and

Two-Line Elements.  VEC2TLE users desiring automatic update of Keplerian

files will find the State Vector links most useful.

The hypertext transfer protocol (http) Uniform Resource Locator (URL) for the

AMSAT-NA web page is:

http://www.amsat.org/

The URL for direct connection to the AMSAT Shuttle Orbital Data is:

http://www.amsat.org/amsat/sarex/orbit.html

Pre-launch time-independent nominal state vectors are usually also available

for the next Space Shuttle mission on the AMSAT web page.  These are

available from the State Vector Tutorial that may be accessed by a link from

the Shuttle Orbital Data page or directly at the following URL:

http://www.amsat.org/amsat/sarex/vectors.html

The AMSAT Web page is a complementary data resource to the STSTLE mailing

list (E.1).  The advantage of the AMSAT Shuttle Orbital Data is that the data

is available on a "by request" basis (when you access the web page) rather

than receiving potentially unwanted updates from a mailing list.

E.3		NASA Spacelink

NASA Spacelink maintains a link to the most current STSTLE orbital data

message on its Web page during Space Shuttle missions.  Therefore this data

is usually current and has updates on slightly lagging the STSTLE mailing

list updates.  Coupled with the large volume of interesting and useful data

provided, NASA Spacelink is, overall, an outstanding resource for

information.  The URL for the NASA Spacelink web page is:

http://spacelink.msfc.nasa.gov/

The link to the STSTLE orbital data changes with each launch because of

mission numbering.  However, this data may be easily found by selecting the

Hot Topics link from the home page, then selecting the Current Shuttle

Mission STS-XX link (where XX = current mission number), and finally

selecting the STS-XX.Keplerian.Elements link.

PAGE 41:

Note that in addition to the WWW access, NASA Spacelink may also be accessed

by Internet File Transfer Protocol (FTP) and by Telnet.

E.4		NASA GSFC OIG RAID-RBBS

Beginning with STS-61, real time State Vectors are posted on the NASA Goddard

Space Flight Center (GSFC) Orbital Information Group (OIG) Reports and

Information Dissemination (RAID) Remote BBS (RBBS) (301) 262-6784 [and Telnet

oig1.gsfc.nasa.gov -- UserID: oig  Password goddard1].  These vectors are

posted hourly, 24 hours per day, while the Shuttle is on orbit.  The vectors

are in the ECI True Equator, True Equinox coordinate system in units of km,

km/s.  The data is typically current within two (2) hours.

These vectors are not formatted for direct read by VEC2TLE.  However, Curtis

Haase has written a program called Vector to Vector Input Format (VEC2VIF), a

freeware program distributed with VEC2TLE.  VEC2VIF reads the vector text

files posted on NASA OIG RAID RBBS and re-formats the data into the common

VIF format interpretable by VEC2TLE.  VEC2VIF has several operating modes,

but default operation will convert the newest state vector in the file to VIF

format.

E.5		RPV BBS

Dave Ransom's Rancho Palos Verdes (RPV) Astronomy BBS (310) 541-7299 is a

source of pre-launch nominal EFG vectors for Space Shuttle missions as well

as current Two-Line Keplerian elements.  Note also that RPV BBS is often the

first on-line source to post late breaking news regarding astronomy and the

space program.  Unfortunately RPV BBS lacks of Internet access -- most users

need to pay long-distance phone charges.  Dave is correcting this situation

by setting up a separate FTP service.

E.6		NASA Shuttle Web

The NASA Shuttle Web page maintains reasonably current orbital data during

missions.  When coupled with the large volume of other useful and interesting

data, the NASA Shuttle Web page is certainly an excellent source for

browsing.  The URL for this page is:

http://shuttle.nasa.gov/

The URL for the State Vector and Keplerian data is:

http://shuttle.nasa.gov/current/orbit/orbitersighting/shuttle.html

PAGE 42:

The orbital data is usually updated once per day.  This data is not formatted

for read by VEC2TLE.  Therefore the vector data must be manually entered if

Keplerian elements are to be computed from the vectors.

E.7		NASA Television

NASA Television is a real-time source for Space Shuttle maneuver burn data

(the state vectors are not particularly legible).  NASA Television is

available by direct downlink from Spacenet 2 (located 69 degrees West),

Transponder 5H (channel 9, horizontal polarization) and on some cable TV

systems.  Some local cable TV stations also provide NASA TV.  The audio (and

in some cases video) portion of NASA Television is rebroadcast by many HAM

radio clubs through repeaters on VHF and UHF frequencies.  For a listing of

known repeater locations and frequencies, see file STSAUDIO.ZIP on the RPV

BBS.

The burn data is useful in that it supplies the changes to the Shuttle's velocity vector at the burn time.  If you supply VEC2TLE with the Shuttle's most current TLE and burn data, you can estimate the post-maneuver Keplerian Element using the Impulsive Delta-V function.

E.8		Other Orbit Propagators

VEC2TLE may be used to convert the elements of other orbit propagators to

Keplerian Elements. One such propagator is David Eagle's Numerical Prediction

of Orbital Events (NPOE).  NPOE can propagate state vectors forward with a

high degree of precision.  Those positions and velocities, in turn, may be

input to VEC2TLE in order to compute Keplerian Elements representative of the

other propagator's orbital elements.

PAGE 43:

APPENDIX F -- GENERAL INFORMATION

This section is used for information pertaining to general topics  related to

VEC2TLE.

F.1		Suggested Uses for VEC2TLE

[This section was provided courtesy of Joel Runes.]

While the normal use of VEC2TLE is to generate Keplerian Elements from State

Vectors as outlined throughout this document, there are a number of

applications of VEC2TLE that may not be immediately obvious.  This section

discusses some of those potential applications, but this discussion should be

considered representative rather than exhaustive.  Uses of the program are

constrained by the imagination of the user more than by the limitations of

the software.

One use of VEC2TLE is to explore the effect of an orbit adjust maneuver on

the orbit of a spacecraft.  If the user has Keplerian Elements for the object

from before the orbit adjust maneuver, an along track, cross track, and

radial delta-velocity may be applied to change the Keplerian Element

parameters or STSPLUS may be used to generate a state vector at the time of

the maneuver.  Given the latter case, adjusting the values of Xdot, Ydot, and

Zdot to reflect the velocity adjustments associated with the maneuver,

VEC2TLE can be used to generate post-burn Keplerian Elements.  To compare the

pre-burn trajectory with the post-burn trajectory, simply use the pre-burn

and post-burn Keplerian Elements in a prediction program such as STSPLUS.

This application has been successfully demonstrated during Shuttle missions

STS-56 and STS-55.  The result was useful post-burn Keplerian Elements prior

to availability of post-burn Keplerian Elements from Space Command.

Another use of VEC2TLE is to explore the effect on a satellite's orbit due to

minor dispersions in velocity and position during the launch phase.  When a

spacecraft such as the Space Shuttle is launched, the desired post-launch

state vector is used by the spacecraft's and/or launch vehicle's guidance

software.  But the orbit achieved may vary from the planned orbit.  What

effect does that variation have on the orbit?  VEC2TLE is a valuable tool for

the exploration of this question.  Suppose that the desired post-launch state

vector is known (or assumed), and that the position and time of the state

vector are held constant.  Suppose further that the achieved velocity in each

axis varies by � 0.2 ft/sec.  VEC2TLE can generate a number of Keplerian

Elements, first for the nominal trajectory, then for trajectories where the

velocity components are adjusted up and down by the 0.2 ft/sec dispersion

assumption.  The resulting Keplerian Elements may be used with prediction

programs such STSPLUS to show the effect of the injection errors on the orbit

achieved and the satellite position at any time thereafter.

Another use of VEC2TLE is to explore the required velocity change to put a

satellite into an orbit different from its initial orbit.  There a large

number of situations that can be examined with the aid of VEC2TLE along these

PAGE 44:

lines, but let's consider a very simple situation.  A spacecraft is in an

orbit with a very low eccentricity -- approximately a circular orbit.  We

desire to raise the orbit's apogee by 500 km.  First we construct Keplerian

Elements that exhibit the low eccentricity and initial altitude.  The process

of constructing the initial Keplerian Elements may be simplified by using the

tables from Part 0 of the Computertalk Space Object Catalog (reference 7 --

may be found on CompuServe Astronomy Forum and many of the BBSs listed in

Appendix F, files NC00.ZIP - NC13.ZIP).  The next step is to generate a state

vector for the time when the apogee raising maneuver is to be performed (this

can be accomplished using STSPLUS).  In VEC2TLE, apply the posigrade burn by

incrementing the magnitudes of Xdot, Ydot, and Zdot by the same percentage

and maintaining the same sign for each of the velocity components.  (An

excellent first guess would be to compute the two-body elliptical velocity

using the standard formula, which may be found in references 3-5.)  VEC2TLE

can now create a new set of Keplerian Elements that will have a perigee of

the original orbital altitude and an apogee that is greater according to the

percentage increase of the velocity components applied.  Using either the

Computertalk Space Object Catalog or displays from a prediction program, you

can determine whether the apogee was raised by more or less than the desired

500 km.  Return to the original state vector and adjust the percentage change

to Xdot, Ydot, and Zdot until the desired 500 km apogee increase is obtained.  To calculate the total change of velocity that raised the apogee by 500 km, take the square root of the sum of the squares of the changes to Xdot, Ydot, and Zdot.

While the previous example was relatively simple, the same basic method of

iteratively creating and examining the resultant Keplerian Elements can be

used to explore strategies for accomplishing the rendezvous between orbiting

objects.  Taking into account the dispersions in the orbit determination

process and the effectiveness of orbit adjust maneuvers, the user can

calculate the required velocity change to accomplish a rendezvous sequence.

The ways in which VEC2TLE, in conjunction with satellite tracking programs

such as STSPLUS, may be used for the teaching of orbital mechanics and

astrodynamics in the high school, undergraduate, or graduate school

environments are limited only by the imagination of the educators.  Educators

who utilize VEC2TLE for such instructional purposes are encouraged to contact

the author so that those educational strategies may be detailed in future

versions of this documentation.

F.2		Planned Upgrades for VEC2TLE

This section provides information regarding planned upgrades to VEC2TLE.

Registered users are always encouraged to suggest items to be added to this

list.  The following upgrades are planned for the next (or some subsequent)

release of VEC2TLE:

-- The ability to read the NASA OIG RAID RBBS vector files directly.

-- An improved orbital revolution estimation algorithm.

PAGE 45:

-- An improved orbital drag estimation algorithm.

-- Support for vectors in the J2000 ECI coordinate frame.

-- Automatic detection of deep space orbits (period greater than 225

minutes).  Vectors representing deep space orbits would be automatically

deosculated to Keplerian elements compatible with the Space Command SDP4

algorithm when SGP4 is selected as the orbit theory.

PAGE 46:

APPENDIX G -- ABOUT THE AUTHOR

Ken Ernandes is an aerospace analyst specializing in astrodynamics analysis,

system simulation, data analysis, and software development.  He has a BS in

Physics/Mathematics from Manhattan College in New York City (1980) and an ME

in Space Systems Engineering/Engineering Management from the University of

Colorado at Boulder (1990).

Ken started in the space program after joining NORAD in October 1981, working

as a crew Orbital Analyst in Cheyenne Mountain.  The job entailed detecting,

tracking, identifying, and computing orbits for all man-made Earth-orbiting

satellites as well as real-time analysis of foreign space events, support of

manned and unmanned space missions, direction of the worldwide tracking

network, precision orbital decay predictions, management of on-line computer

activities, and operational command briefings.

Ken was selected to be an Orbital Analyst Instructor in November 1981.  His

job responsibilities included developing and conducting initial and follow-on

training for all crew-certified Orbital Analysts as well as space

indoctrination training for senior and flag rank officers.  His greatest

achievement as an instructor was the development of the Orbital Analyst

Positional Handbook.  A derivative of this training manual and reference is

still in operational use today.

After leaving active service in the Air Force, Ken joined Martin Marietta

Aerospace, Denver, CO in July 1984.  During his tenure at Martin Marietta,

Ken was responsible for astrodynamics and mission analysis as well as system

and mission simulation for design-phase spacecraft.

Ken joined Science Applications International Corporation in October 1985.

At SAIC he was the lead orbital engineer in the development of test

scenarios; exercising Cheyenne Mountain's computers under wartime conditions.

This led him to being the lead test designer for integration testing of the

Space Defense Operations Center (SPADOC) computer upgrade.  Ken is currently

employed by the Lockheed-Martin Corporation.

Ken may be contacted at the following addresses:

Ken Ernandes

29-34 170 Street

Flushing, NY 11358

USA

CompuServe:    	70511,3107

Internet:      	n2wwd@amsat.org

Packet Radio:  	N2WWD @KC2FD.#NLI.NY.USA.NOAM