MIDI for the Computerphile                             Page 1

A MIDI Primer for Computerphiles

In the course of pursuing my dual interests in music and

computers, I've noticed one thing:  though "crossover" in these

two fields is certainly evident in hardware and software, most

computer folk seem to know little about what those weird musician

types are doing with their digital machines, and musicians, well,

many musicians still think a byte is something you go out for

after playing a set.  Hopefully, this report will be of some use

to computer people who would like to know something about the

computer revolution which has swept the music industry, but don't

know where to start.  Obviously, this is a very large topic, and

I can only present the broadest outlines here.  If there is

sufficient interest, more detailed reports will follow.

A Basic Definition and a Little History

MIDI (Musical Instrument Digital Interface) is a

communications protocol developed jointly by American and

Japanese manufacturers of electronic musical instruments.  It is

a defined standard administered by an independent association,

the International Midi Association, and, at least theoretically,

assures compatibility among equipment produced by different

manufacturers.  In practice, the ideal of total compatibility is

not always achieved, but at least "MIDI standard" has a little

more meaning than "RS232 standard interface".

Now for a little history. MIDI is a fairly recent

innovation; the standard was first proposed in 1981, when,

fortunately, it was realized that unless someone did something,

chaos would prevail in the musical instrument industry (with, of

course, the resultant loss of sales.  Never forget that MIDI was

spawned by manufacturers, not by users.  More on this later.)

Over the past 15 or 20 years, many electronic instruments

have been introduced.  Keyboard synthesizers are examples that

are probably familiar to most people.  These machines were analog

devices -- the pitch of the sound they produced was determined by

a linear-scaled control voltage generated by the keyboard.  With

the advent of microprocessors, it became feasible to produce

digitally controlled instruments, eliminating the inherent

instability and inaccuracy of the analog control approach.

Please note, that the terms digital and analog are used here only

to describe the method of control; they have a totally different

meaning when applied to the technique used to generate the sound.

That will be discussed in the section on Hardware.  Anyway, what

is important is that instead of keyboards that produced varying

voltages, you now have keyboards that produce discrete codes,

just like a computer keyboard.

Now I must digress for a moment, and present a short

discussion on the elements of tonal music.  Tonal music (as

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.

#9

MIDI for the Computerphile                             Page 2

distinguished from atonal music, i.e. noise) can be described as

constituting three determinant parameters:

1 - Notes (comprised of pitch and harmonic data)

2 - Volume (how loud or soft the sounded note is)

3 - Duration (how long the note is sounded)

A digitally-encoded keyboard is capable of generating, as a

discrete quantity, only one of the 3 parameters.  Notes are

determined by which key is pressed.  However, duration can be

determined by measuring how long the key is pressed against a

known time base.  Volume is a little tougher; you can measure the

pressure with which the key is hit, but that will necessarily

require analog measurement.  However, if you measure the time

between the moment the key begins to be depressed and the moment

it is fully depressed, you will know how fast the key is

travelling, which gives a good indication of how hard it was

struck.  This measurement, (beginning of key travel - end of key

travel/time) is called "velocity" and provides the third

parameter.  (Some MIDI instruments use analog pressure

measurements to generate velocity data).

If these three parameters were recorded in real-time, and

then transmitted to an instrument, it would be possible to

reproduce the original performance exactly as the musician played

it.  This technique offers a major advantage over analog

recording technology:  there is none of the degradation of sound

which is inherent in any analog recording process.  The first

devices which attempted to store and reproduce these parameters

are familiar to everyone:  the old-fashioned player piano

captured key presses in real-time and stored them on paper piano

rolls for later retrieval.  The evolution of electronic

instruments resulted in various schemes to electronically store

these parameters.  However, the early attempts at "sequencing"

(the process of coding, storing and retrieving note, volume and

duration data) were individually developed by each manufacturer

-- compatibility between different brands of instruments was rare

(nonexistent).

The MIDI standard was originally proposed to provide a

single standard to be used by all instrument manufacturers, so

that varied and different instruments could function together.

However, as will be shown later, MIDI has evolved considerably

beyond a basic note definition "language".

A Little Technical Data

I won't reproduce the full standard here because it is

readily available from a variety of sources and is not necessary

for understanding the basic principles and applications of MIDI.

MIDI is a synchronous digital protocol.  Data is sent in 8-bit

words at 31.2K baud using a current loop.  Why yet another

format, when good ole' RS232 is sitting there on just about every

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.

#9

MIDI for the Computerphile                             Page 3

computer in existence?  The official Party Line is:  RS232, with

its top speed of 19.2K is too slow to handle all the note data.

Current loop is necessary to suppress interference that would

result from the long cable runs.  Could it be an excuse to sell

more hardware? Hmmmmmm.  Anyway, back to facts.  MIDI protocol

consists of control and data words which may be from 1 to 3 bytes

long, or, in certain situations, longer.  MIDI defines the three

parameters from the previous section as follows:

NOTES:  128 notes are defined, from 0 to 127.  Notes

follow the standard even-tempered chromatic scale.

Notes are NOT defined as specific frequencies,

permitting performers to tune their instruments as

required.

VOLUME:  The primary means of specifying volume is

velocity (see previous discussion).  Velocity is

quantized per note in discrete steps from 0 (softest)

to 127 (loudest).  There are also other parameters

which control relative volume of the entire instrument.

DURATION:  MIDI handles duration of notes with two

parameters:  NOTE ON and NOTE OFF.  NOTE ON is

generated when the key is pressed, NOTE OFF is

generated when the key is released.  These two commands

are sent independently of each other; if a NOTE ON is

issued and a corresponding NOTE OFF is not sent

(because of data errors, mechanical failures, or poor

programming) the note will sound forever (or until the

instrument is turned off).  This presents occasional

problems, particularly in live performance, because, as

in any communications protocol, data errors do

sometimes happen.  MIDI does provide an "All Notes Off"

command.

As stated before, duration must be measured against a known

time standard.  MIDI provides a MIDI clock signal, which is sent

as a specifically designated data byte.  MIDI divides each

musical beat into 128 MIDI clocks.  MIDI also defines the

following parameters:

PROGRAM CHANGE:  This parameter selects different

"patches" or sounds in the musical instrument.  The

programs are numbered 0-127.  Note that the patches

themselves are not defined by the MIDI standard.

Program #32 might be a violin on one synthesizer and

barking dogs on another.

CHANNEL:  MIDI provides for 16 different channels.

Most MIDI commands and data can be specific to a single

channel, i.e. instrument, or can be global.

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.

#9

MIDI for the Computerphile                             Page 4

PITCH BEND:  As the name implies, the pitch of the note

can be "bent" up or down in real time (remember Jimi

Hendrix?).

MODULATION:  This is a control parameter that usually

effects the vibrato sound of the note, though some

instruments can be programmed to use modulation data to

control other parameters.  Modulation and Pitch Bend

are usually controlled from the instrument with wheels

or levers that the performer can rock back and forth.

SUSTAIN:  The same as the sustain pedal on a piano, it

will cause the note to sound until the pedal (or other

control device) is released.

The MIDI spec allows for 127 different control parameters,

although only a small number are currently identified and

standardized.  In addition, MIDI provides a "system exclusive"

message.  Each manufacturer is assigned their own unique "sys ex"

code which allows them to implement custom features without

interfering with other manufacturers customizations.  Ah ha! you

say, doesn't that defeat the purpose of a "standard"?  Yes, to an

extent it does, but remember it was the manufacturers of the

equipment who pushed for a standard, and allows for innovation

and differentiation between brands.  And, as stated earlier, you

must admit that this standard is considerably more consistent

than a "standard" RS232 interface.

The preceding constitutes only the broadest description of

the MIDI protocol, and there are quite a few more features which

I haven't covered here.  However, you should have a general idea

of the kinds of data MIDI can handle.  Now I'll tell you some of

the ways MIDI is used.

What is MIDI Used For?

Ok, we've established that MIDI is both a communications

protocol and note definition language.  What can it do?

1.  Control of Instruments

Any musical instrument can be thought of as having two

distinct components: the sound producing component and the

control component.  As an example, the keyboard of a piano, the

frets on a guitar and the buttons on a clarinet can all be

thought of as control components.  The piano's strings and

hammers, the guitar's strings and acoustic body (or magnetic

pickups if it is electric) and the clarinet's reeds and hollow

body are all sound producers.  The first application for MIDI

permits the separation of control and sound production

components.  The most common (though not the only) MIDI

controller is the keyboard.  The keyboard generates the NOTE ON

and NOTE OFF data as well as various other control data (see

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.

#9

MIDI for the Computerphile                             Page 5

previous section) and passes it on to the sound producing

section.  The sound producing section could be a synthesizer,

digital sampler, drum machine, or any other MIDI equipped sound

producing device.  Most synthesizers combine the sound producing

device and the keyboard controller in a single physical package.

However, MIDI permits them two be addressed as two distinct and

separate sections.

A single musician at a single keyboard can play many

different instruments simultaneously.  There are two ways of

doing this.  The first places several different sound producers

on the same channel, all responding to the same MIDI data at the

same time.  This is a process called "layering" and can be used

to produce full, rich, harmonically complex sounds.  The effect

is of several different instruments all playing in unison.

The second technique is called "splitting" the keyboard --

arbitrarily assigning specific channels to specific notes on the

keyboard.  This permits different instruments to play different

music, all under the control of one musician at one keyboard.  As

an example, the musician might assign the bottom two octaves of

the keyboard to channel 1 and the rest of the keyboard to channel

2.  If a digital sampler set to reproduce the sound of a bass is

assigned to channel 1, and a synthesizer producing a piano sound

is assigned to channel 2, the musician will be able to play the

bass line accompaniment with his left hand while playing the

piano lead with his right.

Note that when instruments are layered, an unlimited number

of instruments may be played.  However, when the keyboard is

split, the maximum number of instruments that may be controlled

is limited to the maximum number of MIDI channels -- 16.

2. Sequencers

Another obvious application for MIDI is the storing of the

MIDI data stream for later playback -- a process known as

sequencing.  This task may be performed either by a dedicated

piece of hardware (called, of course, a "sequencer") or by

general purpose computers equipped with the appropriate software

and interfacing.

Most sequencers allow editing of the MIDI data -- wrong

notes can be corrected, new material can be entered one note at a

time, and sections can be rearranged, moved or copied (much like

a word processor).  Almost all sequencers allow for transposition

(changing key) and tempo changes.  Some will "auto-correct" so

that all notes are played exactly on the beat, eliminating any

sloppiness in playing.

Since MIDI provides 16 unique channels, 16 different

instruments can be controlled simultaneously, allowing the

sequencer to function like a multi-track tape recorder.  Each

instrument is "played" into the sequencer individually on a

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.

#9

MIDI for the Computerphile                             Page 6

different channel.  When all the parts have been entered, the

sequencer can play them back all at the same time, in effect

creating a one-man band (or one-man philharmonic orchestra).

Finally, since MIDI was developed as a professional and

semiprofessional musical tool, several features required for

recording are supported.  Most sequencers allow for some form of

tape sync.  In the most basic form of tape sync, the sequencer

provides a synchronization signal which can be laid down on one

track of a multi-track tape recorder.  When new tracks are laid

down, the sequencer can synchronize to the previously recorded

material.  This makes multi-track recording, over-dubbing, and

similar recording tricks much easier.  The MIDI spec also defines

a MIDI SONG POINTER, which can be thought of as "mile markers" in

the music.  A sequencer that supports MIDI SONG POINTERS is

capable of automated punch-in and punch-out -- the process of

inserting new material into a previously recorded track.

3.  Librarian Software

One of the uses manufacturers of MIDI instruments make of

the SYSTEM EXCLUSIVE command is for data dumps; literally

"dumping" all the parameter data needed to define sounds and

setups out the MIDI line on command.  Software that stores this

data for later recall is called Librarian software.  Most

librarians are not limited to merely storing and retrieving the

dumped data, but are also capable of editing specific patch

parameters -- a task which is more easily performed on a computer

with a full keyboard and video display than on the more limited

displays and entry devices available on the synthesizers

themselves.

A special form of librarian, generally called a Sound

Modeling Program, use the SYSTEM EXCLUSIVE dump command to obtain

wave sample data from digital samplers (see the section on

Hardware).  The wave sample can then be displayed, manipulated,

stored and retrieved by the librarian.  "Serious" sampling with

digital samplers virtually mandates the use of some form of Sound

Modeling Program.

4.  Notation Software

Notation software takes the MIDI note data and translates it

into conventional music notation which can be displayed on the

screen or printed on a dot matrix or laser printer.  It allows a

musician to play music in on the controller keyboard in real time

and get finished musical scores out.  Alternatively, music can be

entered in notational form on the computer keyboard using

wordprocessor-like commands, and the finished result can be heard

played on a MIDI equipped synthesizer.

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.

#9

MIDI for the Computerphile                             Page 7

5.  Other Applications

Lately, MIDI has also found application in non-musical

functions, e.g. controlling mixing boards, stage lighting and

sound processing equipment (reverbs, digital delays, etc.).

Problems with MIDI

As powerful a tool as MIDI is, it is not totally without

problems.  What follows are a few cautionary notes:

1.  As already mentioned, MIDI defines note duration with two

separate data commands: NOTE ON and NOTE OFF.  The possibility of

a NOTE ON being transmitted without a corresponding NOTE OFF

following is an always present danger.  Data can get garbled

during transmission, lines become unplugged (the MIDI standard

utilizes 5-pin DIN connectors which have a nasty habit of

loosening in their sockets), channels accidentally get switched,

etc.  Without a NOTE OFF command, the note will continue to sound

for ever.  This can be a major annoyance in a recording session

(more than annoying if it occurs during that once-in-a-lifetime

hot set) and in live performance, well, you get the idea.

Various manufacturers have come up with different solutions to

the problem, the most common being a button which, when pressed,

produces an ALL NOTES OFF command.  This will, of course, silence

the offending note, but silences all the other notes in the

process.  Because the two note NOTE ON/NOTE OFF structure is

intrinsic to the MIDI spec, we will just have to live with the

occasional stuck note.

2.  MIDI transmits at 31.2k baud.  A little math shows that MIDI

is capable of sending approximately 1000 notes per second.  This

is obviously more than any musician can ever play.  However, when

you divide this capacity among 16 channels, add in the data

stream produced by controllers such as pitch benders and

modulation wheels, then throw in a few program changes for good

measure, you have the possibility of overrunning the data.  To be

honest, I've never heard of this happening, but the possibility

is still there.

3.  Most MIDI instruments contain a MIDI-in jack for receiving

data, a MIDI-out jack for transmitting data, and a MIDI-thru jack

for passing MIDI data along to another instrument.  When

instruments are daisy-chained together, a perceivable delay

develops between the first and last instrument in the chain.

This is the infamous "MIDI delay" of which you may have heard.

This delay can be eliminated by using a device known as a "MIDI

Thru Box".  This is an active splitter that accepts one MIDI

input and divides it into 4,6 or more MIDI outputs, thereby

assuring that all instruments receive the MIDI signal at the same

time.  Does it solve the problem?  You bet!  Does it cost more

money?  You bet!

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.

#9

MIDI for the Computerphile                             Page 8

4.  Another problem, frequently mistaken for MIDI delay, is

inherent in some MIDI synthesizers.  Remember that the keys on

the keyboard are scanned sequentially, in the same manner as a

computer keyboard.  In some brands of keyboard synthesizers, the

internal electronics introduce their own delays in translating

the key presses into sounds.  Though not specifically a MIDI

problem, it is a problem none the less, and is particularly

evident when the musician "grabs" large chords consisting of many

notes.  Fortunately, this problem is recognized as a hardware

"bug" and is usually corrected by the manufacturer (after enough

people complain).

5.  By its very nature, MIDI is designed to permit one controller

on line at a time.  More than one controller on line will result

in inevitable data collisions with the resultant garbling of

data.  Think of two computers sending data to two printers at the

same time.  There are devices available which mediate data

contention between two MIDI controllers.  Generally called Midi

Mergers, they are another relatively expensive solution to what

seems like a simple problem.

6.  Though the MIDI protocol is clearly defined in the spec,

computer storage schemes are left up to the individual software

producer.  There is no MIDI equivalent of an ASCII or EBDIC file.

Consequently, MIDI data files produced by one piece of software

can not be read by another.  This would be fine if there were

such a thing as a program that did everything (and did it well).

However, as in the case of the "integrated" packages that

combined word processors, spread sheets, data bases and

communications software all in one box, such as thing simply

doesn't exist.  Right now, the only way to exchange MIDI data

between programs is by transmitting the data over a physical MIDI

data line between two computers.

7.  As mentioned earlier, the MIDI spec provides considerable

latitude to manufacturers who want to incorporate custom features

in their instruments.  Consequently, MIDI is afflicted with

"creeping non-standardization".  There are already controller

number conflicts between some of the largest instrument

manufacturers, and the SYSTEM EXCLUSIVE command guarantees that

no single librarian program will work with all synthesizers.

8.  Now for what I see as the major problem with MIDI - MONEY!

Until the electronic revolution, quality musical instruments were

carefully crafted, often handmade, and very expensive.  After

all, great instrument making is an art.  Consequently, musicians

have become accustomed to paying a lot of money for the

instruments they play.  This tradition has been maintained by

many manufacturers of mass-produced electronic instruments.

There are exceptions (see the description of the Casio CZ-101 in

the Hardware section), but for the most part, the sales price of

a piece of electronic musical gear frequently does not bear any

correspondence to the cost of its manufacture.  Fortunately, the

current trend is towards dropping prices.  However, walk into any

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.

#9

MIDI for the Computerphile                             Page 9

music store and you will see MIDI cables (2 DIN plugs and 10 feet

of 2 conductor shield cable) for $25 or more.  Though good MIDI

software tends to be very expensive, it should be remembered that

MIDI software publishers have a much more limited market for

their product than publishers of more common business-oriented

packages.

A Brief (and Biased) Hardware Catalog

I would like to conclude this report with a description of

the types of MIDI hardware currently available.  Hardware

selection is a very personal decision, and what I say here

(beyond the basic descriptions) is necessarily biased by my own

preferences.  Here goes:

1.  Sound Producing Devices

The devices which produce the actual sounds can be divided

into two basic categories:

-  Synthesizers - devices which generate basic sound

waveforms, and, by manipulating various parameters of

the sound (envelope, modulation, etc.) can produce a

diversity of tonal textures and colors.

-  Samplers - devices that digitally record, or sample,

a sound, and then play it back at pitches determined by

the controller.

Synthesizers can be divided into two broad categories.

Analog synthesizer use conventional oscillators and filters to

produce different sound waveforms, e.g. sine waves, square waves,

triangle waves, etc.  Digital synthesizers "store" a digitally

encoded waveform in ROM, and produce their sounds by reading the

waveform out to D-to-A convertor.  Generally, analog synthesizers

are thought of as producing "warmer" sounds than their digital

counterparts.  Some common synthesizers are:

Yamaha DX-7 (approx. $1700) - The DX-7 has become something

of a "standard" for digital synthesis, and is used by many

professional musicians.  Most librarian software supports

this machine.

Casio CZ-101 (approx. $300) - Casio has a complete line of

digital synthesizers, but the CZ-101, in my opinion, offers

more "bang for the buck" than anything else on the market.

This machine is supported by many librarians, is capable of

producing an astonishing range of sounds and is an excellent

"starter" for anyone interested in testing the waters of

electronic music.  Two drawbacks:  it has a small keyboard

that is difficult to play, and does not support note

velocity; all notes default to a velocity of 64.

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.

#9

MIDI for the Computerphile                             Page 10

Samplers are available in a wide range of prices and

capabilities.  Factors to be considered in purchasing a sampler:

1 - Sample width.  The more bits per sample, the better the

resolution, dynamic range, and fidelity. This rule is not

written in stone, however, as many manufacturers have

developed data compression algorithms that allow them to

squeeze more information out of smaller width samples, and a

12-bit machine may not necessarily sound better than an

8-bit machine.

2 - Sampling rate.  Higher sampling rates permit the

reproduction of higher frequency sound data.  The Nyquist

rule specifies a sampling rate 2-1/2 times the frequency of

the highest sound to be sampled, i.e. to sample the full

audible audio spectrum (20 Hz to 20,000 Hz), a sampler

should have a sampling rate of (2.5 * 20,000) or 50,000

samples per second.

3 - Number of active samples.  The sound of a "real"

instrument can not be reproduced by sampling only one note

and "stretching" it across the entire keyboard.  Many

samples taken at many different pitches are necessary to

effectively simulate the distinctive voice of an instrument.

Some samplers provide only a single sample at a time which

must be stretched.  Others provide as many as 64 active

samples at a time which may be assigned to specific sections

of the keyboard as needed.

4 - Available memory.  Samplers with a lot of memory can

allow higher sampling frequencies, longer samples, and more

active samples.

Digital Samplers are "where the action is" and new machines

are being introduced all the time.  Some inexpensive samples of

samplers:

Akai  S612 (approx. $1000 with disk storage):  The least

expensive "real" sampler (as contrasted with several

sampling toys that have recently hit the market) the S612 is

also available in an expanded version called the S900 for

approximately $3000. The S612 is limited to one active

sample at a time.  Sampling rate is also limited, which

restricts its ability to sample high frequency sounds.  In

addition, it is a rack-mounted device which requires a MIDI

keyboard to control it.  It is, however, a fully implemented

sampler for a minimum price, and constitutes a good

entry-level machine for those who want to experiment with

sampling.

Ensoniq Mirage (approx. $1700 with keyboard):  The Mirage is

a versatile instrument that offers a user-selectable sample

rates up to 30 kHz with up to 16 active samples. The machine

also has a full sound modification section consisting of the

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.

#9

MIDI for the Computerphile                             Page 11

traditional envelope and filter synthesizer controls,

permitting the user considerable flexibility in customizing

sound samples. Ensoniq offers a large library of factory

samples on 3-1/2" disk (disk drive included with the Mirage)

which range in quality from adequate to extraordinary.  A

good test for a sampler is the ability to recreate the sound

of an acoustic piano, as pianos produce extremely complex

waveforms.  The Mirage does a very credible job with pianos,

as well as other acoustic instruments.  The Mirage is also

available in a less expensive rack mount version (no

keyboard).

Sequential Circuits Prophet 2000 (approx. $2500): 12-bit

sampler with extensive MIDI implementation.  Sampling rate

user-switchable up to 41 kHz.  There has been some criticism

of the quality of factory-produced library samples,  but the

machine is capable of extremely high-quality sampling.

Emu Emulator II (approx. $8000):  A very high quality

sampler the uses a proprietary data encoding scheme to

purportedly wring 14-bit resolution out of 8-bit samples.

The Emulator was one of the first "affordable" samplers (as

compared with the $50K - $100K Fairlight and Synclavier) and

has seen extensive use in professional recording and live

performance.

Drum machines are specialized devices that simulate the

sound of a drum set.  The sounds are ROM-based digital samples,

though some machines permit the user to sample their own sounds.

All drum machines allow the user to define a number of patterns

which can be strung together to form "songs".  A representative

drum machine:

Yamaha RX-15 (approx. $400) - Has 16 different drum sounds

(only 12 available at the same time), memory for up to 99

patterns and 10 songs (depending on complexity).  Good MIDI

implementation, though incapable of MIDI sys ex dumps.  Also

available as the RX-11 (approx. $700) with complete MIDI

implementation.

2 - Keyboard Controllers

Keyboard controllers produce no sounds by themselves but

generate the MIDI data necessary to control MIDI sound-producing

devices.  Keyboard controllers have "actions" that provide a feel

similar to traditional pianos.

MIDI data can also be generated by non-keyboard devices,

including guitars, drums and various wind instruments.  Devices

called "pitch riders" can translate an analog sound input into

MIDI data output.

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.

#9

MIDI for the Computerphile                             Page 12

3 - Computers

This is a volatile area for discussion, with proponents of

different brands fiercely loyal to their machines (as I am

fiercely loyal to mine - an IBM PC-XT).  Anyway, here's a quick

run down:

Inexpensive Machines:

Commodore 64/128:  Many software packages are available, as

well as different interface options.  The machine's 64K of

memory presents a limitation for sequencing and scoring

programs, but low cost of the Commodores makes for them good

MIDI "starter" systems.  If you are considering the

Commodore, avoid software that claims it can use the 64's

internal sound chip to produce "real professional

synthesizer sounds".  The sound chip on the 64 is quite

clever and very versatile, but is limited to three voice

polyphony and has severely restricted sound modifying

capabilities.  It is not a substitute for a synthesizer by

itself.  Passport and Dr. T are two publishers of quality

MIDI software for the Commodore.

Low-cost Atari's:  Same limitations as the Commodore, though

perhaps with fewer quality software packages available.

Noted exception: Hybrid Arts produces well recognized and

well respected software, though unfortunately, only for the

Atari line.

Apple II - About on a par with the cheap Atari's from a

music standpoint.  48K memory presents severe limitations.

Moderately Priced Machines:

Only two worth considering - the Amiga and the Atari ST.

The Atari has slightly more software packages available and

offers a built-in MIDI interface.  The MIDI interface is an

extra-cost option on the Amiga.  I'll avoid jumping into the

Atari vs. Amiga war by saying both machines offer good value

and are well-suited for MIDI and other music applications.

More Expensive Machines:

Macintosh:  In fairness, I must confess to a certain amount

of anti-Mac prejudice.  My criticisms are not new:

expensive peripherals, up-until-recently closed

architecture, hard-to-support serial bus, operating system

designed for computerphobes, etc.  However, if you like it

you like it, and there are some excellent professional music

packages available for it.  However, my preference, hands

down, is:

IBM PC-XT (and PC clones):  The PC offers a great variety of

powerful, professional music software packages, and a

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.

#9

MIDI for the Computerphile                             Page 13

variety of flexible MIDI interfaces are available for it.

It is also the only machine which will run Personal

Composer, the only notation program I've seen which actually

works.  There are other notation programs out, but they are

so riddled with bugs as to be almost unusable, or else are

so limited in their implementations as to impose severe

restrictions on composers.  (I will happily retract the

preceding statements upon compelling evidence to the

contrary). One program worth mentioning is Texture, a

sequencer which has become the de facto standard of

professional musicians.

Final Words

As stated at the outset, MIDI is a very large topic to be

tackled in a single report.  I hope I have presented enough

information to give the MIDI neophyte a basic understanding of

the topic.  Let me also remind you that the "review" material

presented here is highly subjective, particularly in the

"hardware" section.  There are many more fine instruments and

software packages which I have not mentioned here.

A good source for information on MIDI and electronic music

in general is KEYBOARD magazine, which publishes reviews of

hardware and software, features on all aspects of keyboard music

(both acoustic and electronic), occasional how-to articles, etc.

I hope some of you who read this will be motivated to

experiment with MIDI.  The control capabilities of sequencers and

the sound generating abilities of the various synthesizers and

samplers put the ability to create professional sounding music in

the hands of almost anyone.  If there is enough interest, I can

prepare occasional reports that explore specific areas of

electronic music production in greater detail.  Messages for me

can be left at The Sleepy Hollow BBS, 213-859-9334 (24 hours,

1200 baud, 8-bit, no parity, 1 stop bit), which, incidentally, is

the finest BBS I've encountered and is run by a knowledgeable and

dedicated Sysop.

Copyright 1989 by Paul Tauger.  This article may be freely

exchanged, copied and/or distributed provided it is done without

charge.