Audio Beep Errors:

1 beep  - DRAM refresh failure

2 beeps - Parity Circuit Failure

3 beeps - Base 64K RAM failure

4 beeps - System Timer Failure

5 beeps - Processor Failure

6 beeps - Keyboard Controller / Gate A20 Failure

7 beeps - Virtual Mode Exception Error

8 beeps - Display Memory Read/Write Failure

9 beeps - ROM BIOS Checksum Failure

10 beeps - CMOS Shutdown Register Read/Write Error

For 1 beep, 2 beeps, or 3 beeps try reseating the memory first. If

the error still occurs, replace the memory with known good chips.

For 4 beeps, 5 beeps, 7 beeps, or 10 beeps the system board must

be sent in for repair.

For 6 beeps try reseating the keyboard controller chip. If the error

still occurs, replace the keyboard chip. If the error persists, check

parts of the system relating to the keyboard, e.g. try another keyboard,

check to see if the system has a keyboard fuse.

8 beeps indicates a memory error on the video adapter. Replace the

video card or the memory on the video card.

9 beeps indicates faulty BIOS chip(s). It is not likely that this

error can be corrected by reseating the chips. Consult the motherboard

supplier or an AMI product distributor for replacement part(s).

If no beeps are heard and no display is on the screen, The first

thing to check is the power supply. Connect a LED to the POWER LED

connection on the motherboard. If this LED lights and the drive(s) spin

up then the power supply will usually be good.

Next, inspect the motherboard for loose components. A loose or

missing CPU, BIOS chip, Crystal Oscillator, or Chipset chip will cause

the motherboard not to function.

Next, eliminate the possibility of interference by a bad or

improperly set up I/O card by removing all card except the video adapter.

The system should at least power up and wait for a drive time-out.

Insert the cards back into the system one at a time until the problem

happens again. When the system does nothing, the problem will be with

the last expansion card that was put in.

If the above suggestions fail to cause any change in the disfunction

of the system, the motherboard must be returned for repair.