Testmem 0.11� By Craig Hart (C) 1997,8.

� Freeware notice

Ths program is freeware. Do what you like with it.. it's free for your use,

without restriction. It is also unsupported... I do mantain a web page and try

to fix bugs with my software, but I'm just a guy with a pc, not a commercial

operation, so I don't make no guarantees. Having said that, I do try to work

with people to make my software better for everyone. Read on for details of how

you may contact me, obtain newer versions and try my many other diagnostic

packages.

� The Basics

This is a program that tests your extended memory. It needs to be run from a

plain DOS boot floppy - no memory managers or drivers loaded please. Testmem

requires at least a 386 type CPU to run, and will refuse to run on any system

that is not configured to it's liking. Testmem does not test base memory (The

first 640k).

To create a plain DOS boot floppy:

format a: /s /u

copy testmem.exe a:

Boot from the floppy, then run Testmem.

Alternately, if booting DOS 6.x, press F5 when you see  'Starting MS-DOS...' if

booting Win95, press F8 and select 'Safe mode, command prompt only' when you

see 'Starting windows 95...'. Testmem will not work under WinNT, OS/2, Linux,

UNIX, etc.

The screen testmem displays is fairly self-explanitory; if a bad memory

location is found, Testmem will display a message in the middle of the screen

with the bit pattern tried and the bad data returned so you can figure out

which bit(s) are failing. If the error count never gets above 0 no matter how

long you run testmem for, then your memory is probably OK. I recommend running

testmem overnight, since memory problems are usually quite fickle and don't

always show up immediately. No matter how many errors show up, Testmem keeps on

going.

Note: Some memory erros only show up when the computer's insides have reached

operating temperature - something that requires the lid to be on, and the

machine to be running for at least 30 minutes. Make sure you always setup any

questionable machine in as near to possible it's normal operating environment

so as to give yourself the best chance of finding the fault. Press any key to

exit the program (response should be within 1-2 seconds; if not, you can safely

assume the machine has crashed and reboot it).

Testmem disables the CPU's internal cache before starting tests. This is

primarily to cause the L2 Motherboard cache to disable so that it doesn't

interfere with tests.. Therefore a faulty cache will NOT be detected by

Testmem. This has the side-effect of making the program a lot slower than you

might expect... this is normal, and another reason why it needs to be run for

as long a period of time as is possible.

Disabling the L2 cache makes Testmem more accurate - other memory testers fail

to take this step (ie himem.sys) and therefore they sometimes miss bad memory,

since the memory read is really a cache read, and thus not a good test of the

memory itself. Remember, Testmem is a *MEMORY* tester, not a *CACHE* tester!

� Notice

This program is a beta test only of some routines I eventually hope to merge

with my main diagnostics package. It is released for wide-beta testing and YOUR

comment. Please email me or visit my web site to send your feedback and obtain

the latest versions of all my software. I'd be very happy to hear of your

experiences with Testmem - both good and bad.

email:	chart@hyperlink.net.au

web:	http://home.hyperlink.net.au/~chart

What's new to version 0.11�:

I now re-enable the caches when you exit the program... primarily because

people complained about it, but also because it's good form to return a system

to the user the way you found it. A20 is left enabled, however.

I now use 'both' methods of enabling A20 - the "AT" method (via the keyboard

controller) and the "PS/2" method (By setting bit 1 of port 92h) - this should

make testmem operate on all the machines it formerly failed to (I hope).

The actual test routines themselves are unchanged. I hope to re-write the test

routines in protected mode as there is a definate speed advantage to doing so,

but also because it seems that many memory faults are only able to be detected

when the CPU is in 32-bit mode! I'm not quite sure exactly why - perhaps it is

because 32-bit mode is the only mode that can really "push" the memory access

speed towards the theoretical limits, whereas 16-bit mode seemingly can't.