comment <------------------------------------------------------------------

NewBasic Assembler

Small Tutorial

Freeware

Forever Young Software

(C)opyright  1984-2008

All rights reserved

19 Mar 2001

This is a small tutorial on how to program in assembler.  All the code,

unless otherwise stated, is assembled with NBASM.  This tutorial in no

way will teach you the complex language of assembler.   However, it is

my goal to  teach you the basics of assembler and how to you NBASM, or

another assembler,  to create your programs.   We will discuss  how to

create workable, productable, .COM files.  Once you know how to create

.COM files, the move to .OBJ files is not to difficult.

I think the first thing  we will discuss is  program layout.   How does

DOS load and execute your program,  what are the limits to .COM files,

and what are the capabilities of .COM files.

DOS first allocates all available  memory and creates a  256 byte block

of memory at the beginning of this block.  The block is called the PSP.

The Program  Segment Prefix is filled  with information that is needed

by DOS as well as your program.

Then DOS loads a mirror image of your .COM file in to  memory just after

this PSP and points CS:IP to your code (100h).  DS,  ES, and SS are all

pointed to the same segment that CS points to. Since this happens, your

.COM file can only be one segment in length (65536) and all code, data,

and stack must be in this segment.  For this reason, this puts a few re-

strictions on a .COM file:

1. - A valid .COM file can only be 65536 - 256 bytes in length.

Since DOS places a 256 byte PSP at the first of the segment, your

program can only by up to 65280 bytes in length.

2. - Since SS:SP points to the last word in the segment, if you intend

to use the stack, you can not occupy the area at the end of this

segment or your stack will overwrite it.

3. - Since DOS points CS:IP to offset 100h,  you must have a valid asm

instruction at offset 100h.  This means the first  thing in your

program must be a valid code instruction.

4. - As mentioned before,  all your code and data must be  in this one

segment.  Therefore, you can not write extremely large programs.

However, unless you have a lot of strings in your data,  you can

write a fairly large program using this .COM format.

A few benefits of a  .COM file are that you can relocate the  stack to a

different place.  The stack never contains  predefined values (with one

exception described later).  This means that if you know  you are going

to use a larger  stack that is left available by your  program, you can

create a small amount of code to allocate a different block,  and point

SS:SP to the end of this block.  Now your program can occupy all of the

64k of the memory allocated.  *A note*  For that matter, you can always

have your data saved in a file and load it into a different segment and

point DS to this segment.  However, if you where going to do  this, you

might as well have just created an valid .EXE file.

Another benefit of  a .COM file is that it  is loaded much quicker.  DOS

must only load 64k or less, doesn't have to calculate relocation items,

and doesn't have to calculate other program operations at start up.

Also, most .EXE files must have a  256 byte header while a .COM  has no

such header.  Therefore .COM file can be much smaller.

Let us start by thinking of a program we would like to create. This task

must be simple enough for a beginner just learning to program assembly,

but large enough to make a good tutorial covering a lot of areas.

What would you  say to a small file viewer?   Just a simple  viewer that

would display a file allowing you to scroll up and down. Maybe we could

add small enhancements to it that would send it to the printer or some-

thing like that.

Well, first we must realize what we  will need to do.  To create a small

file viewer like this, we will need to do the following:

- get command line

- memory allocation

- open/read/close a file

- screen output

- keyboard input

<---------------------------------------------------------------------------

;  Shall we start?   First we must tell the assembler  how and what  we are

;   going to do.  We want a .COM file, we will be  using x186 instructions,

;   and we want to start at offset 100h (256d) to allow room for the PSP.

;

;  Open a new file and name it V.ASM.  Now enter the following at the start

;   of the file:  (Note that this nbasmtut.doc file is assemble-able as is)

.model tiny

.186

.code

org  100h

;  We have told the assembler to create a .COM file  with the 'model tiny'

;   directive.   A model is  how the program  will use segments.  The tiny

;   model uses one segment for all items.

;

;  Then we tell the  assembler to allow all instructions up to and includ-

;   ing the 186.  Next is  the org 100h.  This tells the assembler to that

;   the following data/code will start at offset 100h in the segment. Then

;   the  .code directive  telling the assembler  this is the  start of the

;   code area, start assembling.

;

;  To start our veiwer we need to know what file to open. When DOS creates

;   the PSP, it puts a count of the characters in  the parameter list used

;   on the command line  at offset 80h.  Then it has 127 characters of the

;   parameter list at offset 81h.  To get this parameter list,  we need to

;   know the length, so let's get the count at offset 80h.  Then let's get

;   the filename from this command line and place it in our filename area.

mov  si,0080h                ; the count byte in the PSP

lodsb                        ; 'mov  al,[si]/inc si'

or   al,al                   ; is al = 0?

jnz  short FIsThere          ; if 0 then no command line there

mov  dx,offset ParmErrS      ; so use DOS print string service

mov  ah,09                   ;  to print an error

int  21h                     ;

.exit                        ; exit to dos (user directive/macro)

FIsThere:  inc  si                      ; else if count > 0 then a command line

mov  di,offset FileName      ; skip first space and point di to filename

GetFileN:  lodsb                        ; get the command line until a 0Dh

cmp  al,13                   ;

je   short GotFileN          ;

stosb                        ;

jmp  short GetFileN          ;

GotFileN:  xor  al,al                   ; make string asciiz

stosb                        ;

;  Now we have an asciiz filename ready for opening. An asciiz string is a

;   string with a NULL char as the last  character in the string.  This is

;   used by most services of DOS to denote eol (End Of Line).  Please note

;   that DOS service 09h that we used above,  uses a dollar sign to denote

;   eol.  (We will add the data to our code later in this tutorial).

;

;  Now that we  have a valid filename asciiz string,  we can open the file

;   for viewing.  However, we need to pick the correct service to open it.

;   DOS has quite a few FILE OPEN services available to us.   Let us pick

;   service 3Dh,  'Open Existing File'.  This service  returns an error if

;   the file does not exist.

mov  ax,3D00h                ; open existing file for read only

mov  dx,offset Filename      ;

int  21h                     ;

jnc  short File1OK           ; if carry, then error occured

mov  dx,offset File1ErrS     ;  print error string

mov  ah,09h                  ;

int  21h                     ;

.exit                        ;

File1OK:

;  Now that we have opened the file and have a file handle in AX, allow us

;   to read in the file.  Being that this is a simple program, and we will

;   assume all files are 32k or less, let us read only 32k of the file.

mov  bx,ax                   ; put handle in bx

mov  ah,3Fh                  ; read from handle

mov  cx,32767                ; read at most 32k

mov  dx,offset Buffer1       ; pointer to our buffer

int  21h                     ;

mov  FileLen,ax              ; ax = amount read

mov  ah,3Eh                  ; close the file

int  21h                     ;

;  Notice that service 3Fh  returns amount read in AX.   We will need this

;   amount for later.

;

;  Now that we have the contents of the file in our buffer, we need to set

;   up our display.   First let us clear  out the part of  the data buffer

;   that is not being used by the file.

mov  di,offset Buffer1       ; clear the data buffer

add  di,FileLen              ; not being occupied by

mov  cx,32768                ; the file

sub  cx,FileLen              ;

shr  cx,1                    ; divide by two for words

mov  ax,000Dh                ;

rep                          ;

stosw                        ;

jnc  short skip_byte         ; if there was a byte left over

stosb                        ;  from shr cx,1 then clear it too

skip_byte:

;  Now let us turn the cursor off and put a header line across the the top

;   of the screen and a status line across the bottom of the screen.   For

;   speed,  we will  write directly to  the screen memory  rather than the

;   slower methods.

mov  ah,01h                  ; turn off cursor

mov  ch,20h                  ; bit number 5

int  10h                     ;

mov  ax,0B800h               ; point es to screen memory

mov  es,ax                   ;

xor  di,di                   ; first position on the screen

mov  ah,HdrClr               ; our background color

mov  si,offset Header        ; point to our header line

mov  cx,80                   ; 80 chars across the screen

PrntHdr:   lodsb                        ;

stosw                        ;

loop PrntHdr                 ;

mov  al,20h                  ; clear the middle section

mov  ah,txtclr               ;

mov  cx,1840                 ;

rep                          ;

stosw                        ;

mov  ah,HdrClr               ; our background color

mov  si,offset Status        ; point to our status line

mov  cx,80                   ; 80 chars across the screen

PrntHdr1:  lodsb                        ;

stosw                        ;

loop PrntHdr1                ;

;  Now we must set up our user input area and print the actual data to the

;   screen.   We first  calculate where we  are in the  data file and then

;   we print 23 lines of text.

mov  si,offset Buffer1

PrintIt:   push si

mov  di,160

mov  cx,23

PrtItL1:   push cx

push di

push di

mov  cx,80

mov  ax,0720h

rep

stosw

pop  di

mov  cx,81

PrtItL2:   lodsb

cmp  al,13

je   short PrtItL2D

cmp  al,32

jb   short PrtItL2l

PrtItL3:   mov  ah,07h

stosw

PrtItL2l:  loop PrtItL2

Skip2end:  lodsb

cmp  al,13

jne  short Skip2end

PrtItL2D:  pop  di

add  di,160

pop  cx

loop PrtItL1

pop  si

;  Now let's pause for key input by the user and then parse it accordingly.

BadKey:    xor  ah,ah                   ;

int  16h                     ;

cmp  ax,011Bh                ; is it the ESC key?

je   short Done              ;

cmp  ax,5100h                ; page down

jne  short NoPD              ;

xor  bx,bx                   ;

mov  cx,22                   ; 22 lines at a time

PDownL:    cmp  byte [bx+si],00h        ; end of data?

je   short PIsBot            ;

inc  bx                      ;

cmp  byte [bx+si],0Ah        ; if 0Ah skip

jne  short PDownL            ;

loop PDownL                  ;

PIsBot:    add  si,bx                   ; move to that position

jmp  short PrintIt           ;

NoPD:      cmp  ax,4900h                ; page up

jne  short NoPU              ;

mov  bx,si                   ;

mov  cx,22                   ; 22 lines at a time

PUpLoop:   cmp  bx,offset Buffer1       ;

je   short PIsTop            ;

dec  bx                      ;

cmp  byte [bx],0Ah           ; skip all 0Ah

jne  short PUpLoop           ;

loop PUpLoop                 ;

PIsTop:    mov  si,bx                   ; move to that position

jmp  short PrintIt           ;

NoPU:      cmp  ax,4F00h                ; end key

jne  short NotEnd            ;

mov  si,offset Buffer1       ;

add  si,FileLen              ; move to the end of the file

jmp  short PrintIt           ;

NotEnd:    cmp  ax,4700h                ; home key

jne  short NotHome           ;

mov  si,offset Buffer1       ; move to the first of the file

jmp  PrintIt                 ;

NotHome:   cmp  ax,5000h                ; down arrow

jne  short NotDown           ;

xor  bx,bx                   ;

DownL:     cmp  byte [bx+si],00h        ;

je   short IsBot             ;

inc  bx                      ;

cmp  byte [bx+si],0Ah        ;

jne  short DownL             ;

IsBot:     add  si,bx                   ;

jmp  PrintIt                 ;

NotDown:   cmp  ax,4800h                ; up arrow

jne  short BadKey            ;  (must have been a key not needed

mov  bx,si                   ;   so just continue.)

UpLoop:    cmp  bx,offset Buffer1       ;

je   short IsTop             ;

dec  bx                      ;

cmp  byte [bx],0Ah           ;

jne  short UpLoop            ;

IsTop:     mov  si,bx                   ;

jmp  PrintIt                 ;  loop

Done:      mov  ax,0003h                ; set screen back to text 0003h

int  10h                     ;  (clear the screen)

mov  ah,01h                  ; turn on cursor

mov  cx,0607h                ; start = 6   finish = 7

int  10h                     ;

.exit                        ; exit to dos (user directive/macro)

;  Now we need the actual data and data space used by  the program we have

;   created.

HdrClr     db  17h

TxtClr     db  07h

FileLen    dw  00h

Header     db  ' A demo Viewer for the Tutorial included with NBASM   Forever Young Software    '

Status     db  ' Press ,  or <esc>                                   Line:                   '

ParmErrS   db  13,10,'Error with command line$'

File1ErrS  db  13,10,'Error with Source File.$'

FileName   dup 128,?

Buffer1    dup 32767,?

.end

; ********** end of file

Assemble with the following:

NBASM V.ASM

Run with the following:

V V.ASM

(Remember to add the valid file name as a parameter:  V.ASM)

NBasm will return any errors if found.

A few things to think about:

-Notice that I left the LINE: part on the status line we created at the

bottom of the screen.  You could easily add some  code to this program

to keep track of the line number and print it in this location.

-Also note that if you load a file  larger than 32k, this program  will

still work OK, but you will  get garbage as the last  part of the file

when it is displayed.

-I'm sure that there are a few more items that I have forgotten or bugs

that I didn't find. Please let me know if you see anything that should

be added or modified.